Model 520

Type Certificate:	6A1

Date Certificated:	January 31st 1952

PCLM:	5	(2 at +104, 3 at +165) Can have 6 or 7 seats with emergency exit (Drawing 5870041) and		structural provision for center seats (Drawing 5870027).

Number Built:	150

Certificated by:	Aero Design & Engineering Company

Serial numbers 1 through 150 	(150 aircraft)

Engines:	Lycoming GO-435-A2

Serial number 1	(1 aircraft)

Lycoming GO-435-C2

Serial numbers 2 through 20, 22, & 24 through 38 	(35 aircraft)

Lycoming GO-435-C2B

Serial numbers 21, 23, & 39 through 150 	(114 aircraft)

Lycoming GO-435-C2B1

none delivered, but was used later as an up-grade

Lycoming GO-435-C2B2

none delivered, but was used later as an up-grade	(but not listed on T.C.D.S.)

Propellers (hub/blade): 	Hartzell HC-12X20-8C/9333C-3

Serial numbers 1 through 14 & 27 	(15 aircraft)

Hartzell HC-12X20-9C/9333C-3

Serial numbers 15, 16, 18-20, 22, 24-26, 28-38, & 80	(21 aircraft)

Hartzell HC-82X20-2/9333C-3

Serial numbers 17, 21, 23, 39-69, 71-74 & 76-79	(42 aircraft)

Hartzell HC-82X20-2A/9333C-3

Serial numbers 70, 75, 81-88, 90-99, 101-125, 127 & 130-150 	(67 aircraft)

Hartzell HC-82X20-2C/9333C-3

Serial numbers 89, 126, 128 & 129	(4 aircraft)

Hartzell HC-83X20-2C/8433

Serial number 100 	(1 aircraft)

Fuel:	80/87 or 91/96 minimum grade aviation gasoline

Fuel consumption:	13.5 U.S. gallons per engine per hour at 2,600r.p.m.

Fuel capacity:	150 U.S. gallons (568 litres) total	[5 inter-connected tanks]

145 U.S. gallons (549 litres) usable

Oil capacity:	6.0 U.S. gallons (22.71 litres) total (3.0 gallons each engine)

4.5 U.S. gallons (17.03 litres) usable

| Weights & Balance: |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | lb | kg |  |  |
| Ramp Weight |  |  |  |  |
| MTOW | 5,500 | 2,495 | Higher weight eligible on serial numbers 31 through 150 incorporating Service Change No. 26-520. |  |
| MTOW | 5,700 | 2,585 |  |  |
| MLW |  |  |  |  |
| ZFW |  |  |  |  |
| Empty Weight (Typical) | 3,800 | 1,724 | (Factory Empty Weight, with standard equipment) |  |
| Useful load |  |  |  |  |
| Basic Operating Weight |  |  |  |  |
| Payload |  |  |  |  |
| Maximum Baggage | 350 | 159 |  |  |
| Maximum Wing Loading | 22.68 lb/sq.ft. | 110.74 kg/sq.ft. | 5,500lb. / 242.5sq.ft. |  |
| Maximum Wing Loading | 23.51 lb/sq.ft. | 114.76 kg/sq.ft. | 5,700lb. / 242.5sq.ft. |  |
| Maximum Power Loading | 10.58 lb/hp | 4.80 kg/hp | 6.43 kg/kW | 5,500lb. / 520hp. |
| Maximum Power Loading | 10.96 lb/hp | 4.97 kg/hp | 6.67 kg/kW | 5,700lb. / 520hp. |
| C.G. range | (+168.6) to (+175.1) - gear extended |  |  |  |
| Effect of retracting gear | +4110 in./lb. |  |  |  |
| Empty Weight C.G. range | None |  |  |  |
| Datum | Located 152 inches (12' 8") forward of wing leading edge |  |  |  |
| Levelling means | Longitudinal | Top of fuselage centerline, aft of wing trailing edge. |  |  |
|  | Lateral | Transverse beams at front or rear of baggage compartment floor. |  |  |

| Control Surface Movements: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | o up | o down | o right | o left | Remarks |
| Aileron | 23 +/-2 | 15 +/-2 |  |  |  |
| Flaps |  | 40 +/-2 |  |  |  |
| Elevator | 20 +/-2 | 15 +/-2 |  |  |  |
| Elevator tab | 2.5 +/-1 | 26.5 +/-0.5 |  |  |  |
| Rudder |  |  | 15 +/-1 | 15 +/-1 | T.C.D.S. says up & left !! |
| Rudder tab |  |  | 13 +/-0.5 | 26 +/-0.5 |  |

| Dimensions: |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Feet/Inches | Inches | Meters | Remarks |
| Wing span | 44’ 0.75” | 528.75 | 13.43 |  |
| Wing chord at root | 8’ 4.00” | 100.00 | 2.54 |  |
| Wing chord at tip | 2’ 9.00” | 33.00 | 0.84 |  |
| Wing - ground to wingtip height |  |  |  |  |
| Wing Aspect Ratio |  |  |  | 8.2 Wing section is NACA 23012 modified. |
| Wing Taper Ratio |  |  |  |  |
| Wing dihedral |  |  |  | 4 degrees |
| Wing incidence at root |  |  |  | 3 degrees |
| Wing incidence at tip |  |  |  |  |
| Wing area | 242.5 sq.ft. |  | 22.53 m2 |  |
| Total Flap area | 32.70 sq.ft. |  | 3.04 m2 |  |
| Total Aileron area | 20.52 sq.ft. |  | 1.91 m2 |  |
| Overall Length | 34’ 2.50” | 410.50 | 10.43 |  |
| Overall Height | 14’ 4.50” | 172.50 | 4.28 |  |
| Ground to fuselage top height | 6’ 5.00” | 77.00 | 1.96 |  |
| Ground to fuselage bottom height |  |  |  |  |
| Total Fin area | 20.60 sq.ft. |  | 1.91 m2 |  |
| Total Rudder area | 13.40 sq.ft. |  | 1.24 m2 |  |
| Tailplane span | 16’ 11.00” | 203.00 | 5.16 |  |
| Ground to tailplane tip height |  |  |  |  |
| Tailplane dihedral |  |  |  | 10 degrees |
| Tailplane - angle of incidence |  |  |  |  |
| Total Tailplane area | 33.06 sq.ft. |  | 3.07 m2 | not including elevators |
| Total Elevators area | 20.54 sq.ft. |  | 1.91 m2 |  |
| Wheel track (tread) | 12’ 4.00” | 148.00 | 3.95 |  |
| Wheelbase | 13’ 6.00” | 162.00 | 4.11 |  |
| Minimum ground turning radius |  |  |  |  |
| Cabin Length (Total) |  |  |  |  |
| Cabin Height |  |  |  |  |
| Cabin Width |  |  |  |  |
| Max. external fuselage width | 4’ 6.00” | 54.00 | 1.37 |  |
| Cabin door Height |  |  |  |  |
| Cabin door Width |  |  |  |  |
| Cabin door, Height to sill | 1’ 8.00” | 20.00 | 0.51 |  |
| Cabin volume | 146 cu.ft. |  | 4.13 m3 |  |
| Baggage compartment Length |  |  |  |  |
| Baggage compartment Height |  |  |  |  |
| Baggage compartment Width |  |  |  |  |
| Baggage door Height |  |  |  |  |
| Baggage door Width |  |  |  |  |
| Baggage door, Height to sill |  |  |  |  |
| Baggage compartment volume | 32 cu.ft. |  | 0.91 m3 |  |

| Performance: |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Feet | Meters |  | Remarks |
| Takeoff distance, over 50ft obstacle | 950 | 290 |  | with zero wind. |
| Takeoff run |  |  |  |  |
| Short-field takeoff, over 50 ft obstacle |  |  |  |  |
| Short-field takeoff run |  |  |  |  |
| Landing distance, over 50ft obstacle | 860 | 262 |  |  |
| Landing run |  |  |  |  |
| Short-field landing distance, over 50ft obstacle |  |  |  |  |
| Short-field landing run |  |  |  |  |
| Maximum Operating Altitude |  |  |  | FAA Operational Ceiling |
| Service ceiling | 24,000 | 7,315 |  |  |
| Single-engine ceiling, with feathered propeller | 10,500 | 3,200 |  |  |
| Single-engine ceiling, with unfeathered propeller | 8,700 | 2,652 |  |  |
|  | nm | sm | km |  |
| Range, maximum cruising power at 60% | 999 | 1,150 | 1,851 | with 30 minutes reserve. |
| Range, economy cruising power |  |  |  |  |
| Range, maximum cruising power, max payload |  |  |  |  |
| Range, economy cruising power, max payload |  |  |  |  |
|  | mph | kts | km/hr |  |
| Manoeuvring | 142 | 123 | 229 | True Indicated |
| Max. structural cruising (Max. Operating Speed) (Vmo) | 200 | 174 | 322 | True Indicated |
| Never Exceed (Vne) | 270 | 235 | 435 | True Indicated |
| Flaps extended – Half (20o) (Vfe) | 150 | 130 | 241 | True Indicated |
| Flaps extended – Full (40o) | 125 | 109 | 201 | True Indicated |
| Landing gear extended - original VLO | 125 | 109 | 201 | True Indicated |
| Landing gear extended - revised VLO | 180 | 156 | 290 | True Indicated |
| Maximum level speed (TAS) | 211 | 183 | 340 | at Sea Level |
| Cruising speed (TAS) - 70% NRP | 197 | 171 | 317 | at 10,000ft / 3,048m |
| Economy cruising speed (TAS) |  |  |  |  |
| Takeoff speed | 82 | 71 | 132 |  |
| Landing speed | 77 | 67 | 124 |  |
| Best rate-of-climb speed (Vy) | 140 | 122 | 225 |  |
| Best Angle of Climb speed (Vx) |  |  |  |  |
| Minimum Control speed (Vmc) |  |  |  |  |
| Stall Speed (Clean) (Vsi) | 67 | 58 | 108 |  |
| Stall Speed - Full Flaps - power off | 60 | 52 | 97 |  |
| Stall Speed - Full Flaps - power on | 40 | 35 | 64 |  |
|  | fpm | mpm |  |  |
| Initial Rate of Climb |  |  |  |  |
| Initial Rate of Climb - Single-engine |  |  |  |  |
| Max. Rate of Climb at Sea Level | 1,700 | 518 |  |  |
| Max. Rate of Climb at Sea Level, single engine | 400 | 122 |  |  |

| Engines (1): |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Characteristics | Piston, air-cooled, Geared, with 6 horizontally Opposed cylinders. |  |  |  |  |
| Manufacturer | Lycoming |  |  |  |  |
| Engine Type | GO-435 |  |  |  |  |
| Engine Variant | -A2 | -C2 | -C2B | -C2B1 | -C2B2 |
| Type Certificate Number | E-228 |  |  |  |  |
| Type Certificate Date | Oct. 19th 1949 | Oct. 19th 1949 | Nov. 6th 1952 | Feb. 8th 1955 | Feb. 28th 1956 |
| Applicable under STC No. | n/a | n/a | n/a |  |  |
| Maximum continuous HP |  | 240 | 240 |  |  |
| Maximum continuous RPM |  | 3,000 | 3,000 |  |  |
| Takeoff HP (5 minutes) |  | 260 | 260 |  |  |
| Takeoff RPM |  | 3,400 | 3,400 |  |  |
| Weight (lb.) |  | 422 | 430 |  |  |
| Reduction Gear Ratio |  | 0.642 | 0.642 |  |  |
| Supercharging Ratio |  | n/a | n/a |  |  |
| Bore & Stroke (inches) |  | 4.875 x 3.875 | 4.875 x 3.875 |  |  |
| Displacement (cu.in.) |  | 434 | 434 |  |  |
| Compression Ratio |  | 7.30 : 1 | 7.30 : 1 |  |  |
| Carburetor Make & Model |  | Bendix PS-5BD | Bendix PS-5BD |  |  |
| Notes: | The -C2B is a -C2 with propeller governor drive integral with reduction gear. |  |  |  |  |
|  | The -C2B1 is a -C2B with angle generator drive. |  |  |  |  |
|  | The -C2B2 is a -C2B with lightweight magnetos. |  |  |  |  |
|  | The Type Certificate for the -A2 was cancelled on Feb. 8th 1955. |  |  |  |  |
|  | Maintenance Manual says Normal Rated Power is 245hp at 3,100rpm. |  |  |  |  |

| Engines(2): |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Characteristics | Piston, air-cooled, Geared, with 6 horizontally Opposed cylinders. |  |  |  |  |
|  |  |  |  | Supercharged. |  |
| Manufacturer | Lycoming |  |  |  |  |
| Engine Type | GO-480 |  |  | GSO-480 |  |
| Engine Variant | -B | -B1A6 | -B1D | -A1 | -A1A6 |
| Type Certificate Number | E-275 |  |  |  | E-284 |
| Type Certificate Date | Apr. 21st 1954 | May 10th 1956 | Jul. 5th 1956 |  | Jun. 30th 1955 |
| Applicable under STC No. | n/a | SA3607SW | n/a |  | n/a |
| Maximum continuous HP | 260 |  |  |  | 320 |
| Maximum continuous RPM | 3,000 |  |  |  | 3,200 |
| Takeoff HP (5 minutes) | 260 |  |  |  | 340 |
| Takeoff RPM | 3,400 |  |  |  | 3,400 |
| Weight (lb.) | 432 |  |  |  | 498 |
| Reduction Gear Ratio | 77 : 120 |  |  |  | 77 : 120 |
| Supercharging Ratio | n/a |  |  |  | 11.27 : 1 |
| Bore & Stroke (inches) | 5.125 x 3.875 |  |  |  | 5.125 x 3.875 |
| Displacement (cu.in.) | 479.7 |  |  |  | 479.7 |
| Compression Ratio | 7.3 : 1 |  |  |  | 7.3 : 1 |
| Carburetor Make & Model | Bendix PS-5BD |  |  |  | Bendix PS-7BD |
| Fuel Grade | 80 / 87 |  |  |  | 100 / 130 |
| Notes: | The -B1A6 is the same as the -B, except has one 6th and five 3rd order counterweights. |  |  |  |  |
|  | The -B1D is the same as the -B, excepting magnetos and has 1.250:1 generator drive. |  |  |  |  |

| Propellers (Two-bladed): |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Characteristics | Constant speed, hydraulic. |  |  |  |  |  |  |
|  | For -C2 engine only |  | with feathering and unfeathering features. |  |  |  |  |
| Manufacturer | Hartzell |  |  |  |  |  |  |
| Hub Model | HC-12X20-8C | HC-12X20-9C |  | HC-82X20-2 |  | HC-82X20-2A | HC-82X20-2C |
| Type Certificate Number | P-845 |  |  | P-878 |  |  |  |
| Type Certificate Date | May 8th 1946 |  |  | October 8th 1952 |  |  |  |
| Hub Material | Alloy Steel |  | Alloy Steel |  |  |  |  |
| Blade Model | 9333C-3 |  | 9333C-3 |  |  |  |  |
| Blade Material | Aluminum Alloy |  | Aluminum Alloy |  |  |  |  |
| Number of Blades | 2 |  | 2 |  |  |  |  |
| Diameter (inches) | 90 |  | 90 |  |  |  |  |
| Diameter (meters) | 2.29 |  | 2.29 |  |  |  |  |
| Cut-off permitted | 2” |  | 2” |  |  |  |  |
| Prop tip ground clearance (inches) |  |  |  |  |  |  |  |
| Prop tip fuselage clearance (inches) |  |  |  |  |  |  |  |
| Distance between propeller centers |  |  |  |  |  |  |  |
| Pitch setting at 30” Station - Low | 19o |  |  | 18o |  |  |  |
| - High | 36o |  |  | 28.5o |  |  |  |
| - Feathered | n/a |  |  | 89o |  |  |  |
| Max. Hub & Blade weight (lb.) | 73 | 79 |  | 71 |  |  |  |
| HP - Maximum continuous | 260 |  | 260 |  | 260 |  |  |
| HP - Takeoff | 260 |  | 260 |  | 280 |  |  |
| RPM - Maximum continuous | 2,180 |  |  | 2,330 |  | 2,330 |  |
| RPM - Takeoff | 2,180 |  |  | 2,330 |  | 2,330 |  |
| Note: The HC-12X20-9C incorporates a snap action feathering feature and cannot be unfeathered in flight. |  |  |  |  |  |  |  |
| Note: Maintenance Manual says propeller rpm at Normal Rated Power is 1,925rpm. |  |  |  |  |  |  |  |

| Propellers (Three-bladed): |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Characteristics | Constant speed, hydraulic. |  |  |  |  |
|  | with feathering and unfeathering features. |  |  |  | Reversing. |
| Manufacturer | Hartzell |  |  |  |  |
| Hub Model(s) | HC-83X20-2 | HC-83X20-2A | HC-83X20-2B | HC-83X20-2C | HC-83X20-3A |
|  |  | HC-83X20-2A1 | HC-83X20-2B1 | HC-83X20-2C1 |  |
| Type Certificate Number | P-884 |  |  |  |  |
| Type Certificate Date | May 5th 1954 |  |  |  |  |
| Hub Material | Alloy Steel |  |  |  |  |
| Blade Model(s) | 8433 | 8433 | 8433 | 8433 | 8433 |
|  |  | V8433N |  |  |  |
| Blade Material | Aluminum Alloy |  |  |  |  |
| Number of Blades | 3 |  |  |  |  |
| Diameter (inches) | 84 |  |  |  |  |
| Diameter (meters) | 2.13 |  |  |  |  |
| Cut-off permitted | none |  |  |  |  |
| Prop tip ground clearance (in.) |  |  |  |  |  |
| Distance between prop. centers |  |  |  |  |  |
| Pitch setting at 30" Station -Low | 16o |  |  |  |  |
| - High | 26.3o |  |  |  |  |
| - Feathered | 87o |  |  |  |  |
| Max. Hub & Blade weight (lb.) | 91 |  |  |  |  |
| HP - Maximum continuous | 290 |  |  |  |  |
| HP - Maximum Takeoff | 280 |  |  |  |  |
| RPM - Maximum continuous | 2,180 |  |  |  |  |
| RPM - Maximum Takeoff | 2,180 |  |  |  |  |
| Notes: | The HC-83X20-2A1 / V8433N combination is suspect. An ‘X’ hub with a ‘V’ blade? |  |  |  |  |
|  | The -3A hub is also suspect. The ‘-3’ denotes reversing but not feathering features. |  |  |  |  |

| Engine & Propeller combinations: |  |  |
| --- | --- | --- |
| Engines | Propellers (Hub / Blade) | Factory or other |
| Lycoming GO-435-A2 | Hartzell ? | Factory |
| Lycoming GO-435-C2 | Hartzell HC-12X20-8C / 9333C-3 | Factory |
|  | Hartzell HC-12X20-9C / 9333C-3 | Factory & Field up-grade |
| Lycoming GO-435-C2B | Hartzell HC-82X20-2 / 9333C-3 | Factory |
|  | Hartzell HC-82X20-2A / 9333C-3 | Factory & Field up-grade |
|  | Hartzell HC-82X20-2C / 9333C-3 | Factory |
|  | Hartzell HC-83X20-2C / 8433 | Factory |
|  | Hartzell HC-82X20-2A / 8433 | Field - a suspect combination |
|  | Hartzell HC-83X20-2 / 8433 | Field |
|  | Hartzell HC-83X20-2A / (blade unquoted) | Field - blade should be 8433 |
|  | Hartzell HC-83X20-2B / blade unquoted) | Field - blade should be 8433 |
|  | Hartzell HC-83X20-2B1 / (blade unquoted) | Field - blade should be 8433 |
|  | Hartzell HC-83X20-2C1 / (blade unquoted) | Field - blade should be 8433 |
|  | Hartzell HC-12X20-8C / 9333C-3 | Field -an ‘illegal’ combination? |
|  | Hartzell HC-83X20-3A / 8433 | Field - a suspect hub (Reversing) |
| Lycoming GO-435-C2B1 | Hartzell HC-82X20-2 / 9333C-3 | Field |
|  | Hartzell HC-83X20-2A / (blade unquoted) | Field - blade should be 8433 |
|  | Hartzell HC-83X20-2C / (blade unquoted) | Field - blade should be 8433 |
| Lycoming GO-435-C2B2 | Hartzell HC-82X20-2 / (blade unquoted) | Field |
|  | Hartzell HC-83X20-2B / (blade unquoted) | Field - blade should be 8433 |
|  | Hartzell HC-83X20-2C / 8433 | Field |
| Lycoming GO-480-B | Hartzell ? | Field - s/n 88 |
| Lycoming GO-480-B1A6 | Hartzell HC-83X20-2A1 / V8433N | Field - ‘X’ hub / ‘V’ blade - suspect? |
| Lycoming GO-480-B1D | Hartzell ? |  |
|  | Hartzell HC-83X20 / (blade unquoted) | Field |
| Lycoming GSO-480-A1 | Hartzell ? | Field - s/n 88 |
| Lycoming GSO-480-A1A6 | Hartzell ? | Field - s/n 88 |

Notes:

The Model 520 had no MLG uplock system (the 500A family was the first to incorporate this feature). The retracted gear was held up in the wheel-well by normal hydraulic system pressure. A loss of the single hydraulic pump required the crew to periodically re-pressurize the system by use of the manual hydraulic pump recessed in the floor between the crew seats (the handle was stored in one of the seat pockets). In the 1960’s I observed the Commander Service Center installing (per STC) an electric pump to replace the manual pump in one of these aircraft.

Landing Gear Doors:

The NLG door configuration consists of a pair of short forward doors and another pair of longer aft doors and was a very well designed, purely mechanical system that sequenced the opening and closing of the doors. It was powered by the movement of the gear strut. As the NLG started to move on extension, the system opened both pair of doors. As the gear strut approached full down the system then closed the longer aft pair of doors. Upon retract the system opened the aft pair of doors and as the strut approached full up the system then closed both pair of doors. The system remained essentially unchanged for all subsequent aircraft.