# BN-2 Islander Checklist

For use in flight simulator only! Do not use it in real world!

### Before Start:

*Circuit Breakers	CHECKED

Parking Brake	ON

Battery Master Switch	ON

Generators	CHECKED OFF

Beacon	CHECKED ON

Passenger Notice	CHECKED ON

Carb. Heat	OFF

Flaps (full range)	CHECKED & SET

*Busbar Isol. Lamp	PRESS TO TEST

Elevator Trim	CHECKED & SET

Mixture	RICH

Props	MAX RPM

Throttles	CLOSED

*Friction Lock	CHECKED & SET

Radios	SET

Altimeter	X-CHECKED

Central Warning Panel	TEST

Rudder Trim	CHECKED & NEUTRAL

Fuel Selectors	TANK TO ENGINGE

Fuel Quantity	CHECKED

Eng 1 magneto	TEST DOOR WARNING

All Doors & DV-window	CLOSED

### Engine Start:

Sequence: 2, 1

*If on External Power:

External Power	CONNECTED

External Supplies Switch	ON

Aux. fuel pumps	ON, PRESSURE CHECKED

Cold engine : Throttle	PUMP 3-4 x

Throttle	CA. 1/10 OPEN

Left Magneto	ON

CLEAR PROP!

Starter	MAX. 6 TURNS

Right Magneto	ON

Oil Pressure	CHECKED WITHIN 30”

*If on External Power:

Ext. Power	DISCONNECTED

Ext. Supplies Switch	OFF/STARTER ISOL.

### After Start:

Generator undervolt lights	LIT

Generators	ON

Generator undervolt lights	OUT

Pitot Heat	AS REQUIRED

Vacuum gauges	3-5”

Aux. fuel pumps	AS REQUIRED

Ammeters	GREEN, BAT

Fuel Pressure	GREEN

Oil Pressure	30 PSI

Flight Instruments	CHECKED & SET

Radios	CHECKED & SET

*Heater/Vent	AS REQUIRED

### Runup:

Brakes	CHECKED

Parking Brake	ON

Power	17”

Magnetos	CUTOUT CHECKED

Magnetos: max. drop / diff.	175 / 50 RPM

Power	2100 RPM

Props pitch control	CHECKED

Carb. heat	CHECKED

Power	1500 RPM

Props feather control	CHECKED

Ammeters	CHECKED, BAT

Voltmeter	27-29 V

### Before Takeoff:

*Heater	OFF

Flight Controlts	CHECKED & FREE

Left & Right Magnetos	ALL ON

Aux. fuel pumps	ON, PRESSURE CHECKED

Fuel Quantity	CHECKED

Fuel selectors	AS REQUIRED

Electrics / 27-29 V charge	AS REQUIRED

Flight Instruments	CHECKED & SET

Engine Instruments	CHECKED

Carb. Heat	OFF

Flaps	T/O

Trim	SET FOR T/O

Mixture	RICH

Props	CA. 2500 RPM

### Takeoff:

Rotate	46-51 KTS

V2	50-55 KTS

Vmc	61 KTS

Flaps (at 200 ft AGL)	UP

Aux. fuel pumps	OFF

Climb (Vx + Vy)	65 KTS

Heater	AS REQUIRED

Cruise	CA. 23” / 2300 RPM

Mixture	LEAN AS REQUIRED

VB	104 KTS

VTurb	88 KTS

### Approach:

Fuel Selectors	AS REQUIRED

Aux. Fuel pumps	ON, PRESSURE CHECKED

*Heater	OFF

*Blower	ON IF REQUIRED

Carb. Heat	AS REQUIRED

Mixture	RICH

Flaps	FULL

Props	MAX RPM

Approach speed	65 KTS

VThr	53-58 KTS

### Go Around:

Positive Climb: Flaps	T/O

Climb (Vx)	65 KTS

Flaps (at 200 ft)	UP

### After Landing:

Aux. Fuel pumps	OFF

Flaps	UP

Pitot Heat	OFF

*Heater	OFF

### Parking:

Parking Brake	ON

Mixture	IDLE CUT-OFF

Magnetos	ALL OFF

all Electrics	OFF

Generators	OFF

Battery Master Switch	OFF

Controls	LOCKED

Chocks	IN POSITION

## A & E Checklists

## Fire:

### Ground Fire:

Parking Brake	ON

Both engines:

Mixtures	IDLE CUT-OFF

Throttles	CLOSED

Fuel Selectors	OFF

Aux. Fuel pumps	OFF

Magnetos	OFF

Generators	OFF

ATC	INFORM

Evacuation	INITIATE

Passenger Evac.	ANNOUNCE

Battery Master Switch	OFF

Left Magnetos	CHECKED OFF

Fire Extinguisher	APPLY

### Engine Fire in Flight

Engine #	CONFIRMED

Mixture #	IDLE CUT-OFF

Prop #	FEATHER

Throttle #	CLOSED

Fuel Tank #	SELECT OFF

Aux. fuel pump #	OFF

Magnetos #	OFF

Flaps	UP

Generator #	OFF

Life Generator	CHECKED ON

Rudder Trim	ADJUST

If Fire persists:

ATC	INFORM

### Cabin Fire in Flight

Fire Extinguisher	APPLY

If Fire persists:

ATC	INFORM

- Forced Landing ASAP!

When fire is out.

DV window	OPEN

Cabin ventilation blower	ON

### Forced Landing:

Transponder	SQUAWK 7700

ATC	INFORM

ELT	ON

If required: Door(s)	Unlock

### Engine Failure, Takeoff run:

If below 61 kts	STOP

Parking Brake	ON

If Fire: Ground Fire procedure

If NOT below 61 kts AND NO Fire:

Engine #	CONFIRMED

Mixture #	IDLE CUT-OFF

Throttle #	CLOSED

Fuel Selector #	SELECT OFF

Magnetos #	OFF

Aux. fuel pumps #	OFF

Generator #	OFF

Life Generator	CHECKED ON

ATC & COMPANY	INFORM

### Engine Failure after Takeoff

Climb (Vx)	65 KTS

Flaps	UP

Mixture	CHECKED FULL RICH

Carb. Heat	CHECKED OFF

Engine #	CONFIRMED

Prop #	FEATHER

Mixture #	IDLE CUT-OFF

Throttle #	CLOSED

Fuel Tank #	SELECT OFF

Aux. fuel pump #	OFF

Magnetos #	OFF

Generator #	OFF

Life Generator	CHECKED ON

Rudder Trim	ADJUST

Warning: Min. Feather 1300 RPM

### Flight on 1 Engine:

Fuel unbalance	X-FEED AS REQUIRED

Approach: Flaps	T/O

Approach Speed	65 KTS

Committed: Flaps	FULL

### Flaps Fault at 0:

Final Speed	65 KTS

### Main Amplifier Failure:

Loudspeakers	USE

### Engine unfeathering:

Fuel Tank #	SELECT ON

Left Magneto #	ON

Aux. fuel pump #	ON, PRESSURE CHECKED

Mixture #	RICH

Prop #	CRUISE RPM

Throttle #	IF REQUIRED PRIME 2-4 x

Throttle #	1 cm OPEN

External Supply Switch	ON

Starter #	ENGAGE

When engine running:

Right Magneto #	ON

Engine # gauges	CHECKED

Engine # power	AS REQUIRED

When engine stabilized:

Aux. fuel pump #	OFF

Ext. Supply Switch	OFF/STARTER ISOL.

Generator #	ON & CHECKED

Vacuum gauge	CHECKED

## Electrical:

### Load reduction:

Fuel Quantity	CHECKED, THEN OFF

All non-essential Circuit Brakers	PULL

Heater & Blower	OFF

If no icing: Pitot & Stall Warning Heater	OFF

All Light Switches	OFF/AS REQUIRED

Panel Lights	OFF/AS REQUIRED

Map Lights	OFF/AS REQUIRED

COM 2 & NAV 2	OFF

ADF & Marker	OFF

Radio Transmissions	LIMIT

Battery Life	30-35 min.

### Battery Failure:

Indications (one or more):

Both Generator undervolt lights lit momentarily, then out

Ammeter (both GEN)	ZERO OR RED

Ammeter (BAT)	DISCHARGE

Equipment from Generator Busbar	U/S

Actions:

Battery Master Switch	OFF

Both Generators	CHECKED ON

Generators available if load reduced

Both Generators	RESET

- Apply procedure Load reduction

### Generator Busbar Failure:

Inidications (one or more):

BUSBAR light	LIT

Voltmeter	BELOW ZERO

Equipment from Generator Busbar	U/S

Eng. Instruments	U/S

Magnesyn Compass	U/S

- Apply procedure Both Generators Failure

### Both Generators Failure:

Indications (one or more):

Both Generators overvolt lights	LIT

Both Generators undervolt lights	LIT

Ammeter (both GEN)	ZERO OR RED

Ammeter (BAT)	DISCHARGE

Voltmeter	24 V OR LESS

Generator Circuit Brakers	TRIPPED

If overvolt only:	AS REQUIRED

Actions:

Both Generators	OFF

Both Generators Circuit Brakers	PULL

Flaps Fault: at present position!

- Apply procedure Load reduction

### Single Generator Failure:

Indications (one or more):

Generator # undervolt light	LIT

Ammeter #	ZERO OR RED

Generator Circuit Braker	TRIPPED

Actions:

Generator #	OFF

Generator Circuit Braker #	PULL

Load reduction	MAX 50 A

*) This item of the normal checklist makes no sense in Flight Simulator because the used element is not available.