Archer II Checklist

By David Voogd

For flight simulator use only!

| Va 2550lbs | 113 kts |
| --- | --- |
| Va 1634lbs | 89 kts |
| Vx | 64 kts |
| Vy | 76 kts |

Startup

Master on

Fuel on right

Carb heat cold

Throttle ¼”

Mixture full rich

Prime engine

Fuel pump on

Engage starter

After engine starts

Engine 1000 rpm

Check oil pressure and temp

Radios and electronics on

Brake check

Taxi for run-up

Run-up

Fuel on left

Fuel pump off

Engine 2000 rpm

Check oil pressure and temp

Suction 5”

Check mags

Max drop 175 rpm

Max diff 50 rpm

Press annunciator test and check

Carb heat hot

Test mixture control

Throttle idle - check engine

Carb heat cold

Engine 1000 rpm

Transponder on standby

Check ATIS

Taxi clearance

Pre Takeoff / Holding Short

Primer locked

Master on

Mags on both

Carb heat cold

Mixture rich

Fuel pump on

Fuel on fullest tank

Flaps and trim set

Controls free and correct

Transponder on alt and assigned code

Tower frequency

Takeoff clearance

Takeoff

Check oil pressure and temp

Full Throttle - check RPMs

Rotate at 50 kts

Maintain 64 kts until clear of obstacle

Climb

Maintain 76 kts or 87 kts enroute

Lower nose every 500’ to check for traffic and cool engine

Cruise

Fuel pump off

Maintain selected RPM from charts

Lean mixture above 5,000’ to slightly rich of peak EGT

Fuel pump on when switching tanks

Run on first tank for one hour then other tank for two hours

Decent

Carb heat as required

If power off approach used open throttle every 30 seconds to prevent overcooling

Approach and Landing

Primer locked

Master on

Mags on both

Carb heat hot

Mixture rich

Fuel on fullest tank

Fuel pump on

Landing clearance

Slow to 75 kts

Lower flaps in increments

Maintain 66 kts on final

After Landing

Clear runway

Flaps up

Carb heat cold

Unnecessary electronics off

Transponder standby and 1200

Taxi clearance

Shutdown

Fuel pump off

Radios and electronics off

Live Mag Check

Mixture idle cut off

Mags off

Master off