#### North American AT-6D

FS2002/2004 MODEL - TOTAL FREE

The T-6, one of the most popular warbird aircraft today, is an outstanding single engine trainer that has been used by air forces of over 30 countries. It is an excellent aircraft for anyone transitioning to a high horsepower tailwheel fighter
Normal cruise speed is 155 MPH (30 GPH) at 8000'. The aircraft is stressed for aerobatics and is capable of most maneuvers with the exception of sustained inverted flight, snap rolls, outside loops, and inverted spins.

The T-6/SNJ/Harvard aircraft have been produced in a number of model designations. Most of the changes are small and do not affect today’s warbird operator. The following section is devoted to discussing the most common differences.

Fuel Capacity-   The T-6 has 110 gallons on all models except the T-6G and Harvard MK IV, which have 140 gallons. With a cruise fuel burn of 30 GPH, 110 gallons is adequate for most operators.

Tail wheel steering/locking systems-  The Navy type is lockable only. The pilot is able to lock the tailwheel to a straight-ahead position for take off and landing. Steering is accomplished by differential braking. The steerable type system (also called P-51 type) uses an interconnect from the rudder pedals to the tailwheel steering system. This system allows the pilot to steer the aircraft by use of the rudder pedals. Full forward stick movement unlocks this system. When unlocked the tailwheel becomes full swivel and steering is again by differential braking. Either of these systems is adequate for most civilian operators.

Hydraulic system-  The original system incorporated a pilot controlled bypass. In order to use the gear or flaps, a small button must first be pushed before activation of the system. This button pressurizes the system and a time delay circuit depressurizes the system after approximately 45 seconds. Later aircraft (TG-G/Harvard MK-4) had a modified linkage that engaged the system automatically. For practical purposes, either system is satisfactory. There are several variations in other areas such as instrument panel layout and cockpit glass. Many aircraft have been modified to incorporate various combinations of the above systems. For the most part, any of these systems work well for the civilian operator.

For the average civilian operator, the exact model should not be as big a concern as finding a good clean aircraft at the investment level desired. Most of these aircraft have been in civilian ownership for a number years; therefore, they have been well taken care of. A pre-purchase inspection by a qualified shop is always a good idea. General condition and lack of major corrosion are important. Since these aircraft were designed as trainers they may have had some damage in their lifetimes. If the damage was repaired and the appropriate parts were replaced, the damage history of the aircraft will not be a major issue. The level of restoration is a big variable and greatly affects the price of the aircraft. It is best to acquire the most aircraft available for the investment and to not be overly specific on an exact model.

###### COST OF OPERATION & MAINTENANCE

Fuel: 30 GPH @ 2.80 per gallon = $84.00/hour

Oil: 2 quarts/hour

Number in Civilian Registry: 600

Average Number Sold per Year: 35

These aircraft consist of simple electric and hydraulic systems and can be maintained very easily by most general aviation shops. There is no special test equipment required that is different from normal civilian aircraft. Parts are readily available from several sources. As a private owned Part 91 Aircraft, an Annual Inspection is required. Most aircraft in good condition can be inspected for $2500 or less.

BASIC SPECIFICATIONS

Engine: Pratt & Whitney R-1340AN-1 engine, 600 HP

Propeller: Hamilton Standard Two Blade 12D40 Propeller

Wing Span: 42' 5"

Length: 29' 6"

Height: 11' 9'

Normal Gross Weight: 5300 LBS.

G Loading: +5.67, -2.33

http://www.courtesyaircraft.com/T6_prof.htm

DRAWINGS

“Scale Airplane Drawings – Volume 2”

PAUL MATT

Other airplanes in:

http://lucariny.sites.uol.com.br

out/2003