ARMSTRONG WHITWORTH

AW.650

ARGOSY

Cockpit & Flight Manual

Version 1.0 October 2005

For use with Rick Piper’s Argosy 200 series for Flight Simulator 2004

By Fraser McKay

Many thanks to David Maltby for his assistance with the autopilot manual functions.

CONTENTS

- MUST READ

- MAIN INSTRUMENT PANEL

- CENTRAL WARNING SYSTEM

- FLIGHT SYSTEM & AUTOPILOT

- CENTRE INSTRUMENT PANEL

- QUADRANT & PEDESTAL

- FUEL PANEL

- GENERATOR PANEL

- STARBOARD OVERHEAD

- PORT OVERHEAD

- REAR OVERHEAD

- RADIO PANEL

- ENGINE AND FLIGHT HANDLING

Argosy 222

- MUST READ

It is strongly recommended that this manual be read in its entirety to obtain maximum enjoyment from the panel and avoid difficulty.

In particular, the Dart 532 engine handling properties require familiarisation,

and the correct procedures must be followed for successful starts to be made both

on the ground and relighting in the air. Additionally note that the throttle and propeller RPM controls are interconnected and power is effected by movement of a single lever for each engine, referred to as the throttle. While the engines are running, do not attempt to alter the pitch of the propellers using the keyboard or joystick controls, the correct blade angle for the required power setting will be automatically selected through movement of the throttle lever.

A number of the aircraft’s systems require AC power, provided by the three main inverters which are on by default. The inverters will fail as the main bus voltage drops

below 25VDC, however, selection of the battery isolation switch on the starboard side panel  to “Isolate” will simulate the connection of ground power and will allow the inverters to function when the engines are not running. However due to Flight Simulator restrictions, this will not prevent the battery discharging and services will be lost as its output decays.

Argosy 222

2. MAIN INSTRUMENT PANEL

- Pitot Heat Failure Lamp

- Stopwatch. Click to start, stop and reset.

- Airspeed Indicator

- Director Horizon

- Servo Altimeter

- Accelerometer

- RMI Input Magnetic Indicators

- Radio Magnetic Indicator

- Beam Compass

- Vertical Speed Indicator

- SFS Input Magnetic Indicator

- Autopilot Beam Coupled Lamp

- Marker Lamps

- Standby Gyro Horizon

- SFS Omni Bearing Selector

- Course Deviation Indicator

- Autopilot Engage & Trim Indicator

- Master Fire Warning Lamp

- Engine Deice Failure Lamp

- Central Warning System Alert Lamp

- Autopilot Power Fail Lamp

- Water Methanol Contents Gauges

- Water Methanol Pumps & Cocks Switches

- Water Methanol Pressure Lamps

- DME 1 and 2 Indicators

- Hydraulic Pressure Failure Magnetic Indicators

- Brake Pressure Indicator

- Turn & Slip Indicator

3.CENTRAL WARNING SYSTEM

The Central Warning System (CWS) consists of a master red Alert warning lamp on the main instrument panel (20), connected to the warning lamps of the aircrafts main systems. All of the lamps in the CWS have a push/pull lens screen.

The systems connected to the CWS are:

Fuel Filter Icing (4)

LE Duct Overheat (2)

Inverter Failure (3)

Generator Failure (4)

Busbar Failure (2)

Battery Isolated (1)

Fuel low pressure (4)

The Alert lamp will flash when any failure causes one or more of the system warning lamps to light. The relevant system warning light may be pulled if the fault cannot be cleared, which will cancel the Alert warning lamp and reset the CWS. The system warning lamp will remain on as long as the fault persists and should be pressed in again if the fault is finally cleared.

To avoid nuisance from the Alert lamp continuously flashing whilst on the ground, its own lens screen may be pulled, causing the lamp to remain on steady. The lamp must be pressed in again before taxying.

FLIGHT SYSTEM & AUTOPILOT

Radio Selector.

The radio selector is used to select Nav1 or Nav2 information to the Flight System. The top switch is selected to 1 or 2 as desired. Note that the Azimuth Director and Pitch Director will only respond to Nav1 displacements so if Nav2 is selected the radial and glide path must be flown raw. When the switch is Off, the Azimuth Director will respond to differences between the Heading Pointer and Heading Index on the Beam Compass.  A magnetic indicator shows the input to the Flight System.

Director Horizon

- Beam Flag. Visible when a steady VOR/Localiser signal is received

- G/P Flag. Visible when a steady Glide Path signal is received.

- Pitch Director. Shows demand for pitch corrections.

- Horizon Bar. Moves relative to aircraft bank.

- Pitch Pointer. Moves vertically relative to aircraft pitch

- Pitch Scale.

- Bank Scale.

- Azimuth Director. Shows demand for heading and bank corrections.

- Ring Sight. Placing the Ring Sight over the Azimuth Director will correctly bank the aircraft.

- Pitch Scale Setting Knob.

Beam Compass

- Sense Switch. Points up when flying towards a station and down when

flying from it. With the switch across, no signals are received.

- Compass Scale. At its top datum, the Compass Scale shows the  VOR or ILS course selected on the Omni Bearing Indicator. Additionally it may be

rotated using the area over the Compass Warning Light.

- Heading Pointer. Shows aircraft heading relative to the Compass Scale.

- Heading Index. Used to set desired heading

- Beam Displacement Bar. Shows aircraft deviation from the selected course.

- Beam Displacement Scale.

- Setting Knob. Used to alter the Heading Index.

- Compass Warning Lamp (Inoperative).

Argosy 222

Omni Bearing Indicator

The Omni Bearing indicator is used to select the desired radial/localiser to the Flight System using the setting knob on the right. The switch on the left may be used to select the reciprocal bearing to that shown in the window. The bearing shown is that relative to the radio aid selected on the Radio Selector . A To/From indicator shows if the aircraft is flying to or from the station.

Course Deviation Indicator

The CDI shows deviations from the course and glide path of a facility selected on the OPPOSITE Radio to that selected on the Radio Selector; i.e. if Nav1 is selected to the Flight System, Nav2 information will be displayed on the CDI and vice versa. It is recommended that for ILS approaches where a VOR bearing is not required, that the ILS be tuned to both radios.

Radio Magnetic Indicator

A twin pointer RMI is fitted, the red single pointer can show either VOR1 or ADF1, the green dual pointer VOR2 or ADF2. Switching is on the radio panel and two magnetic indicators adjacent to the RMI show the selection which has been made. Note the MI will show black if VOR is selected and an ILS frequency has been tuned.

Autopilot Controller

- Power Switch. The Power switch must be pulled to initiate the supply to the Autopilot. This is effectively the FS Autopilot master. When the AP is ready for use the amber Ready lamp will illuminate.

- Engage Button is pressed to engage the AP and hold the current pitch attitude.

- Channel Switches . May be used to isolate the Rudder, Elevator or Aileron channels from the Autopilot.

- Height/Airspeed Lock. Turned to engage the height lock or airspeed hold. The airspeed function operates by varying pitch, therefore airspeed and height locks cannot be used simultaneously.

- Alter Heading Button. When engaged the aircraft will turn onto a heading preselected on the heading index, and will follow any change on the heading index.

- Beam Coupling Switch. Pull to engage the FS Nav1 hold function. Only available with SFS selector at Radio 1.

- Glide Coupling Switch. Pulled to engage FS Approach Hold function. This will also cause the Beam Switch to engage. Only available with SFS selector at Radio 1.

- Pitch Switch.  Used to manually vary the nose up or down pitch after pressing the green Engage button.

- Turn Knob. Used to accomplish manual turns using the autopilot. Not available with the Alter Heading button in.

- Ready Lamp. The lamp will extinguish when any function is engaged.

Note that the Autopilot will only respond to Nav1 information and that once the Engage button is depressed, all Pitch/Azimuth Director indications are relative to AP commands.

Engage & Trim Indicator

The Engage and Trim Indicator shows elevator trim movements on the centre scale. Additionally three flags marked IN will appear when the relevant control

surface channel is engaged.

5.CENTRE INSTRUMENT PANEL

The centre panel houses the instruments pertaining to the engines. From top to bottom these are:

1. Engine shaft RPM

2. Engine Torque. The instrument has a yellow paint line to indicate the

minimum dry takeoff power, ie without water-methanol, and

a green paint line to show  the minimum wet power, that is

with water-methanol selected.

3.Turbine Gas Temperature. This is the most critical of the engine values and its

limitations must not be exceeded.

4.Fuel Flowmeter . The pointer shows pounds per hour, the subscale pounds

used. This is a division of the total fuel used, therefore it will

continue to count if a  particular engine is shut down

5. Oil Pressure & Temperature Gauge.

The centre line consists of :

1. Synchroscope

2. SFS Radio selector

3. OAT gauge

4. Flap Position Indicator

5. Undercarriage Position Indicator

6. Undercarriage selector

6.QUADRANT & PEDESTAL

The quadrant is opened with the  icon.

Throttle Levers

At the forward end of the centre pedestal is the quadrant. From the left there are four throttle levers controlling the interconnected fuel flow and propeller RPM controls. These are duplicated on the right for the second pilot.

Fine Pitch Stop Lever

Between the throttles for the left and right engines is the fine pitch stop lever.

This lever is mechanically connected to the throttle levers so that when they are opened approximately half way the lever will jump forward to the Stop Armed position. For convenience the lever will return to the Stop Removed position when the throttles are closed and the weight is on the wheels, in reality this selection was made manually. Once the lever has moved to the Stop Armed position, when the throttles are opened beyond approximately 13000rpm, and the propellers have moved into Flight Fine Pitch, the Fine Pitch Stops will engage, preventing the propellers moving back into Ground Fine Pitch during flight when the throttles are retarded.  On touchdown, when the lever is returned to Stop Removed, the circuit is completed with the throttle microswitches and the Fine Pitch Stops will withdraw, allowing the propellers to move into the Ground Fine range.

The lights pertaining to the propeller locks open in a separate window by selecting the  icon. There are four Propeller Below Flight Fine lamps, which indicate when the propellers have moved below the Fine Pitch Stops into the Ground Fine range, and a single Fine Pitch Stop Withdrawn lamp, which illuminates when the stops have withdrawn. It is essential to monitor the lights on takeoff and landing to ensure correct operation.

High Pressure Fuel Cock Levers

In the centre of the quadrant are four High Pressure Fuel Cocks which directly control the supply of fuel to the engine. Each lever has three positions; Open in the fully forward position which introduces fuel under pressure to the engine; Shut in the centre, which cuts off the fuel supply to the engine; and Feather in the rearward position, which both cuts off the fuel supply and allows operation of the manual feathering controls. The mouse areas for each lever are adjacent to the position labels on the quadrant.

Feathering Controls

Manual feathering is accomplished using the Feathering Pump switches on the forward quadrant. The appropriate HP Cock must be in the feathering position for the feather pump to engage. A Feather Pump warning lamp for each engine

will light when the propeller is feathered. The switch is also used to unfeather a propeller when relighting.

Relight Switches

There are four Relight switches for operating the igniters in the air and a single warning lamp will light if any igniter is on.

To the rear of the quadrant is the pedestal, which is opened with the  icon.

Fire Warning Lamps and Extinguisher Discharge Switches

There are four engine Fire Warning lamps at the forward end of the pedestal, and a Master Fire Warning is fitted to the main panel . The guarded Fire Extinguisher switches are inoperative.

Low Pressure Fuel Cocks Switches

The LP cocks are electrically operated by four switches on the pedestal, and isolate the fuel supply upstream of the flowmeters. Each cock has an associated magnetic indicator on the fuel panel which shows if it is open or closed.

Fuel Trim Switches and Indicators.

The purpose of the Fuel Trimming system is to allow the highest power output for the ambient conditions without exceeding TGT limitations. The trimmers , when set below the 100% untrimmed position, reduce the fuel flow without altering RPM, by varying the datum of the interconnected throttle and propeller controls.

The trimmers should be set to 50% for normal engine starting, then trimmed in accordance with the chart to ambient conditions. After takeoff, the setting should be as close to 100% as possible , without exceeding TGT limitations.

A mouse area between switches 2 and 3 allow all trimmers to be set simultaneously, the tooltip showing the average setting.  Note that the Fuel Trim must be correctly set for a successful start or relight to be made. The engines will fail if the TGT limitations are grossly exceeded.

7.FUEL PANEL

The Fuel Panel is opened with the  icon.

Fuel System

The Argosy has four fuel tanks consisting of groups of cells, although the military versions were equipped with auxiliary long range tanks. This aircraft has a maximum capacity of 800 imperial gallons ( 960 USG/6400lb ) in the outboard tanks, and 930 imperial gallons ( 1116 USG/7440lb) in the inboard tanks giving a total of 3460 imperial gallons ( 4152 USG/29120lb ). All of the instrumentation pertaining to the fuel and water-methanol systems is calibrated in pounds.

There are two Booster Pumps in each tank group, which raise the pressure of the fuel for delivery to the engine. It is normal practice to switch both on, although either will provide sufficient pressure. Each engine has a red low fuel pressure warning lamp which is connected to the CWS.

Crossfeed Cocks enable fuel to be drawn from any tank or tanks, although normally each tank feeds its respective engine. To operate the crossfeed system, open all the Crossfeed Cocks, and observe the magnetic indicators are in line with the mimic diagram, then switch off the Booster Pumps in all the tanks except those which are to be used. Always ensure the Booster Pumps are all on before closing the Crossfeed Cocks or fuel starvation may result.

The four additional magnetic indicators show the position of the LP Cocks.

Note that because the dolls-eye type magnetic indicators have only two positions, they will also show cross-line with electrical power off.

Water-Methanol

The characteristics of the Dart engine are such that an increase of 1’C in ambient air temperature produces a rise of approximately 4’C in jet pipe temperature. In order to remain within engine temperature limits, the rate of fuel flow must be reduced, by way of the Fuel Trimmers , with a resultant power loss. Water Methanol introduced in the first stage compressor, is used as a power restorative in conditions where ambient air temperature/pressure would otherwise limit the performance of the engine on take off or go around. The W/M effectively cools and densifies the pre combustion air before being burnt in the normal way.

The system may be required if ambient conditions are substantially higher than ISA, and the pumps should be selected on while taxying, ensuring the green pressure warning lamps come on. Injection will take place at engine speeds above 14500 RPM if conditions dictate and fuel trim is set correctly. The system should be switched off at a safe height after climb power has been selected. Selection of climb power will stop the injection. The system should also be selected on in the approach if there is a possibility it may be required for an overshoot. There are two contents gauges which are set to 500lb each side by default. Clicking on the relevant gauge will replenish the tanks.

8.GENERATOR PANEL

The generator panel is opened with the  icon.

28VDC Supplies

28VDC power is supplied by four 9kw generators driven by their respective engine accessory gearboxes. Each generator has an associated control switch, failure warning lamp connected to the CWS, and ammeter. A single voltmeter can be used to display the output voltage of any one generator, or the batteries, through the voltmeter rotary switch.

A battery isolation switch, when selected to Isolate disconnects the batteries from the main busses, and simulates the connection of ground power. The red Isolate warning lamp is connected to the CWS. Note that with the switch in this position, services will be available which would not otherwise be on battery power alone, but power will last only as long as the battery voltage is greater than 13V.

9.STARBOARD OVERHEAD

The Starboard Overhead panel is opened with the  icon.

115VAC Supplies

115 volts AC is required for much of the instrumentation and for the autopilot and is supplied by three inverters controlled by their individual three position switch on the starboard overhead panel. The switches should be set to on, however when set to Failure, relays operate to allow an element of load transfer;

No1 set to Failure: No2 Takes Load

No2 set to Failure: No3 Takes Load

No3 set to Failure: No2 Takes Load

No2 can only replace No1 OR No3 but not both.

The inverters are supplied by the main 28VDC system, therefore will fail when bus voltage drops below 25V. On the ground, selection of the Battery Isolation Switch to Isolate will override the batteries and power the inverters for as long as battery power is available. Each inverter has a red failure warning lamp connected to the CWS, and individual output can be checked on the adjacent voltmeter through the selector switch.

Inverter failure will deprive the aircraft of the following systems:

No1:

Director Horizon                       Beam compass                         Autopilot*

VOR/ILS 2                                 VHF 1

No2:

ADF 1 & ADF2                          Servo Altimeter                       Standby Attitude

PUD Cyclic Timers 1 & 4         Oil Gauges 1 & 4                     VOR/ILS 1

VHF2                                          Transponder

No3:

PUD Cyclic Timers 2 & 3        Fuel Contents                            W/M Contents

Oil Gauges 2 & 3                      Flowmeters

*Autopilot not available with load transfer.

Air Conditioning

Air for the pressurisation of the cabin is supplied by compressors mounted on engines 1, 2 and 4, and a two position magnetic indicator shows white in the case of a failure. Gauges are provided for cabin height and differential pressure as well as a cabin rate of climb/descent indicator. On the bottom left of the panel is the pressure controller, which has two pointers, the longer of which should be set to the anticipated cruising altitude using the setting knob. The short pointer will show the cabin height provided at that altitude.

Immediately after takeoff, the Dump Valve knob should be rotated until the three position magnetic indicator shows Shut, then close nos1 and 2 Air Supply Master switches to initiate the pressurisation. No4 Air Supply should be closed when cruising above 15000ft and opened again passing that altitude in the descent. During the descent set the pressure controller to airfield height + 1000ft.

After landing, open the Dump Valve fully and open the Air Supply switches.

The cabin temperature and air supply can be adjusted by repeatedly clicking on the respective switches in the desired direction.

10.PORT OVERHEAD

The Port Overhead panel is opened with the  icon and houses the anti atmospheric controls.

Power Unit Deicing

An alternator mounted on each engine and taking its supply from the main DC system provides 208V power for the deicing of the engine air intakes, oil cooler intakes, propeller blades and spinners. The system operates through a cyclic timer, which supplies the current to different parts of the power unit cyclically, so that ice is allowed to form while the current is off, and is shed in small fragments when current is supplied, rather than continuously melted ice flowing rearward and refreezing on unprotected parts of the engine and airframe.

The alternators each have a master switch, which is normally left in the Run position, an undervolt/earth leakage warning lamp and a failure lamp.

The PUDs are controlled by individual switches, and an amber warning lamp will show steady to indicate a failure of the PUD, or will glow dim and bright in accordance with operation of the cyclic timer.

The system should be selected on if the OAT is below 10’C unless it is certain that icing conditions do not exist.

Note that the system has been arranged so that ice ingestion due to lack of ice protection will cause the engines to flame out.

Tail Unit Deicing

The tail is electrically heated, either continuously or cyclically and is controlled by a single switch, and a timer switch which should be set in accordance with the OAT. The Ice Warning lamp will illuminate if ice is detected and any of power unit or airframe systems are not switched on.

Mainplane Deicing

The wings are thermally deiced using hot air bled from nos 2 and 3 engines which is passes through a heat exchanger prior to entering the leading edge ducting. Two duct temperature indicators are provided and two overheat warning lamps will illuminate whenever a duct temperature exceeds 180’C.

It is essential that should an inboard engine fail, the flap on that side be closed immediately, otherwise cold air will be allowed into the system thus diluting the hot supply from the other side. The system must be switches on below +10’C unless it is certain that icing conditions do not exist. On the ground the flaps should never be opened until full power has been attained and the aircraft is accelerating, otherwise there may be insufficient airflow to prevent the ducts overheating.

Windshield Heating

The windshield is heated normally by an electrical supply from No1 alternator, however in the event of failure, switching allows either No3 or 4 alternators to supply the system. The Heat Increase lamp will illuminate if there is insufficient protection, requiring the switch to be set to 2/3 or Manual. An overheat warning is also provided but is not connected to the CWS.

Fuel Heaters

Each engine has a Fuel Filter Heater which should be used during the approach or during flight if icing is anticipated or the OAT is 20’C or less. Additionally the heaters should be selected On while taxying for takeoff if the OAT is below 5’C. As use of the fuel heaters causes a significant power loss, therefore they must be off for take off and landing.

The red Filter Blocked lamps are connected to the CWS, and the heaters must be selected on immediately the warning occurs.

11.REAR OVERHEAD

The rear overhead panel contains the starting controls, and those for the internal and external lighting. (See Engine & Flight Handling).

12.RADIO PANEL

The radio panel is fitted with dual VHF Comm./Nav equipment as well as dual ADF receivers, and switching allows either to be selected to the RMI.  Note that the relevant inverters must be functioning for the radios to be available. Beneath the radio controllers are the switches for the landing lamps, and pitot heaters. The door warning lamps may be pressed to operate the doors.

13. ENGINE AND FLIGHT HANDLING

* Note that the levers on certain game controllers appear to travel only         between 1% and  99%, the maximum and minimum detents must be selected

with the keyboard.

Engine Starting

Having completed the Before Start checks:

- Fuel Trimmers..........................................................................SET

- Brakes.........................................................................................ON

- Throttles...........................................................................CLOSED

- HP Cocks..........................................................................CLOSED

- Booster Pumps.........................................ALL ON, LAMPS OUT

- Prop Lamps..........................................................GROUND FINE

- Start Master........................................................................START

- Engine Selector..............................................................SELECT 4

- Propellers...................................................................ALL CLEAR

- Starter Button......................................................................PRESS

...........STARTER LAMP ON

...........1200-1500 RPM HP COCK OPEN

...........CHECK OIL PRESSURE, FUEL FLOW

...........STARTER LAMP OFF appx. 4500 RPM

...........MONITOR RPM and TGT

Repeat for engines 3, 2, and 1

- Start Master.............................................................................SAFE

- Engine Selector................................................................SELECT 4

- Battery Isol Switch................................................................NORM

Note that maximum TGT may be MOMENTARILY exceeded on startup, however if this appears to be the case , Fuel Trim must be reduced , then reset when the engine has stabilised.

Taxying

After completion of the Before Taxy checks,  open the throttles to approximately 11000 RPM and release the brakes. Once the aircraft is moving, the inners should be throttled back to idle, maintaining around 10500 RPM on the outers.

Take-Off

If there has been a change in OAT and/or ambient pressure between start up and take-off the Fuel Trimmers must be reset. When the Taxy and Before Take-Off checks have been completed and clearance received enter the runway and return the throttles to idle. At the commencement of take-off , open the throttles smoothly to approximately 12000 RPM, observing the Oil Pressure and TGTs,

then fully open the throttle to full power, 15000 RPM and ensure the minimum

torque has been achieved. Check that the TGTs are within the green arc, and that the Fine Pitch Stop lamps have extinguished.

Do not open the throttle too rapidly, or with the F4 key, as there is a danger that the Autofeather system will operate if sufficient power has not built up before the throttles are fully forward.

Rotation speed, Vr, is 7 knots below the safe climbout speed, V2 (see table).

Climb out initially at V2 to the acceleration height, Ha, usually 400ft above airfield level. Accelerate to the flap retraction speed, round about 135kt, then reduce power to 14200 rpm, 14500 rpm being max continuous. The climb performance of the model is a bit better than the real aeroplane, 500-1000ft/min gives 180-195kt. Complete the After Take-Off and Climb checks. Once established in the climb, the Fuel Trimmers should be set to as near 100% as possible, keeping within TGT limits.

Cruise

Cruise power should be left at 14200 RPM and the airspeed allowed to build up. Complete the cruise checks and continue to monitor the systems.

The Normal Operating Speeds Vno are:

Sea level – 10000ft  235kt

11000ft  231kt

12000ft, 227kt

13000ft  223kt

14000ft  219kt

15000ft  215kt

16000ft  211kt

17000ft  207kt

decreasing by 4 kt per 1000ft    25000ft  175kt

The Never Exceed Speed, Vne is :

Sea level – 10000ft  270kt

11000ft  265kt

12000ft, 260kt

13000ft  255kt

14000ft  250kt

15000ft  245kt

16000ft  240kt

17000ft  235kt

decreasing by 5 kt per 1000ft    25000ft  195kt

Max speed for operation of undercarriage or with undercarriage down 160kt

Max speed for flaps Up to Take Off                   180kt

Max speed for flaps Take Off to Approach       160kt

Max speed for flaps Approach to Land             140kt

Descent

The engines can overheat when the throttles are retarded as well as when accelerating , therefore the Fuel Trimmers must be set to full decrease, 0%, before the  power is brought back to 11000-11500 RPM for descent. Flight idle is 10400 RPM.  The aeroplane will descend comfortably at up to 2000ft/min.

Approach

For an ILS approach it is desirable to be level at approximately 2500ft clean, at around 170kt . As the glide slope deviation bar on the CDI approaches the centre of the instrument, select flap 12’ and extend the undercarriage, 11000-11500 RPM is sufficient to maintain the glide path, allowing the speed to reduce to 140kt.

Final Approach and Landing

Established in the final, the Fuel Trimmers, should be set to a figure consistent with the conditions at the destination airfield, the figure having been extracted from the table as early as possible. The flaps can be extended to 24’ below 160kt, and the speed 130kt, reducing power slightly if necessary. At approximately 600ft, select flap 40’, reducing speed to 10kt above the target threshold speed, adjust power to achieve the threshold speed as the fence is crossed.

On touchdown, close the throttles and ensure that the Fine Pitch Stops have withdrawn and the Below Stop lamps are all on.  If the lamps do not come on, under no circumstances may the throttles be opened.

During the After Landing checks, when clear of the runway, the outboard engines should be shut down by moving the HP Cocks for those engines to Shut, and power maintained at around 11000 RPM on the inboard engines, checking the speed with the brakes. Pull the No1 & 4 generator lamps to maintain the CWS.

Closing Down

On stand apply the parking brake and check the throttles are closed. Allow the TGT’s to stabilise and close the remaining HP Cocks. Turn off all the Booster Pumps and complete the Shutdown checks.

Argosy 222

Emergency Procedures

Manual Feathering

Should the need arise to shut down an engine in flight:

- HP Cock..........................................................TO FEATHER

- Feathering Switch..............................................................ON

- Throttle........................................................................CLOSE

Automatic Feathering

The Autofeather system will feather the propeller blades if the throttle is more than 4/5 open and the torque pressure is less 50 psi. If the system operates it MUST be followed by completion of the Manual Feathering Drill, i.e.

- HP Cock...........................................................TO FEATHER

- Throttle.........................................................................CLOSE

Flame Out

If propeller has not autofeathered:

- Throttle………………………………………………. CLOSE

- Relight………………………………….…ON (15 min MAX)

- Booster Pumps………………………………….. …..ALL ON

- Throttle…………………………………………………OPEN

- Relight………………………………………………….....OFF

- Anti Icing…………………………………….AS REQUIRED

Unfeathering and Relighting

Relighting in the air should be successful at any normal speed, however 140-160kt is optimal.

- Flaps......................................................................................................UP

- Fuel Trimmer....................AS PER OTHER TRIMMERS, 50% MIN

- Airframe Deicing (Inboard engine).................................................OFF

- Throttle ........................................................................................CLOSE

- Relight Switch......................................................................................ON

- HP Cock...........................................................................................OPEN

- Feathering Switch...........................................ON UNTIL LIGHT OUT

- Throttle.....................................OPEN SLOWLY UNTIL RPM RISES

- TGT..............................................................................................CHECK

- Relight Switch....................................................................................OFF

- Throttle...................................................OPEN TO MATCH OTHERS

- Fuel Trimmer.....................................................................................SET

Ice Ingestion

- Relights………………ON, UNAFFECTED ENGINES (15 min MAX)

If Propeller has not autofeathered…………………..FLAMEOUT DRILL

Otherwise……………………………………….………..RELIGHT DRILL

Late Anti Icing Selection

- Relights…………………………………………ALL ON, (15 Min MAX)

- PUDS……………………………………………………….NOS1 & 3 ON

If Engines run normally for 3 minutes:

- PUDS……………………………………………………….NOS2 & 4 ON

- Relights………………………...ALL OFF AFTER A FURTHER 6 Min

Fuel Starvation

Fuel Filter and/or Low Pressure lamps on:

- Fuel Heaters………………………………………..……………..ALL ON

- Booster Pumps …………………..ALL ON (EXCEPT EMPTY TANKS)

- LE Deicing…………………………………………ON ( +10’ & BELOW)

- RPM……………………………………..MAX CONTINUOUS IF POSS

- Fuel Contents………………………………………………..……CHECK

The information contained in this manual is based on AW Argosy data, The information is for Flight Simulation use only and should not be considered for use with the real aircraft

All brand names used throughout remain copyrights of their owners and are used as reference only.

Fraser A. McKay, October 2005