1. Limits of use – Weight – Speeds – Load factor

================================================

The Swift is a foot launch able glider designed to fly in lift.

Unlike other footlaunchable aircraft, with the Swift it is easy to fly much faster than the Vne (Never Exceed speed). To fly safely it is important to keep in mind the speed limits, and always stay under these limits.

- Aerobatic manoeuvres and spins are forbidden.

- Authorized manoeuvres:

- 60° of bank in the roll axis

- In pitch: - 30° nose up in respect to the horizon

- 30° in a dive in respect to the horizon

- Pilots weight range: 55 to 100 kg (120 à 220 lbs)

- Vne (Never Exceed speed): 120 km/h or 74 mph

- Vra (Maximum speed in Rough Air): 100 km/h or 62 mph

- Va (Manoeuvring speed)1: 85 km/h or 53 mph

- Vfe (Maximum speed with flaps set to 20° or more): 80 km/h or 50 mph

- Vs (Stall speed, flaps set to 0°) at maximum take-off weight: 37 km/h or 23 mph

- Maximum load factors: + 5,3 g/- 2,65 g (tested with 1.5 safety coefficient).

The flaps

===========

The flaps are controlled with rope and clamcleat.

Pulling the rope adds flaps deflection and slows the wing while pitching the nose up slightly. Thorough knowledge of all the flight characteristics of the Swift at various flap settings is necessary in order to fly the Swift safely.

Modify the flaps setting allows you to change your glide angle and in effect the minimum speed.

A mark on the control rope indicates 0° when lined up with the clamcleat end facing to the pilot. The angular value is shown on the front triangle sticker.

- Sudden release of the flaps can cause loss of altitude near the ground. You must check that the cord is well cleated before manoeuvring near the ground.

Experienced pilots keep their hand on this control to be able to adjust their glide angle.

You can control the 0° value of the flaps lining up the

elevons and the flaps with the tip fairings (winglets).

Take off

=========

1. In general

- Do not over control; the Swift tends to auto correct deviations due to turbulence in

part.

- Do not keep the controls in extreme positions (too much back stick, flaps at 30 or 40

degrees) this will decrease the efficiency of the wing

- Flaps 10 to 15 degrees, keep the joystick slightly forward. The machine takes off

very quickly.

- Quickly climb slightly above the tow machine, to get out of its turbulence.

- Trim with the flaps to reduce joystick pressure.

3.Winch

======

Flaps set from 10 to 15°, keep the joystick slightly forward. The machine takes

off very quickly.

- Make sure you have adequate speed in respect to height close to the ground in

case of a cable break.

- Trim with flaps do not go over 15 degrees.

- Keep the speed between 60 – 80 km/h (or 37 – 50 mph).

- Do not release under tension.

In Flight

==========

The Swift is easy to fly, and gives very good control authority.

- Beware of the speed: the Swift has very little drag and can rapidly go very fast. And

the fairings do not allow a pilot use to other forms of free flight to realize the speed.

- Consequence: always fly with a good air speed indicator!

The Swift has very little inertia in pitch but much softer roll and yaw characteristics.

There is a tendency for inexperienced pilot to pitch oscillate and over control. This is

not dangerous.

In general avoid ‘piloting’ too much the machine. The Swift flies very well on its

own. It flies in a straight and in a stable manner. All control actions decrease the

performance of the machine.

Adapt the flap setting and the speed to the situation:

- Take off: flaps 20° (10° in strong wind)

- In lift: flaps 10°, speed + / - 43km/h or 27 mph (min. sink)

- Best glide: flaps 0°, speed 65 to 75 km/h (40 to 47 mph) in no wind conditions.

- High speed: negative flap

- Never Exceed Speed (Vne): 120km/h or 74 mph

- Maximum speed in Rough Air (Vra): 100km/h or 62 mph

Landing

=========

Do an approach at the right speed: classic error is to come in too fast. In this

condition, the Swift is very long to land, the aircraft can fly all the field without landing or otherwise very violently. On the final approach, no faster than 50 km/h or 31 mph (depending of course of the weather conditions – estimated wind gradient, wind speed, turbulences).

- The more flap you have on, the slower you fly, and consequently the more

adverse yaw. The control response decrease at low speed. Avoid high bank turn near

the ground on approach.

It is best to do a classic glider approach (not an S-turn approach as in paragliders or

hang gliders)

- Down wind leg, flaps 0 to 20°

- Base leg, flaps 20°

- Final, flaps at 20° with a speed between 45 and 50 km/h (28 and 31 mph).

Technical data and speeds

==========================

Empty 48 kg / 106 lbs without fairing

Maximum take off weight 158 kg / 347 lbs (version without motor)

Span 12,8 m / 42 ft

Surface area 12,5 m² / 136 ft²

Aspect ratio 12,9

Sweep (at 25% of the cord) 20°

Recommended pilot weight 55 à 100 kg (120 à 220 lbs)

Vne (Never Exceed Speed) : 120 km/h or 74 mph

Vra (Maximum Rough Air Speed) 100 km/h or 62 mph

Va (Manoeuvre Speed) 85 km/h or 53 mph

Vfe (Maximum speed with flaps set to 20° or more) 80 km/h or 50 mph

Vs (Stall Speed, flap sets to 0°) at maximum take-off weight 37 km/h or 23 mph

Minimum Speed, flap sets to 20° at maximum take-off weight 32 km/h or 20 mph

Best Glide 27 : 1 @ 70 km/h / 43 mph

Minimum Sink 0,6 m/s @ 43 km/h / 27 mph

Load Factor: + 5,3 g/- 2,65 g (with 1.5 safety coefficient).

Wings made in composite materials:

The skin is in a sandwich of aramide/Carbon / epoxy / PVC foam

The spar is mainly carbon/epoxy

The cockpit frame is made in Cromoly (steel 4130), aeronautic quality, TIG welded, and aluminium tubes in Zicral alloy.

The Variometer

============