| GENERAL DESCRIPTION: The C-17 Globemaster III is a high-wing, four-engine, T-tailed military transport. |  |
| --- | --- |
| EXTERNAL DIMENSIONS |  |
| Wingspan to winglet tip | 169.8 feet (51.74 m) |
| Length | 174 feet (53.04 m) |
| Height at tail | 55.1 feet (16.79 m) |
| Fuselage diameter | 22.5 feet (270 inches) (6.86 m) |
| ENGINES |  |
| Four Pratt & Whitney PW2040 (military designation F117-PW-100) 40,440 pounds thrust each |  |
| CARGO COMPARTMENT |  |
| Cargo compartment crew | One loadmaster |
| Cargo floor length | 68.2 feet (20.78 m) |
| Ramp length | 19.8 feet (6.04 m) |
| Loadable width | 18 feet (5.49 m) |
| Ramp-to-ground angle | 9 degrees |
| Ramp capacity | 40,000 lbs. (18,144 kg) |
| Aerial delivery system capacity |  |
| Pallets | Eleven 463L pallets (including 2 on ramp) |
| Single load airdrop | 60,000-lb. platform (27,216 kg) |
| Sequential loads airdrop | 110,000 lbs. (49,895 kg) (60 feet of platforms) (18.29 m) |
| Cargo handling system | Eighteen 463L Pallets (including 4 on ramp) |
| Combat offload | All pallets from aerial delivery or cargo handling systems |
| SEATING |  |
| Sidewall (permanently installed) | 54 (27 each side, 18 inches wide, 24 inch spacing center to center) |
| Centerline (stored on board) | 48 (in sets of six back-to-back, 8 sets) |
| Palletized (10-passenger pallets) | 100 on 10 pallets, plus 54 passengers on sidewall seats |
| AEROMEDICAL EVACUATION |  |
| Litter Stations (onboard) | Three (3 litters each) |
| Litter Stations (additional kit) | Nine |
| Total Capability (contingency) | 36 litters, 102 ambulatory |
| COCKPIT |  |
| Flight crew | 2 pilots |
| Observer positions | 2 |
| Instrument displays | 2 full-time, all-function, head-up displays (HUD), 4 multi-function cathode-ray tube displays (CRT) plus conventional instruments as backup. |
| Navigation system | Digital electronics |
| Communication | Integrated radio management system |
| Flight controls | Quad-redundant electronic flight control with mechanical actuated backup system |
| WING |  |
| Area | 3,800 sq. ft. (353.03 sq. m) |
| Aspect Radio | 7.165 |
| Wing sweep angle | 25 degrees |
| Airfoil type | Supercritical |
| Flaps | Fixed-vane, double-slotted, simple-hinged |
| WINGLET |  |
| Height | 8.92 feet (2.72 m) |
| Span | 9.21 feet (2.81 m) |
| Area | 35.85 sq. ft. (3.33 m) |
| Sweep | 30 degrees |
| Angle | 15 degrees from vertical |
| HORIZONTAL TAIL |  |
| Area | 845 sq. ft. (78.50 sq. m) |
| Span | 65 feet (19.81 m) |
| Aspect ratio | 5.0 |
| Sweep | 27 degrees |
| LANDING GEAR |  |
| Main, type | Twin-strut tandem per side of aircraft, three wheels each strut |
| Width (outside to outside) | 33.7 feet (10.27 m) |
| Tires | 50x21-20 |
| Nose, type | Single strut, steerable with dual wheels |
| Tires | 40 x 16 - 14 |
| Wheelbase | 65.8 feet (20.06 m) |

| PERFORMANCE | C-17 |  | C-17 Extended Range |
| --- | --- | --- | --- |
| Maximum Takeoff Gross Weight | 585,000 lbs. (265,351.50 kg) |  | 585,000 lbs. (265,351.50 kg) |
| Empty Weight | 277,000 lbs. (125,645.10 kg) |  | 281,500 lbs. (127,686.30 kg) |
| Usable Fuel (@ 6.7 lb/gal) | 181,054 lbs. (82,124.71 kg) |  | 244,688 lbs. (110,988.60 kg) |
| Payload: Maximum | 169,000 lbs. (76,657.11 kg) |  | 165,900 lbs. (75,250.97 kg) |
| At 4,000 nautical miles | 57,000 lbs. (25,854.77 kg) |  | 101,000 lbs. (45,812.83 kg) |
| Range with Payload |  |  |  |
| 160,000 lbs. | 2,400 nautical miles |  | 2,400 nautical miles |
| 40,000 lbs. (paratroops) | 4,400 nautical miles |  | 5,300 nautical miles |
| Ferry (empty) | 4,700 nautical miles |  | 6,250 nautical miles |
| Service Ceiling | 45,000 ft. (13,716 m) |  | 45,000 ft. (13,716 m) |
| Cruise Speed | 0.74 Mach |  | 0.74 Mach |
| Takeoff Field Length (Max. gross wt.) | 7,740 ft (2,359.15 m) |  | 7,740 ft. (3,510.81 m) |
| Landing Field Length (160,000 lbs. of Cargo) | 3,000 ft. (914.40 m) | 3,000 ft. (1,360.78 m) |  |