DIGITAL MINI PANELS

FOR FS9 AND FSX

This package contains a set of gauges that display in “DIGITAL” style, most of the necessary information required for control and navigation of aircraft. They are particularly useful in approach and landing situations. For some aspects of control and navigation, the main panel will still be required, for example, there are no full manual start/stop, Com radio, mixture control or ATC facilities. Some autopilots may also have further features that are not included with the autopilot in this gauge.

The gauges are all inclusive and contain all the information in a single gauge (except for VOR1 and 2 gauges and an icon for switching panels, more on them later). There are three separate gauges for aircraft that have different types of engines, namely reciprocating, jet and turboprop. Then there is a choice of centigrade or farenheit for the engine parameters. Apart from the engine parameters, all other features are identical.

They are suitable for aircraft with up to four engines. If each engine is controlled separately, the parameters for each engine will be displayed. If the gauge is used on an aircraft with less than four engines, the information for the “missing” engines will display as 0 (zero).

INCLUDED GAUGES

This package contains 9 gauges:-

Digital Mini Panel (Recip Cent).xml	For aircraft with reciprocating

Digital Mini Panel (Recip Far).xml 	engines

Digital Mini Panel (Jet Cent).xml		For aircraft with jet engines

Digital Mini Panel (Jet Far).xml

Digital Mini Panel (Turbo Cent).xml	For aircraft with turboprop engines

Digital Mini Panel (Turbo Far).xml

For each engine type there is a choice of centigrade or farenheit display of the engine parameters, all other details are identical.

Digital VOR1.xml

Digital VOR2.xml

The VOR gauges are retained as separate gauges to allow for usage in other panels if required.

Main_Mini_Swap.xml

This gauge is a separate gauge and is to be used on both the selected mini panel and the main panel to allow for swapping between panels.

INSTALLATION

Copy the entire folder DIGITAL MINI PANELS to your FS9 or FSX “Gauges” directory.

Select the aircraft in which you wish to install the mini panel, and open the PANEL.CFG file for that aircraft.

At the top of this file will be a list of [Window Titles]. Add a new window to this list with the next  number. For example:-

Window04=ENGINES

Window05=COMPASS

Window06=THROTTLE_PANEL

Window07=DIGITAL MINI PANEL

Scroll down until you reach the current last window entry (in the above example it would be [Window06], then leaving a line AFTER the last entry for that window, ADD ALL OF THE FOLLOWING BETWEEN THE LINES ONLY (Copy and Paste from here to avoid mistakes). Remember to change the window number to the same number you gave it in the [Window Titles] section.

[Window07]

Background_color=0,0,0

size_mm=1024,768

window_size_ratio=1.000

position=0

visible=1

ident=1936

window_size= 1.000, 1.000

window_pos= 0.000, 0.000

gauge00=Digital Mini Panels!Digital Mini Panel (Recip Cent),  92,550,840,218

gauge01=Digital Mini Panels!Digital VOR 1,  354,633,100,100

gauge02=Digital Mini Panels!Digital VOR 2,  474,683,100,50

gauge03=Digital Mini Panels!Main_Mini_Swap,  114,571,19,19

NOTE: This is where you need to change the type of gauge you want (Reciprocating, Jet or Turboprop and whether you want Centigrade or Farenheit engine displays). Just change the wording between the brackets in gauge00, to agree with the gauge names above, being careful not to alter anything else.

Now you need to place the panel swap icon into your main panel.

Scroll back up to the main panel window, (usually [Window00], and add the following line, AFTER THE LAST GAUGE ENTRY. This will place the panel swap icon into your main panel

gaugeXX=Digital Mini Panels!Main_Mini_Swap,  0,0,19,19

Replace the XX with the next number in sequence.

The first 2 numbers at the end of the line represent the horizontal and vertical distances in and down from the top left corner. You may wish to place them somewhere else, so you will have to experiment with these 2 numbers. As an example, on a 1024x768 panel, 1005 and 749 would place the icon in the bottom right corner. In this case the line should read:-

gaugeXX=Digital Mini Panels!Main_Mini_Swap,  1005,749,19,19

Similarly, the last 2 numbers represent he horizontal and vertical size of the icon. 19 is about right for a 1024x768 panel, but you can change these as required.

That’s all for the installation process.

FEATURES

Most displays on the panel will be obvious, but a few might need some explanation.

Firstly mouse clicking on ALL the buttons will activate/de-activate that feature.

Secondly for displays that are adjustable, clicking in the red shaded areas will decrease that selection and clicking in the blue shaded area will increase that selection.

Thirdly, if a particular feature is not modelled in the aircraft, the button or selection area will not function.

The full panel gauge

The icon in the top left corner swaps the digital mini panel and the main panel. There is a matching icon on the main panel.

Towards the lower right of the panel, the BAT and AV buttons switch the battery and avionics masters on. There will be no display until both these functions are activated.

Now back to the top left and we will work downwards and to the right.

Furthest left is the bank of light switches.

Then ground speed and true heading displays

Flaps, spoilers and elevator trim are clickable on the display to increase/decrease the respective function. Clicking on the ARM button will arm the spoilers.

The OAT displays the outside temperature in °C and °F

The main bus displays the voltage and amps

The simulation rate is clickable on the display to increase/decrease the rate.

Time functions included are local and UTC time as well as a flight timer. The flight timer will start automatically when the indicated air speed is above 35 knots and will stop when the IAS is below 35 knots. This is to discount taxiing time and to eliminate a headwind starting the timer at wind speeds below 35 knots.

The wind dir/speed function displays the direction and speed in knots of the prevailing wind.

The panel at the top displays the equivalent of the primary flight instruments plus a GPWS and the position of the elevators. Note that this is the position of the elevators only and not an indication of the pitch of the aircraft. The actual pitch is given to the right of centre on the bottom.

The bank angle is displayed in digital format in degrees with left or right arrows displaying the direction of turn. If the bank angle exceeds 25 deg the arrows will change to red.

The ground proximity warning system (GPWS) will indicate by changing to a flashing red colour, that you are less than 350ft above the ground.

The ADF display includes the selected frequency of the NDB and the bearing to that station. The frequency is selectable by clicking the upper half of the frequency display to increase and the lower half to decrease. The frequency is selectable by hundreds, tens, units and fractions, by clicking over those respective areas.

The VOR1 gauge (separate gauge) is basically a standard gauge with standard features, made to look a bit more “digital”. The OBS is selectable by mouse clicking.

Like the VOR1, the VOR2 gauge (separate gauge) is basically a standard gauge made to look a bit more “digital”. It has a localizer but no glidescope. The OBS is selectable by mouse clicking.

The Top of Climb/Bottom of Descent display shows the calculated time (in minutes) and distance (in nautical miles) to the top of climb or the bottom of descent based on the altitude and vertical speed that has been input into the autopilot, and the current ground speed. If any of these figures change the display will continually re-calculate to provide real time information.

Current Position is the current geographical position of the aircraft.

The NAV1 and NAV2 radio displays show the current active and standby frequencies. The frequencies are selectable by clicking the upper half of the frequency display to increase and the lower half to decrease. The frequency is selectable by units and fractions, by clicking above those respective areas. The active frequencies are not directly selectable, they must be selected as “standby” then swapped.

Clicking either of the active frequency displays, (in this case  ) will swap the active and standby frequencies.

The engine parameter display section shows various values for different aspects of engine management for up to four engines. Not all available information is displayed here, and you may need to switch to the main panel, or to another panel, to view other information. If the aircraft has less than 4 engines the values for the “non-existent” engines display as 0 (zero). It is interesting to note that the MP (manifold pressure) of the “non-existent” engines will reflect the drop in atmospheric pressure as the altitude increases. The example shows the gauge fitted to a single engine aircraft.

Depending on the type of engines fitted, and the choice of the appropriate gauge, different parameters will be displayed. In each case, the labels for each of the parameters should be self explanatory

Note that the throttle position value  is the positional setting of the throttle levers, not the true power output of the engine(s) at any given instant, which will always lag behind the setting position.

The total fuel and section displays the total fuel used and the total fuel remaining. The range section shows the distance (nautical miles) and time (hours and minutes) to empty, based on current usage.

The pause and sound buttons toggle the pause and sound functions off/on.

The autopilot buttons activate the various autopilot functions. The heading, altitude and vertical speed inputs for the autopilot can be selected by clicking on the left or right of the selector knobs. The NAV1 display is a relay from the OBS selector on the VOR1 gauge.

The A_STRT button operates the autostart function. The brake and gear buttons activate the parking brake and the landing gear respectively.

LEGAL

These gauges are FREEWARE and should not be used for any commercial purpose. If they are to be used, whole or in part, within any aircraft package, this document should be included in full as part of that package and the author’s permission should be sought prior to publication.

Darrel Woodhouse 2010

d.woody@bigpond.com.au