- Emerald Air  –  A true Gem among VA's

- Boeing 757 - 200 RR eng.

- Aircraft operation and flight manual

- Aircraft Category

- Pilot Qualification Category:	4	(heavy jets)

- Route Utilization Category:	3-4	(medium to long-haul)

- Approach Speed Category:	D	(up to 165kts)

- ICAO Landing Capability:		IIIa

- Aircraft Description

The Boeing 757 was developed in the late 70’s to replace the aging 727. The aircraft born, however, surpassed all expectations and set up a new category combining the advantages of single-aisle aircraft and wide-bodied heavies. The 757 is capable of serving a multitude of roles from shuttle services between nearby hubs to intercontinental point-to-point traffic with outstanding efficiency. The 757-200 carries 200-230 passengers to 3.900 nautical miles. It’s got advanced aerodynamics, high.bypass Pratt & Whitney or Rolls-Royce turbofan engines and a semi-glass cockpit common with its bigger cousin, the Boeing 767.

- Aircraft Specifications

- Dimensions:

- Length:	155 ft 3 in	47.3 m

- Wingspan:	124 ft 9 in	38.0 m

- Height:	44 ft 6 in	13.6 m

- Capacity:

- Cabin:	195 passengers + 6 crew

- Cargo bay:	1,670 ft3	47.3 m3

- Fuel tanks:	11,276 US Gal	42,680 L

- Weights:

- Empty Weight:	132,500 lb	60,100 kg

- Zero Fuel Weight:	168,545 lb	76,450 kg	(current flight model set to  75% payload)

- Max TakeOff Weight:	255,000 lb	115,660 kg	(max fuel quantity 100% at 75% payload)

- Max Landing Weight:	209,500 lb	95,025 kg	(max fuel quantity   55% at 75% payload)

- Engines:

- Number & Type:	 2 × Rolls-Royce RB211-535 E4 turbofans located under the main wings

- Thrust:	2 × 40,200 lb	179 kN	APR

- Avionics:

4-Channel Autopilot with Heading, VOR Radial and GPS Course tracking + Cat IIIa automatic approach and landing capability + automatic IAS and Mach hold through integrated Autothrottle control.

Structural Limitations:

- Max. Operating Mach Number (MMO):	M 0.86

- Max. Operating Airspeed Limit (VMO):	350 KIAS

- Max. Flaps Extended Speed (VFE):

01° (1 notch)	240 KIAS

05° (2 notch)	220 KIAS

15° (3 notch)	210 KIAS

20° (4 notch)	195 KIAS

25° (5 notch)	190 KIAS

30° (6 notch)	162 KIAS

- Max. Gear Extended Speed (VLE):	270 KIAS

- Max. Gear Operating Speed (VLO):	250 KIAS

- Max. Tire Speed on Ground (AFM):	195 KIAS

- Stall  Speed with Full Flaps (VSO):	112 KIAS

- Performance Data:

- Max. Cruise Speed:	480 kts	888 km/h

- Max. Certified Altitude:	42,000 ft	12,800 m

- Max. Range:	3,900 nm	7,228 km	(without reserve fuel)

- Runway Requirements:

- Min. TakeOff Runway Length:	7,220 ft	2,200 m	(at Sea Level, MTOW)

- Min. Landing Runway Length:	7,220 ft	2,200 m	(at Sea Level, MLW)

- Max. T/O & Land Crosswind Component:	30 kts

- Max. T/O & Land Tailwind Component:	10 kts

- Flight Characteristics

- Takeoff:

T/O Weight		Fuel	Flaps	Power	V1	VR	V2

193,110 lb	  87,594 kg	33%	05°	93% N1	139	142	147	KIAS

217,675 lb	  98,736 kg	66%	05°	94% N1	148	151	154	KIAS

242,240 lb 	109,878 kg	99%	05°	95% N1	154	157	159	KIAS

- Note: Never exceed 100% N1 in Takeoff Phase.

- Climb (100% fuel):

Altitude	Power	Airspeed	Mach	Climb Rate

< 10,000 ft	92% N1	300 KIAS		3,800 fpm

< 15,000 ft	92% N1	300 KIAS		3,300 fpm

< 20,000 ft	92% N1	300 KIAS		2,500 fpm

< 25,000 ft	92% N1	300 KIAS		1,800 fpm

< 30,000 ft	91% N1		M 0.70	1,500 fpm

< 35,000 ft	91% N1		M 0.70	1,200 fpm

< 40,000 ft	91% N1		M 0.70	   900 fpm

Note: Never exceed 95% N1 in Climb Phase. Comply with speed restrictions under FL 100.

Typical Cruise (FL 350):

Cruise	Power	Mach	KIAS	KTAS	Fuel pConsumption

Normal	81% N1	M 0.80	277	460	10,500 PPH	1,590 GPH

Fastest	89% N1	M 0.83	290	480	12,000 PPH	1,818 GPH

- High Altitude Cruise (FL 400):

Cruise	Power	Mach	KIAS	KTAS	Fuel Consumption

Normal	82% N1	M 0.80	247	460	8,500 PPH	1,288 GPH

Fastest	89% N1	M 0.83	257	475	9,500 PPH	1,439 GPH

- Note: Never exceed 92% N1 in Cruise Phase.

- Descent:

Altitude	Power	Airspeed	Mach	Descent Rate

> 25,000 ft	45% N1		M 0.70	2,500 fpm

< 25,000 ft	50% N1	300 KIAS		2,500 fpm

Note: Let airspeed build up then apply speed brakes when necessary. Comply with speed restrictions under FL 100.

- Approach:

Distance	Flaps	Power	Airspeed	Descent Rate	(3° G/S)

LOC active	15°	49% N1	200 KIAS

G/S  active	20°	49% N1	180 KIAS	945 fpm

Outer Marker 	30°	49% N1	150 KIAS	790 fpm

- Note: Reduce your airspeed and descent rate by deploying flaps gradually.

- Landing:

Landing Weight	Fuel	Flaps	Power	VTT

179,710 lb	81,515 kg	15%	30°	41% N1	135	KIAS

194,600 lb	88,070 kg	35%	30°	43% N1	141	KIAS

209,490 lb 	95,025 kg	55%	30°	45% N1	146	KIAS

Note: At 25 feet AGL cut throttle and flare. On touchdown gently push forward yoke to bring the nose wheel down. When the nose gear is down, apply reversers and spoilers. At 60 kts disengage reversers and apply full wheel brakes. Do not attempt to land over certified MLW!

- Fuel Management

- average fuel consumption during climb from SL to FL 350:	22,000 PPH	3,333 GPH

- average fuel consumption during normal cruise at FL 350:	10,500 PPH	1,590 GPH

- average fuel consumption during descent from FL 350 to SL:	  4,450 PPH	   675 GPH

- Fuel required to cover a given distance (at FL 350, no headwind, D = distance in nm):

5500 + 22.8×(D-180) + 1040 lbs	833 + 3.45×(D-180) + 157 Gal

IMPORTANT NOTICE: the above manual describes the flight characteristics of the aircraft as modeled in Microsoft Flight Simulator 2000™. Under no circumstances may any part of this manual be used in real life aviation!