Boeing-McDonnell-Douglas F-15C Eagle,

83-020 of the 325th Fighter Wing at Tyndall AFB, Florida

### F-15 from the 325th Fighter Wing, Tyndall AFB, Florida

### Fort Worth, Texas

The 325th Fighter Wing, comprised of 4,100 personnel and 78 F-15 aircraft, trains pilots, controllers, and support personnel for worldwide assignment to combat air superiority units. 

Wing personnel also manage the southeastern air combat maneuvering instrumentation range and provide mission-ready F-15 air superiority forces in support of Commander in Chief North American Aerospace Defense Command/l st Air Force contingency plans. The wing is composed of the 1st, 2nd and 95th Fighter Squadrons. 

The wing was activated at Tyndall Air Force Base, FL, in July 1981 as the 325th Fighter Weapons Wing. Its heritage was enriched by adoption of the emblems, awards and history of its predecessor, the 325th Fighter Group "Checkertail Clan" of World War II fame. 

The original fighter group was activated August 3, 1942, at Mitchel Field, NY, and trained with P-40s. It entered combat April 17,1943, escorting medium bombers, flying strafing missions and making sea sweeps from bases in Algeria and Tunisia. The unit received its first Distinguished Unit Citation for its surprise attack on German airdromes over Villaroba Jan. 30, 1944. In May 1944, the P-47s were replaced by P-51 Mustangs which unit personnel flew until the end of World War 11.By then the 325th's motto - Lacare Et Liquidare (Locate and Liquidate) - had earned the respect of the allies and Germans. 

After the 325th's return from Europe it underwent a series of inactivations, reactivations and designations known at times as the 325th Fighter Group (All-Weather) and 325th Fighter Interceptor Group. Assigned aircraft included P-61s in 1947, F-82s in 1948 and F-94s in 1950. 

Then on August 18, 1955, the group was reactivated as the 325th Fighter Group (Air Defense), assigned to Air (later Aerospace) Defense Command, and equipped with F-86 Sabre jets. Its tactical units were the 317th and 31 8th Fighter Interceptor Squadrons. 

On October 18, 1956, the Air Defense Command directed a wing organization be set up at McChord Air Force Base, WA, and the 325th Fighter Wing (Air Defense) was activated. The group became a unit of the new wing. The group's two tactical squadrons, the 317th and 318th Fighter Interceptor Squadrons, transitioned from the F-86 to the F-1 02A delta-wing, all-weather interceptor. 

In August 1957, the 317th Fighter Interceptor Squadron was assigned to Alaskan Air Command, and the 325th gained the 64th Fighter Interceptor Squadron from Alaska. Early in 1960, the 325th Fighter Group began a gradual phase-out of the F-1 02 and a transition to the F-1 06 Delta Dart. Before the completion of this transition, the 325th Fighter Group was discontinued March 25, 1960. 

With the activation of the unit at Tyndall in July 1981 as the 325th Fighter Weapons Wing, the wing accomplished the operations, test and evaluation and maintenance portions of the complex United States Air Force Air Defense Weapons Center mission, which was directly related to combat readiness training for air defense. Primary aircraft were the F-106 and T-33. 

On October 15,1983, it was redesignated the 325th Tactical Training Wing, and assumed its air superiority training responsibilities as part of the United States Air Force Air Defense Weapons Center. The wing began receiving the F-15 Eagle in April 1984. 

When the United States Air Force Air Defense Weapons Center was inactivated September 12, 1991, the 325th Tactical Training Wing assumed the role as installation host. It was redesignated the 325th Fighter Wing October 1, 1991.

# F-15 Eagle

The McDonnell Douglas (now Boeing) F-15 Eagle is a twin-engine, all-weather tactical fighter designed to gain and maintain air superiority in aerial combat. It is considered among the most successful modern fighters with over 100 aerial combat victories.[3][4] Following reviews of proposals, the United States Air Force selected McDonnell Douglas' design in 1967 to meet the service's need for a dedicated air superiority fighter. The Eagle first flew in July 1972, and entered service in 1976. The F-15 is expected to be in service with the U.S. Air Force until 2025.[5]

Since the 1970s, the Eagle has also been exported to Israel, Japan, and Saudi Arabia. Despite originally being envisaged as a pure air superiority aircraft, the design proved flexible enough that an all-weather strike derivative, the F-15E Strike Eagle, was later developed, and entered service in 1989.

### Origins

Following studies in 1964-1965, the U.S. Air Force developed requirements for an air superiority fighter in October 1965. Then on 8 December 1965, the Air Force issued a request for proposals (RFP) for the new fighter. The request called for both air-to-air and air-to-ground capabilities. Eight companies responded with proposals. In the following study phase, four of these companies developed some 500 design concepts. Typical designs featured variable-sweep wings, weighed over 60,000 lb (27,200 kg), included a top speed of Mach 2.7 and a thrust-to-weight ratio of 0.75.[6] The designs were not accepted by the Air Force as they compromised fighter qualities for ground attack qualities.[7] Acceptance of the Energy-Maneuverability (E-M) theory by the Air Force led to a change in requirements for improved maneuverability by the spring 1967. The design mission weight was reduced to 40,000 lb (18,100 kg), top speed reduced to Mach 2.3-2.5 and thrust-to-weight ratio increased to 0.97.[6]

In 1967 U.S. intelligence was surprised[8] to find that the Soviet Union was building a large fighter aircraft, the MiG-25 'Foxbat'.[9] It was not known in the West at the time that the MiG-25 was designed as a high-speed interceptor, not an air superiority fighter,[10] so its primary asset was speed, not maneuverability. The MiG-25's huge tailplanes and vertical stabilizers (tail fins) hinted at a very maneuverable aircraft, which worried the Air Force that its performance might be better than its U.S. counterparts. In reality, the MiG's large fins and stabilators were necessary to prevent the aircraft from encountering inertia coupling in high-speed, high-altitude flight.

The F-4 Phantom II of the USAF and U.S. Navy was the only fighter with enough power, range, and maneuverability to be given the primary task of dealing with the threat of Soviet fighters while flying with visual engagement rules.[9] As a matter of policy, the Phantoms could not engage targets without positive visual identification, so they could not engage targets at long ranges, as designed. Medium-range AIM-7 Sparrow missiles, and to a lesser degree even the AIM-9 Sidewinder, were often unreliable and ineffective at close ranges where it was found that guns were often the only effective weapon.[11] The Phantom did not originally have a gun, but experience in Vietnam led to the addition of a gun. An external gun pod was tried and later the M61 Vulcan was integrated internally on the F-4E.

### [edit] F-X program

There was a clear need for a new fighter that overcame the close-range limitation of the Phantom while retaining long-range air superiority. After rejecting the U.S. Navy VFX program (which led to the F-14 Tomcat) as being unsuited to its needs, the U.S. Air Force issued its own requirements for the Fighter Experimental (F-X), a specification for a relatively lightweight air superiority fighter.[12] The requirements called for single-seat fighter having a maximum take-off weight of 40,000 lb (18,100 kg) for the air-to-air role with a maximum speed of Mach 2.5 and a thrust to weight ratio of nearly 1 at mission weight.[13] Four companies submitted proposals, with the Air Force eliminating General Dynamics and awarded contracts to Fairchild Republic, North American Rockwell, and McDonnell Douglas for the definition phase in December 1968. The companies submitted technical proposals by June 1969. The Air Force announced the selection of McDonnell Douglas on 23 December 1969.[14] The winning design resembled the twin-tailed F-14, but with fixed wings. It would not be significantly lighter or smaller than the F-4 that it would replace.

F-15A cockpit

The Eagle's initial versions were designated F-15A for the single-seat configuration and F-15B (originally TF-15A, but this designation was quickly dropped, as the F-15B is fully combat-capable) for the twin-seat. These versions would be powered by new Pratt & Whitney F100 engines to achieve a combat thrust-to-weight ratio in excess of 1 to 1. A proposed 25 mm Ford-Philco GAU-7 cannon with caseless ammunition was dropped in favor of the standard M61 Vulcan gun due to development problems. The F-15 retained conformal carriage of four Sparrow missiles like the Phantom. The fixed wing was put onto a flat, wide fuselage that also provided an effective lifting surface. Some questioned if the zoom performance of the F-15 with Sparrow missiles was enough to deal with the new threat of the high-flying MiG-25 "Foxbat", but its capability was eventually demonstrated in combat.

McDonnell Douglas F-15A (S/N 71-0280) first flight.

The first F-15A flight was made in July 1972 with the first flight of the two-seat F-15B (formerly TF-15A) following in July 1973.[15]

The F-15 has a "look-down/shoot-down" radar that can distinguish low-flying moving targets from ground clutter. The F-15 would use computer technology with new controls and displays to lower pilot workload and require only one pilot to save weight. Unlike the F-14 or F-4, the F-15 has only a single canopy frame with clear vision forward. The USAF introduced the F-15 as "the first dedicated USAF air superiority fighter since the F-86 Sabre."[16]

The F-15 would be favored by customers such as the Israel Air Force and Japan Air Self-Defense Force. However, criticism from the fighter mafia that the F-15 was too large to be a dedicated dogfighter, and too expensive to procure in large numbers, led to the Lightweight Fighter (LWF) program, which led to the USAF F-16 Fighting Falcon and the middle-weight Navy F/A-18 Hornet.[17]

### [edit] Further development

USAF F-15C taxiing for takeoff.

The single-seat F-15C and two-seat F-15D models entered production in 1978 with the models' first flights in February and June of that year.[18] These new models have Production Eagle Package (PEP 2000) improvements, including 2,000 lb (900 kg) of additional internal fuel, provision for carrying exterior conformal fuel tanks and increased maximum takeoff weight of up to 68,000 lb (30,700 kg).[19] The additional takeoff weight allows internal fuel, a full weapons load, conformal fuel tanks, and three external fuel tanks to be carried. Other improvement on the C and D models included strengthened landing gear, radar improvements, and a new digital central computer.[20] An overload warning system was also added, which allows the pilot to fly the fighter to 9 g at all weights.[18]

The F-15 Multistage Improvement Program (MSIP) was initiated in February 1983 with the first production MSIP F-15C produced in 1985. Improvements included an upgraded central computer; a Programmable Armament Control Set, allowing for advanced versions of the AIM-7, AIM-9, and AIM-120A missiles; and an expanded Tactical Electronic Warfare System that provides improvements to the ALR-56C radar warning receiver and ALQ-135 countermeasure set. The final 43 F-15Cs included the enhanced-capability Hughes APG-70 radar, which was developed for the F-15E. The earlier MSIP F-15Cs with the APG-63 were later upgraded to the APG-63(V)1, which significantly improves reliability and maintainability while providing performance similar to the APG-70. The improvements were retrofitted to existing F-15s.[21]

In 1979, McDonnell Douglas and F-15 radar manufacturer, Hughes, teamed to privately develop a strike version of the F-15. This version competed in the Air Force's Dual-Role Fighter competition starting in 1982. The F-15E strike variant was selected for production in 1984.[22]

Beginning in 1985, F-15C and D models were equipped with the improved P&W F100-220 engine. It added a digital engine control to allow for quicker throttle response, less wear, and reduced fuel burn. The original F100-100 engines were upgraded to a similar configuration with the designation F100-200E starting in 1997 and were ongoing as of 2007.[23]

Recent upgrades include retrofiting 178 F-15C fighters with the AN/APG-63(V)3 Active Electronically Scanned Array (AESA) radar with delivery beginning in early 2009.[24] Additionally, the Air Force also plans to upgrade other F-15s with the Joint Helmet Mounted Cueing System (JHMCS).[25]

## [edit] Design

F-15C performing a maximum performance takeoff.

The F-15 has an all-metal semi-monocoque fuselage with a large cantilever shoulder-mounted wing. The empennage is all-metal twin fins and rudders with all-moving composite horizontal tail surfaces outboard of the fins. The F-15 has a spine-mounted air brake and retractable tricycle landing gear. It is powered by two Pratt & Whitney F100 axial-flow turbofan engines with afterburners mounted side-by-side in the fuselage. The cockpit is mounted high in the forward fuselage with a one-piece windscreen and large canopy to increase visibility.

The F-15's maneuverability is derived from low wing loading (weight to wing area ratio) with a high thrust-to-weight ratio enabling the aircraft to turn tightly without losing airspeed. The F-15 can climb to 30,000 ft (10,000 m) in around 60 seconds. The thrust output of the dual engines is greater than the aircraft's weight, thus giving it the ability to accelerate in a vertical climb. The weapons and flight control systems are designed so that one person can safely and effectively perform air-to-air combat.[12] The A and C-models are single-seat variants that make up the bulk of F-15 production. B and D-models add a second seat behind the pilot for training. E-models use the second seat for a bombardier/navigator.

A multi-mission avionics system includes a head-up display (HUD), advanced radar, inertial guidance system (INS), flight instruments, ultra high frequency (UHF) communications, and Tactical Air Navigation (TACAN) and Instrument Landing System (ILS) receivers. It also has an internally mounted, tactical electronic-warfare system, identification, friend or foe (IFF) system, electronic countermeasures suite and a central digital computer.[26]

A wing over maneuver displays the clean lines and high-wing design of an F-15E from Elmendorf AFB, AK.

The heads-up display projects, through a combiner, all essential flight information gathered by the integrated avionics system. This display, visible in any light condition, provides the pilot information necessary to track and destroy an enemy aircraft without having to look down at cockpit instruments.[27]

The F-15's versatile APG-63/70 pulse-Doppler radar system can look up at high-flying targets and down at low-flying targets without being confused by ground clutter. It can detect and track aircraft and small high-speed targets at distances beyond visual range (the maximum being 120 nautical miles (220 km) away) down to close range, and at altitudes down to treetop level. The radar feeds target information into the central computer for effective weapons delivery. The capability of locking onto targets as far as 50 nautical miles (90 km) with an AIM-120 AMRAAM missile enables true beyond visual range (BVR) engagement of targets. For close-in dogfights, the radar automatically acquires enemy aircraft, and this information is projected on the head-up display. The F-15's electronic warfare system provides both threat warning and automatic countermeasures against selected threats.[28]

F-15E with speed brake deployed and conformal tanks fitted.

A variety of air-to-air weaponry can be carried by the F-15. An automated weapon system enables the pilot to perform aerial combat safely and effectively, using the head-up display and the avionics and weapons controls located on the engine throttles or control stick. When the pilot changes from one weapon system to another, visual guidance for the required weapon automatically appears on the head-up display.[29]

The Eagle can be armed with combinations of four different air-to-air weapons: AIM-7F/M Sparrow missiles or AIM-120 AMRAAM advanced medium range air-to-air missiles on its lower fuselage corners, AIM-9L/M Sidewinder or AIM-120 missiles on two pylons under the wings, and an internal M61A1 20 mm Gatling gun in the right wing root.[30]

Low-drag conformal fuel tanks (CFTs) were developed for the F-15C and D models. They can be attached to the sides of the engine air intake trunks under each wing and are designed to the same load factors and airspeed limits as the basic aircraft.[30] However, they degrade performance by increasing drag and cannot be jettisoned in-flight (unlike conventional external tanks). Each conformal fuel tank can hold 750 U.S. gallons (2,840 L) of fuel.[31] These tanks increase range thus reducing the need for in-flight refueling. All external stations for munitions remain available with the tanks in use. Moreover, Sparrow or AMRAAM missiles can be attached to the corners of the conformal fuel tanks.[19] The 57 FIS based at Keflavik NAS, Iceland was the only C-model squadron to utilize CFTs on a regular basis due to its extended operations over the North Atlantic. With the closure of the 57 FIS the F-15E is the only U.S. variant to carry them on a routine basis. The American CFTs were also provided to Israel and Saudi Arabia but only Israel uses them (as needed) on their entire fleet.

M61 Vulcan mounted on the side of right engine intake.

The F-15E Strike Eagle is a two-seat, dual-role, totally integrated fighter for all-weather, air-to-air and deep interdiction missions. The rear cockpit is upgraded to include four multi-purpose CRT displays for aircraft systems and weapons management. The digital, triple-redundant Lear Siegler flight control system permits coupled automatic terrain following, enhanced by a ring-laser gyro inertial navigation system.[32] For low-altitude, high-speed penetration and precision attack on tactical targets at night or in adverse weather, the F-15E carries a high-resolution APG-70 radar and LANTIRN pods to provide thermal imagery.[28]

The APG-63(V)2 Active Electronically Scanned Array (AESA) radar has been retrofitted to 18 U.S. Air Force F-15C aircraft.[33] This upgrade includes most of the new hardware from the APG-63(V)1, but adds an AESA to provide increased pilot situational awareness. The AESA radar has an exceptionally agile beam, providing nearly instantaneous track updates and enhanced multi-target tracking capability. The APG-63(V)2 is compatible with current F-15C weapon loads and enables pilots to take full advantage of AIM-120 AMRAAM capabilities, simultaneously guiding multiple missiles to several targets widely spaced in azimuth, elevation, or range.

## [edit] Operational history

The largest operator of the F-15 is the United States Air Force. The first Eagle (F-15B) was delivered November 14, 1974.[34] In January 1976, the first Eagle destined for a combat squadron, the 555th TFS, was delivered.[34] These initial aircraft carried the Hughes Aircraft (now Raytheon) APG-63 radar.

The first F-15 kill was scored by IAF ace Moshe Melnik in 1979.[35] In 1979–81, during Israeli raids against Palestinian factions based in Lebanon, F-15As downed 13 Syrian MiG-21 "Fishbeds" and two Syrian MiG-25 "Foxbats", the latter being the aircraft the F-15 was designed to kill. Israeli F-15As and Bs participated as escorts in Operation Opera and served during the 1982 Lebanon War. During the latter, Israeli F-15s shot down 40 Syrian jet fighters (23 MiG-21 "Fishbeds" and 17 MiG-23 "Floggers") and one Syrian SA.342L Gazelle helicopter.[36] Later on, in 1985, IAF Eagles, in Operation Wooden Leg, bombed the PLO headquarters in Tunisia.[37] This was one of the few times air superiority F-15s (A/B/C/D models) were used in tactical strike missions.

Royal Saudi Air Force F-15C pilots shot down two F-4E Phantom IIs flown by the Iranian Air Force in a skirmish in June 1984, and shot down two Iraqi Mirage F1s during the Gulf War.[38][39][40]