AGUSTA A109A II

Pilot Checklist

NOTE:

Due to the highly customised gauges included in this helicopter, you cannot use the default auto engine start sequence (Ctrl+E) in FS2004.

Included is a fully automatic start sequence, or a fully manual start sequence, at pilot’s discretion.

Helicopter defaults with AUTO start activated.

Prior to Use: Under the Aircraft, Realism Settings Menu, make sure that ENABLE AUTOMIXTURE is NOT checked.

BEFORE ENGINE START

Pre Flight Inspection 				Complete

Wheels								Chocked NO BRAKES

Doors									Closed

Harnesses							Adjusted and Secure

Seats and Pedals					Adjusted

Circuit Breaker Panel    			On as Required

Wander/Map Light					Stowed and OFF

All Overhead Switches				OFF

Anti Collision Light					OFF

Power Levers						Full and Free then to CUT OFF

Rotor Brake							Free and OFF( Toggle ON and OFF)

OAT									Note Temperature

Compass Deviation Card			Fitted and Checked

Radio Nav Aids						ALL Off

Engine and Electrical Inst.			Condition / Indication

Flight Instruments					Condition / Flags

Clock									Wound / Set

Stand By Compass x 2				Condition/Headings

Landing Gear 						Selected Down

Parking Brake						ON

Nose wheel Lock					ON

Compass System Switch			On MAG

Comms Panel / Intercom			As Required

Force Trim Switch					ON

Servo Switch 						ON BOTH

Cabin Extinguisher					Secure / Serviceable

Landing Lights 						OFF

Collective								Full DOWN / Friction ON

Cyclic									Friction OFF

MOBILE PHONES					ALL OFF

Pedals								Full / Free Movement- Centered

USE EITHER MANUAL OR AUTO ENGINE START SEQUENCES

### AUTO ENGINE START

##### MANUAL / AUTO START SELECT		AUTO

Battery Switch								ON

Outside Air Temperature					Check: If less than 32 F, Engine Heater ON for start

Auto Start Switch						ON

AT 70 % N2 (Approx)					Throttle back to maintain 60%N2

Generator 1& 2				DC GEN /Load Lights ON

Essential Bus CB Lights - ON x 3

DC Amps Below 75 Amps

Avionics										ON

RADIO / NAVAIDS / TCAS  ON  As Required

CB AMBER ADVISORY LIGHTS - ON AS REQUIRED

CB RED WARNING LIGHTS - ALL OFF

### MANUAL ENGINE START

Engines AUTO/MAN Switch		MAN (Switch down, green MAN start indicators showing on Overhead Panel)

FUEL CONTROL PANEL

Fuel Pumps									1 & 3/ 2 & 4/ -ON Check CB Lights - Pumps ON

Indicators HORIZONTAL

Fuel Pressure Warning 	CB Lights - OFF

Fuel Valves									ON / Indicator VERTICAL/ Check CB Lights

Fuel Cross Feed							Open / Indicator HORIZONTAL / Check CB Light

Fuel Quantity								Check Quantity + Warning CB x 2 –OFF

Hydraulic Oil Pumps 						Both ON

# MAIN ELECTRICAL PANEL

Battery Switch 								ON (Bat Switch OFF if AUX Pwr is used)

Voltage										CHECK > 24 Volts  ( If LOWER  investigate )

#### Anti Collision Light						ON

Cockpit Lighting							As Required

Inverters 1 - 2 - 3							Turn ON Sequentially

Warning/Caution/Advisory Lights		CHECK 3 Positions inc Wheels Up Audio

Engine Temps and Pressures			ZERO / Ambient

Landing Gear Lights						3 GREENS / Switch To Test - RED

ROTOR RPM Beeper						DECREASE for 10 Secs

Rotor Disc									CLEAR

Battery Voltage								Check  > 24 V

Outside Air Temperature					If less than 32 F, Engine Pre-Heater ON for start

Starter Button								PRESS (CHECK Starter Relays and Ignition														System CB Lights ON)

Throttle (ENSURE TOT >200 C)			MOVE to FULL @ >13% N2 (11 %  Below 7 C OAT)

OBSERVE									MAX TOT 927C - Momentary Peak - 1 Second														ABOVE 810 C - MAX 10 SECONDS

BLADES turning by 40 % N2

Engine / Transmission Pressures Rising														Hydraulic Pressure SERVOS Rising

AT 70 % N2 (Approx)					Throttle back to maintain 60%N2

Starter Relay CB Lights -OFF

Ignition System CB Lights- ON

Engine Pre-Heater OFF

Generators 1 + 2							ON Sequentially - DC GEN Caution Lights ON

Essential Bus CB Lights - ON x 3

DC Amps Below 75 Amps																RADIO / NAVAIDS / TCAS  ON  As Required

Avionics  									ON

CB AMBER ADVISORY LIGHTS - ON AS REQUIRED

CB RED WARNING LIGHTS - ALL OFF

<<<<<<<<<<<<PROCEED TO RUN UP CHECKS >>>>>>>>>>>>>

RUN  UP

De Ice Eng No. 1						ON  - TOT Rise Advisory Light ON/ CB Light ON 													Leave ON If Ambient Temperature Requires													(Operations in + 4 C or Lower and Visible Moisture)

De Ice Eng No. 2						Repeat As No.1

Pitot Heat 1 + 2							ON Individually -

Leave ON if Required as Per Eng De Ice

Flight Instruments						ADI - ERECT / TESTED

CROSS CHECK :

R M I - H S I - STBY COMPASS

CHECK / SET ALTIMETERS

RAD ALT - Check and Set

SAS / AUTOPILOT CHECKS

#### CYCLIC									Re-Center

PRE TAKE OFF CHECKS

Circuit Breakers 							ALL IN / Lit as Required

Anti Collision Light						ON

Position Lights							AS REQUIRED

Anti Ice / Pitot Heaters					AS REQUIRED  (+ 4 C or Less OAT)

RRPM CIRCUIT BREAKER			Light Out

Power Levers							FULLY FORWARD GENTLY

Beep to 100% RRPM / Match Torques

Ts and Ps								Green / Within Limits

Altimeters 								SET As Required

Parking Brake							OFF - Toe Brake ON

Warning Panel 		(CWP)				Clear

Collective Friction						OFF

Nose Wheel Lock						ON - Centered As Required

TIME										NOTE

HOVER CHECKS

NR											100%

TORQUES								MATCHED

Ts and Ps								GREEN

Warning Panel		(CWP)				CLEAR

AFTER TAKE OFF CHECKS

Landing Gear							UP

Anti Icing									As Required

Altimeters									Set

Landing Lights							OFF

EN ROUTE  CLIMB AND DESCENT CHECKS

Anti Icing								As Required

Fuel									Sufficient - Management Left/Right

Ts and Ps							Check - All Green

Altimeters								As Required

Radio/Nav Aids						As Required

PRE LANDING CHECKS

LANDING GEAR					DOWN - Below 120 KIAS

Landing Gear Indicator/Lever		3 Greens / Down (INDICATED)

BRAKES								As Required - (ON in Confined Area)

NOSE WHEEL LOCK				ON

Warning Panel		(CWP)			CLEAR

FUEL	Sufficient						As Expected

Ts and Ps							Green

ANTI ICING							As Required - (ON in +4 C or Less)

GO AROUND

Landing Gear						UP

Altimeters								As Required

Anti Icing								As Required

AFTER LANDING CHECKS

Parking Brake						ON

Nose wheel Lock					ON

Control Frictions						ON

Power Levers						TO IDLE for 2 MINUTES

Landing Light						OFF

Clock									START for 2 minutes

SHUT DOWN CHECKS

Radio/Nav Aids/TCAS				OFF

ALL SAS								OFF

Gen Bus 1 + 2 						OFF

Anti Ice/ Pitots						OFF

Power Levers 						OFF - Fully Aft

Cross Feed Valve					CLOSED - Indicator Vertical

Fuel Pumps 1 + 2					OFF - Indicators Vertical

Fuel Pumps 3 + 4					OFF - Indicators Vertical

Fuel Valves x 2						OFF- Indicators Horizontal

Rotor Brake							APPLY Below 200 RRPM

Inverters 1, 2, 3						OFF (If Aircraft is to be moved immediately

leave on to protect VGs)

TOT									Monitor, Motor the Engines if TOT

Rises above 400 C

Fuel Quantity							NOTE Gals remaining

All Lights								OFF

Avionics								OFF

Battery								OFF