# PREFLIGHT

CONTROLS 	 Check free movement

UNDERCARRIAGE 	 Brake fluid and tire PSI

(main 22 & tail 35)

PROP 	 Turn prop 10 times

(open 3 oil drain valves on bottom cylinders, then close to vertical position)

OIL 	 Check level 12-14 liters & cap secure

open TOP engine shutter

oil cooler no obstruction

FUEL 	 2 drains

(check level, vents clear, caps secure)

ALL 	 Covers and locks removed

aft battery switch on

start air pressure above 25 on gauge

# PRESTART

Prop  	Forward

Mix - 	Rich

Oil & Engine shutters  	Open

Fuel gauges 	 Check

Fuel valve  	L or R

Primer  	Unlock (prime 3-4 shots)

Throttle  	Open 1/2 inch (1000 RPM)

Switches Bottom 6  	On (Bat, Gen, Starter, Instruments, Beacon)

Air Valve  	Open

Start  	Push

Mags  	On (1+2) after prop turning 4 times

Switches Top 5  	On

Oil 	 .14 mPa (20 seconds after start)

Primer  	Locked in

# RUNUP

1300 RPM 	 until CHT 100*C & OIL 30*C

1600 RPM 	 check GENERATOR (28 volts)

1860 RPM 	 check MAGS (drop 60 RPM) & Carb Heat

2000 RPM 	 exercise PROP (1400 RPM - 3 times)

Prop 	 reduce RPM to 1860

(move Throttle - RPM constant)

Throttle 	 680 mm Hg

(Prop - Forward)

check instruments green

Prop 	 reduce RPM to 2050

(check instruments green)

Throttle 	 Idle (Prop - Forward)

Throttle 	 Full open and close within 6 seconds

# PRETAKEOFF

RECIT.CIGAR.WUFTDM (radios, electrics, controls, inside, trim. cowls, instruments, gas, altimeter, runup. wind, undercarriage, flaps, time, dg, mixture)

# ENGINE TAKE OFF

Throttle 	 Full Open

Prop 	 2350 RPM (5 min)

Prop 	 2050 RPM (at 500 feet)

# ENGINE CRUISE

Full Throttle & 2050 RPM 	(max continuous)

680 mm Hg & 1860 RPM 	(75% power)

630 mm Hg & 1730 RPM 	(60% power)

Engine shutters 	 140*C-210*C (230*C max)

Oil shutters 	 50*C-65*C (75*C max)

Mixture 	 adjust (monitor CHT)

# SHUTDOWN

Avionics 	 Off

700 RPM 	 2 minutes

2000 RPM 	 20 seconds

700 RPM

Mags 	 Off

Throttle 	 Full open (closed after prop stops)

Switches 	 Off

Fuel valve 	 Closed

Air valve 	 Closed

Pneumatic dehydrator 	 Drain (bottom right of engine)

# INFORMATION

Takeoff distance (21* flap) 	 200 meters

Landing distance (44* flap, 3 point landing) 	 200 meters

Gross weight 	 2866 pounds

Empty weight 	 2156 pounds

Baggage area 	 66 pounds max

Usable fuel 	 22+22 US Gal (170 Liters) (min 91 Octane)

Fuel flow 	 14-22 GPH (50-85 LPH) // average is 60 LPH.

Oil capacity 	 23 liters // average consumption 1 LPH

Engine type 	 AI-14RA (251 HP)

Aircraft serial no -	CF21940944

Length - 27' // Span - 37' // Height - 10'

Dive 	 speed 110 kts (MP not under 330 mm Hg)

EMERGENCY 	 fuel pump on

check fuel PSI, switch tanks

V SPEEDS - WILGA

Vr (21*) 	 48 kts

Vx (21*) 	 51 kts

Vx (44*) 	 49 kts

Vy (0*) 	 62 kts

Va 	 93 kts

Vfe 	 80 kts

Vref (44*) 	 67 kts

Vs (0*) 	 57 kts

Vso (44*) 	 51 kts

Vparachute 	 60-80 kts