Documentation for the True ATC Adventure

LH 390 FRA- STR

Frankfurt - Stuttgart (EDDF - EDDS)

by Steffen Gerlach

Version: Feb. 1998

Contents

1.	Introduction

2.	General information

3.	Installation

4.	Sound files

5.	Scenery

6.	Aircraft / panel

7.	Routing/Briefing

8.	Required charts

9.	Tips for a smooth flight

10.	GPWS

11.	Background ATC chatter

12.	Features not implemented

13.	Speakers wanted

14.	Credits / the voices

15. 	Version history

16.	Guarantee/legal notice

17.	Key summary

1. Introduction

This is positively the final version of LH390. In the meantime even Lufthansa has removed it from their timetable...<g> - and it has been rechristened LH044. All I can really say in my defence is that it is far easier to modify a familiar and well-loved flight than it is to start a new one from scratch.

During the planning of my most recent adventure (FRA-NCE / LH4384) I used LH390 as a test bed and did not want to deny you this improved version by merely leaving it gather dust in a dusty corner of my hard disk...<g>. As part of the overall learning process I have put a few new ideas into action, implemented the modified GPWS and incorporated some users' suggestions.

Due to its brevity, LH390 is still my favourite flight, but I have vowed to develop a new short flight next time round...<g>

2. General information

This is a regular flight between Frankfurt and Stuttgart taken from the Lufthansa's Winter Timetable (27th October 1996 - 29th March 1997) which was still valid when I started work on this project<g>. The real-world flight, (dep. 8:55 a.m., arr. 9:40 a.m.) is served by an Airbus A321. In Lufthansa's Summer Timetable (1st July - 25th October 1997) the departure time was put back to 9:15 a.m., so that the flight then arrived in Stuttgart at 10:00 a.m. Like my first ATC Adventure (Frankfurt to Munich), this project is intended to provide a welcome change to the rather boring standard BAO Flightshop-generated ATC adventures. It is not just that it is tedious always having to listen to the same old background chatter and robotic messages which sound as though they have been assembled from the cutting room floor of Star Wars, what we actually hear has little or nothing to do with the way IFR approaches are conducted under the guidance of air traffic controllers at major international airports in real life. The type of approach encountered in Flightshop, where the controller holds you by the hand and guides you round the circuit, is only practised at small airports under VFR (Visual Flying Regulations) conditions. Instead, routes to major airports are prescribed and have been standardized by the local aviation authorities (in Germany the DFS - Deutsche Flugsicherung). This information is published at regular intervals, is part of an airline pilot's standard equipment and forms the basis of real-world flight plans.

Despite all the excellent add-ons that we have for Flightshop adventures today (copilot, GPWS, weather, more background ATC chatter etc.) one key feature is still missing - an intelligent Automatic Terminal Information Service (ATIS), i.e. real weather reports. It is not just the form of radio contact used in Flightshop that is incorrect, but even the language used - at least within Germany. Despite all the hard work that has gone into producing traditional Flightshop adventures, they unfortunately have little to do with reality. As a keen Microsoft flight-simmer I wanted to try and create an adventure which is as close to a real flight in an airliner as I could possible get within the limitations of Flightshop.

In this adventure - as in real life - ATC will guide us to our destination via SIDs and STARs (Standard Instrument Departure routes, STandard Arrival Routes). Furthermore, every time you switch to a new frequency or control centre, you will be routed by a different real-life air traffic controller rather than by synthetic voice messages. Over 50 individual ATC messages have been specially recorded for this adventure for this purpose.

Just to make sure that this adventure is as realistic as possible, I've also had both the route and radio messages checked out by a real air traffic controller! With the exception of a few limitations, what you hear is very close to what could happen in reality.

That is why I have incorporated Version 7.2 of Wilco van Deijl's GPWS module (revised and modified by Stephen Coleman) into this update. Wilco's Ground Proximity Warning System module introduces functions you would expect to find on a real jet airliner, e.g. a flight management system (with a Waypoint Navigation facility), a Ground Proximity Warning System, speed callouts on take-off and in the air, altitude callouts by the aircraft's synthetic voice on approach - and even an autoland facility. The GPWS also permits you to enter the autopilot heading and altitude and COM/NAV frequencies directly using the keyboard, which makes your jet a whole lot easier to handle in the air.

I have, however, had to compromise on a few points. It has proved necessary to alter the radio frequencies for Frankfurt airport by 0.05 MHz so that when you tune into the Ground or Tower frequencies you hear a real ATIS report as broadcast by the Rhine-Main Airport rather than being bombarded by the standard Flightshop ATIS messages which have nothing to do with this adventure. Still, I think this is something we can live with.

3. Installation

Copy the files as follows:

LH390.ADV		\Flight Simulator\adv

LH390.STN		\Flight Simulator\pilots

*.WAV		\Flight Simulator\adv\wav

I have also included some background material for you to study:

FRA-STR.BMP		A small map of the route taken by LH390 in bitmap format

LH390.FPF		A flight plan in Flightplanner 4.0b format

LH390.TXT		A flight plan in text format, generated with Flightplanner 4.0b

LH390.FPL		A flight plan in Navigator 6.0 format

LH390.DOC		This file

4. Sound files

In the past I have always used 11 kHz-8bit mono wav-files in Windows-PCM format for my adventures. For the first time this full version uses the original FS98 format - Microsoft 4-bit ADACPM compressed files - with the exception of the cockpit announcements where better sound reproduction is important. As well as possibly sounding more realistic, this method has the added bonus that it brings with it a space saving of 48%. This of course means reduced download costs... <g> and indeed reduced upload costs for me.<g>

5. Scenery

This adventure requires the BAO Europe 1 Scenery. The radio navigation beacons are included in FS98, but Frankfurt and Stuttgart airports are missing.

I would also recommend using Skylink's Frankfurt airport scenery to enhance the adventure still further, it really is very good. Unfortunately it is shareware, but is currently the best scenery available for Frankfurt airport (also as regards frame rate) and well worth the small investment. It is available through CompuServe "Go SWREG" or directly from SkyLink, Michael Ackermann, e-mail: skylink2@compuserve.com. Naturally the adventure can also be run using just Europe 1 alone, but you will then have no yellow lines to guide you across the acres of concrete at Frankfurt, nor any taxiway or runway signs, let alone a docking gate. A freeware scenery  EDDFTAXI.ZIP is also available for Frankfurt and it goes a long way to improving the adventure. It doesn't have Gate B20, but for that Gate A23.

I would also recommend using the improved textures "E1E2TEX.ZIP" for Europe1 which are available from Compuserve's FSForum or various Internet sites. You should have also installed the official BAO/Microsoft patch and "FSConverter98" which cure some problems with third party sceneries and other snags. These files can be downloaded free of charge from http://www.microwings.com, from Microsoft's FS Internet site or FSFORUM or SIMGAMES in Compuserve or form various other mailboxes or Internet sites.

The icing on the cake, however, is a third party scenery for Stuttgart. I always used to recommend Peter Hiermeier's very fine freeware add-on scenery EDDS1296.ZIP which shows a very realistic Stuttgart airport using specially designed buildings rather than the standard BAO real estate. In the meantime this has been superseded by 'German Airports 1' by Thomas Hirsch and Peter Hiermeier. They have even managed to improve upon their already fine sceneries and still achieve an excellent frame rate - goodness knows how. Their CD is published by Aerosoft and can be obtained from http://www.aerosoft.com. This is a definite find for those of you who like flying in Germany. A further recommendation is Peter Fuß's static scenery EDDSSTAT.ZIP which places four jets in Lufthansa and Air France colours on the apron at Stuttgart. Thomas Hirsch and Peter Hiermeier offer an alternative static scenery on their homepages - both these sceneries are freeware.

If you are using a scenery other than Skylink's Frankfurt, your aircraft will not be correctly positioned at the start of the adventure because the different scenery versions are not perfectly aligned with one another. In this instance, you can use the slew option to move your aircraft to the correct position and store this new situation as LH390.STN. It is important you use the correct name because otherwise the adventure will still start with the original default situation in future. NB.: be careful when saving your new version as it is possible to lose the prescribed weather settings. It is also possible to alter your choice of aircraft, panel, time of day, season or weather etc. by the same method. However, if you do alter the weather, please be aware that the ATIS reports provided at Frankfurt will not change accordingly.

To be able to get maximum enjoyment out of the GPWS, you will need the file "GPWS_V72.ZIP" from the Internet and BAO Flightshop. Both include WAV files which are called up by this adventure but which I am not permitted to include in this package for copyright reasons.

6. Aircraft / Panel

You really ought to use one of the airbus A321s in Lufthansa livery available from the Internet or FSForum for this flight. After all, if a job's worth doing....

With regard to installing the aircraft: the FS98 Converter only converts MDL files in FS5 format and not in FS6 format. The most practical solution I have found is to unzip your downloaded aircraft zip-files into the FS98 "Old Aircraft" folder and to store C:\Programs\Microsoft Games\Flight simulator\Old Aircraft\ [or whatever] as the default path under "Options" and "Aircraft Converter Options" in your FS98 converter. Then every time you start the converter and click on "Aircraft" the one you are looking for always pops up immediately - here "Lha_a321". The aircraft is installed within seconds by double-clicking the air-file...<g>

The adventure automatically loads my favourite Lufthansa A321 (lha_a321.zip by Project Freeware), available from the Project Freeware Internet site which can be reached via a hyperlink from http://www.flightsim.com. If you do not have this aircraft installed, the adventure announces an error and loads the standard FS98 B737 instead. By the way, if you wish to respray your default B737 in Lufthansa livery, you can download the file "LH-FS98.ZIP" from the Internet which contains everything you need to transform your aircraft.

Naturally you can also call up the situation file LH390.STN outside the adventure and select a different plane instead. If you do this, it is important you remember to store the situation file under the same name.

I use Eric W. Ernst's glass cockpit (G24eng98v2.zip) panel. Eric's panels are simply the best! A big thank you to him...<g>. His full range of panels, all of which can be recommended, are available from http://www.flightsim.com/efpanels.

If you do not use this same glass panel, the panel you use for the A321 may appear incorrectly sized when the adventure loads the situation. The only way round this problem is to call up LH390.STN and to call up your preferred aircraft and panel (e.g. the normal FS98 B737) and to store the situation under the name LH390.STN again with your new configuration. When you restart your adventure, your new panel should be displayed correctly.

7. Routing/Briefing

A. Flight Plan

Frankfurt Rhein-Main Airport Gate B20 		364 feet	N 50 02.9	E 008 34.2

/Terminal1 --> Taxiway N

Ried VOR(RID  112.20	178°	6-7000 feet	N 49 47.0	E 008 32.6

Neckar NDB  (NKR  292)	166°	13,000 feet	N 49 20.2	E 008 44.0

Luburg VOR (LBU  109.20)	138°	5000 feet	N 48 54.9	E 009 20.5

ILS Glidepath Rwy 25 intercepting	130°  	4500 feet	N 48 48.0	E 009 33.3

Stuttgart Airport Gate 11 at Terminal 1		1267 feet	N 48 41.2	E 009 11.3

B. Description

Ramp:

We are standing at Gate B20 at Frankfurt airport, listening to the current ATIS weather report before carrying out our pre-start checks and obtaining permission to start up our engines (Frankfurt Ground Control on 121.95 MHz). The copilot has been very busy and already pre-programmed the on-board computer for our flight.

In Frankfurt we will be taking off from the 'Startbahn 18 West'. During its construction this was the site of heated battles with environmentalists. Runway 18 is generally used for flights heading South or East because of the prevailing westerly wind in Frankfurt. We travel to the runway via taxiway November and holding point N. If you have one of the above third-party sceneries for Frankfurt, you just need to follow the yellow taxiway lines. Please do not exceed 5-8 knots in the vicinity of the gates, do not take tight 90° bends at more than 8-10 knots and keep your taxiing speed down to 15-25 knots on the straight - this is not Hockenheim. Furthermore, if you keep your taxiing speed down to 20 kts the adventure will run smoothly. If you travel too fast, you may have problems with the way some of the sound files are played back.

The radio sequence for start-up clearance, push-back and taxi has been simplified somewhat for this adventure. In reality, after having obtained start-up permission from Ground Control, you would also have to contact the Apron Controller from the airport authority (Frankfurt Flughafen AG) which is responsible for taxiway N.

Ground Control clears you to follow the SID 'Neckar four Lima' (NKR 4L) which is normally used when flying south from Frankfurt.

Taxiing:

Now that we have received clearance for starting the engines we can start the pushback procedure. This now occurs automatically without your having to lift a finger. All the pilot has to do is follow the instructions of the Ramp Agent regarding the brakes. The aircraft is pushed back by the tug until it is standing correctly aligned on the taxiway. Normally we would start the engines during pushback, but since this is not possible during the FS98 video-sequence, we have to wait until afterwards. The only FS98 option available to you whilst the video is being played is the escape key to break off the routine, but if you do that, you will have to push back manually...<g>.

Once we have received clearance to taxi we can start to roll, after which our route is also cleared. Once we are safely on Taxiway N and the cockpit signal (gong) has sounded we can trigger the captain's first passenger announcement by pressing the speech button Ctrl+K. When we reach the S-bend on taxiway N, our copilot tunes into the Tower frequency and we press Ctrl+K again to announce our presence to Tower Control (frequency 124.80 for Rwy 18 - the parallel runways in Frankfurt have a separate frequency).

Take-off:

We have informed the Tower that we will soon be ready for take off and after a short departure briefing we are ready to go. Tower gives us clearance for a rolling takeoff. For a particularly elegant take-off you can always use the new FS98 autothrust and takeoff power facility.<g>. Once you have rotated and achieved a positive rate of climb, retract the gear and we are away.

After the copilot has entered the transponder/squawk code (6612 in this instance) and tuned into the Frankfurt Departure Control frequency (DF Radar: 120.15 MHz) Radar Control is contacted automatically. It is necessary to press Ctrl+K for all further radio communications initiated by the pilot. Start to raise the flaps and gradually reduce thrust to remain at an IAS of 250 knots below an altitude of 10 000 ft. Your rate of climb should be 2500 ft/min. Once you have achieved a positive rate of climb you can engage the autopilot (main switch) and, if you wish you can set the GPWS waypoint navigation facility by pressing '[comma]', 'a', '[comma]'. The first programmed waypoint is RID VOR. In the real world, the autopilot is often not engaged until an interim cruising height has been reached. In this adventure the autopilot can fly the aircraft virtually all on its own, even from just after take-off if so desired. Once we reach 10,000 ft we can put our foot down until we have attained our cruising speed of 380 kts... <g>

Climb to 4000 ft immediately after take off as prescribed by the SID. Frankfurt Radar will then provide further instructions, especially regarding altitude, whereby the controllers will, as far as possible, allow the aircraft to climb at a constant rate rather than forcing us to level out at an intermediate altitude.

After following the Standard Instrument Departure route (SID) 'Neckar 4L' we turn direct to Ried VOR (RID, 112.20) on a heading of 178°. From there we head towards NDB Neckar (NKR, 292) on the outbound radial 166°. From Neckar NDB we turn towards Luburg VOR (LBU 109.20) on a heading of 138°. ATC will provide us with clearance for the relevant flight level and routes as we fly. When we cross the threshold height - in this instance 6000 ft, due to the prevailing weather conditions - we must reset the altimeter to a standard pressure of 1014 millibars to obtain a correct altitude reading. This is the standard pressure used above 6000 ft and is prescribed for all aircraft. ATIS provides information on the given threshold height which depends on the air pressure on a particular day. If we did not adjust the altimeter, we would be at the correct height for our pre-set pressure of 1024 millibars, but physically would be at a different height to any other aircraft in the area with correctly adjusted altimeters, which could result in a collision...

On our flight we will encounter some rather heavy 'clear air turbulence' at about 12,000 ft, at which point we will request a new and less bumpy flight level than the 13,000 ft allocated by ATC.

Once we have turned towards Luburg after leaving Neckar behind us and the copilot has adjusted the settings accordingly, press Ctrl+K to play back the Captain's second passenger announcement.

Descent:

Before we start our descent the copilot will tune into Stuttgart ATIS, to permit us to hear the 'endless' local weather report tape. This report is updated at regular intervals or if there is any sudden change in the weather. We are obliged to listen to this information and must confirm with ATC that we have indeed heard it. The main thing is that we have heard the correct QNH so that our altimeter is set correctly.

We are then instructed to descend and are passed on to Stuttgart Approach Controller (COM 125.05 - don't forget to press Ctrl+K). With foresight, we have of course already started to reduce our cruising speed - this ought to be done just after the Captain's second passenger announcement because we have quite a bit of speed to dissipate if we are to make a clean descent - i.e. without flaps - at a reasonable speed. We start our descent 17 DME inbound from Luburg (i.e. 17 nautical miles before the Luburg navigation aid). We can set the autopilot to 250 kts with a rate of descent 2500 ft/min.

Due to the prevailing westerly wind, we can expect to be cleared for Runway 25, i.e. that we will approach the airport from the east on a heading of 255°. We are cleared for an ILS approach to Runway 25 using the STandard Approach Routine (STAR) contained in the appropriate manuals. For this purpose, we must arrive at Luburg at a height of 5000 ft, i.e. once we have been cleared for the ILS approach we can descend at our own discretion unless ATC instructs us otherwise. When we reach Luburg we supply ATC with our ATO (actual time over [Luburg]) as requested. This is done automatically by our copilot. The adventure recognizes and displays our actual ATO (see cockpit clock) in text on screen every time you fly the adventure. The WAV file, unfortunately, is not updated and gives a 'good average' which I achieved during practice. At LBU we change course to 130° outbound (i.e. fly on a heading of 130° away from LBU) and continue to descend to 4500 ft. We then turn right towards the ILS path and intercept it from below (NB.: never intercept an ILS path from above). It is possible that after tuning NAV1 to the ILS beacon frequency (STW 109.09) we have no display initially. However, do not worry, we will pick up the station when we intercept the ILS glide slope at the latest. Do not rely on the autopilot when lining up on the glidepath but fly the curve a bit tighter manually. The GPWS flies us to the point beyond Luburg where we bank to starboard to line up with the ILS beacon - from here on we are on our own <g>. If you are worried about turning too early, press Ctrl+I to check your relative position as otherwise the adventure will cut out - you cannot take a short cut and miss this step out, unfortunately <g>.

Once we are established on the glide path (with both Nav1 needles centred) contact Stuttgart Approach (press Ctrl+K). We are then requested to get in touch again when overflying the outer marker (abbrev. OM and combined with NDB radio beacon - SG 306). In the meantime we have enough to do intercepting the glide path. Once established on the glidepath we can activate the autopilot to lock onto it and turn our attention to the flaps, regulating the thrust and lowering the gear. Then we have to contact the Tower Controller (118.80) in accordance with ATC instructions (Press Ctrl+K). We are now starting on the final phase of our journey. Reduce speed to 160 kts if you have not already done so, set flaps and lower your gear. After obtaining clearance to land, touchdown, apply air brakes, reverse thrust, then apply the brakes at 70 knots, close air brakes and shut down reverse thrust.

You can leave runway 25 via exit F. Once we are clear of the runway we must inform the Tower Controller (press Ctrl+K), who instructs us to contact Stuttgart Ground (121.90) (press Ctrl+K). Tower Control will give us clearance to taxi to the ramp and parking position 11 at Terminal 1. On our way to the ramp we press Ctrl+K so that the captain can say good-bye to the passengers.

Gate 11 is the second space on the left between an LH Airbus on the left and an Air France aircraft on the right. The adventure scans whether you really have reached your parking position and ends once the aircraft has stopped or virtually stopped. Again, in reality there would be an Apron Controller here too.

Please do not forget that there is a maximum speed limit of 250 knots below 10 000 ft for the benefit of slower moving VFR traffic.

Should you fail to receive an ATC message at any point, press Ctrl+I (see below), to discover the distance and heading you should take to reach your next waypoint. You have to fly relatively close to the waypoints for the adventure to move on to the next stage - otherwise life would be far too simple...

Should you miss a waypoint, there is a little trick you can use to get back on course - switch to the slew mode and 'reverse'(!) your aircraft until it is inbound of the last navigation beacon and, using Ctrl+I make sure that you fly over the beacon this time round, then the adventure will pick up where it left off...<g> [probably better than my 'holding pattern method', i.e. circling round until I pick it up again]

The GPWS flies so accurately that you can rely on it entirely, but despite this, do not forget that, as captain, it is YOUR responsibility to monitor all systems at all times, even when on autopilot.

One last comment regarding real-life flying. As I explained above, I have altered the real frequencies used by Frankfurt Tower and Frankfurt Ground by 0.05 MHz to avoid the irritating standard Flightshop ATIS messages. I feel this solution is by far the lesser of two evils. The frequencies for Frankfurt Radar and Stuttgart are, however, taken from real life.

The main thing is enjoy yourselves, have a pleasant flight and a smooth landing.

8. Required charts

The BAO Europe 1 Handbook and the accompanying copy of Jeppesen low altitude en-route chart E(LO)6.

The map of Frankfurt airport on p. 51, SID Frankfurt on p. 57, Stuttgart ILS approach, p. 99 and the map of Stuttgart airport on p. 97 (unfortunately now obsolete).

Unfortunately all these maps are now obsolete (printed 1995) and this flight has been prepared using up-to-date maps.

9. Tips for a successful flight

Key combinations available in-flight:

- CTRL+K makes radio contact, i.e. broadcasts the pilot's/copilot's message. If your Com receiver is set to the correct frequency, ATC will answer. If it is tuned to the wrong frequency they will not answer because they will not hear you. In this adventure you need to press Ctrl+K each time the pilot contacts a new station. The other messages are played back automatically.

- Ctrl+X activates or deactivates the copilot whilst the Adventure is running

- CTRL+R repeats the last radio or ATIS message.

- CTRL+I shows the heading and distance to the next waypoint, thus allowing you to correct your course independently of your instruments. The adventure establishes whether you have reached the waypoint with a tolerance of 0.5-3.5 nautical miles, depending on whether you are on the ground, cruising or on final approach. You will not be given clearance for the next stage of your flight until you have reached the respective waypoint, so make sure you are familiar with the route before setting out and keep all the necessary documentation to hand. Should you lose your way, you can find your position relative to the next waypoint by pressing Ctrl + I. This adventure does not recognize if you fly the incorrect route or take off from the incorrect runway. In such cases you will lose radio contact with Radar Control and receive no further instructions.

- GPWS waypoint navigation: the integrated GPWS enables a virtually fully automated flight in the Waypoint Navigation Mode. However, if you edit way points or use GPWS functions which generate long screen messages, you may overfly a waypoint without obtaining clearance for the next one. This is because only GPWS code is processed during GPWS operations. If you wish to fly in the waypoint navigation mode, please consult the section on the GPWS below.

- Help from co-pilot : At the beginning of the adventure you are asked if your 'copilot' should set the radios and navigation instruments (COM, NAV 1+2, ADF, transponder). I would recommend answering (y)es if it's your first attempt at flying this adventure because it means you can concentrate better on other things.

You are also asked if you wish to have the ATC messages displayed as text to simplify understanding or whether you would prefer to do without and just listen to the radio messages as you would in real life.

The 'flowing text' option under 'Settings' in Flight Simulator must be activated because otherwise the ATIS messages will not be displayed correctly.

10. GPWS

The programmed waypoints:

1. RID VOR (Ried)

2. NKR NDB (Neckar)

3. LBU VOR (Luburg)

4. Turning point for localizer

5. Intercepting point for ILS glidepath - Rwy 25

Invalid keys

Since the Adventure compiler only supports 20 ONKEYs (keys which can be used during the adventure), I had to 'borrow' a few keys from the GPWS module. The keys I've borrowed are:

R	for radio altitude

K	for activating/de-activating FO callouts during the flight

D	for decision height (fixed at 100 ft.)

G	for GPWS mode (all functions are active the whole time)

Ctrl+U	for ending the adventure. You can always end the adventure with "Reset Flight"

[Spacebar]	for displaying the waypoint

Modified keys

Because of problems with funny characters (umlauts etc.) the keys used to enter the autopilot altitude and heading settings have been modified:

Shift+A	Adjust autopilot altitude with 0...9·

Shift+H	Adjust heading with 0...9·

Pre-defined waypoints

The waypoints for this flight have already been entered into the GPWS beforehand so that you don't have to key them all in yourself. If you want the GPWS to fly your plane you should press the following keys - please consult the flight documentation for the point in time at which they should be activated.

[Comma]	Waypoint mode

a	Activate waypoint 1

[Comma]	Leave waypoint mode, wpt navigation ON

z	Autopilot ON (do not engage until after take-off!)

2 x [Comma]	Leave waypoint mode, wpt navigation OFF

d	Direct to the next waypoint, i.e. press [comma] d [comma]. You can go to a specific waypoint by pressing the relevant number (e.g. 5) beforehand

Warning

If you so wish, you can fly using the GPWS autopilot. There should be no conflicts with the adventure. However, you may receive no ATC messages as long as you are in the waypoint mode (i.e. after pressing [comma] once). In addition, GPWS functions which generate long screen messages - such as the spacebar, for example - may result in your flying over a waypoint without obtaining clearance for the next stage of your journey, so as a precaution I have disabled the space bar. It is advisable to use the GPWS functions sparingly when in the air. At the same time, you may also miss a waypoint because the adventure may be busy calculating distances and headings and broadcasting messages. Furthermore, the GPWS waypoints do not coincide exactly with the radio beacons and intersections. They are slightly premature because GPWS also takes into account the turn performance of your aircraft.

A list of the keys used for this adventure, together with the modified GPWS keys, can be found at the end of this document.

For instructions on general usage of GPWS, I refer you to the detailed description accompanying the programme provided by the author Wilco van Deijl. It gives a full explanation including the normal key assignment. Naturally you will need to download the file GPWS_V72.ZIP from the Internet or FSForum if you wish to enjoy all the various GPWS cockpit sounds as well. The WAV files need to be installed in your ADV\WAV sub-folder.

Since the GPWS regularly adjusts the trim and insists on playing the appropriate "trim1" or "trim2" WAV on each occasion, it is possible that another WAV which FS98 has been itching to play and has had to keep in a sort of holding pattern whilst the trim1 WAV has been playing comes out distorted or in an extremely slowed down form. That can either sound like an earthquake - or in some cases, if the message has been speeded up, like Donald Duck...<g>. This can also happen if you are climbing out of Frankfurt faster than at 2500 ft/min, if you accelerate the simulation by pressing "R", or should you break the 250 kts speed limit.

I have taken a lot of time and trouble trying to ensure that everything goes smoothly, but unfortunately we are treading a very fine line with such complex adventures. The solution is: "if at first you don't succeed, try, try again!" <g>

If things do get too frustrating, though, one alternative is to delete the "trim1" and "trim2" WAV files in the FS98 \ADV\WAV sub-folder...<g>. Or you can fly without the Waypoint Navigation mode - that at least cures this particular problem.

Sometimes this adventure puts your processor under a lot of strain because a lot of things happen in a very short time. This can sometimes result in a bit of a WAV air-traffic jam and can result in a WAV which is played immediately after another one being played back at the wrong speed. Normally it is then played back v-e-r-y slowly and sounds more like an earthquake than the human voice.

I have tried to eradicate this problem as far as possible and everything runs smoothly on my PC which, I must admit, does have a relatively fast CPU (AMD K6/200). Should you experience this sort of problem, one way of getting round it is to select a slower rate of climb (especially when outbound RID). I recommend a climb rate of 2500 ft/min, but choosing a slower rate of climb can help avoid queued WAVS because they are then better spaced and played back over a longer period since the rate of playback depends on your rate of climb.

The Captain's announcements to the passengers are optional. If you press Ctrl+K at the relevant time they will be played back, but otherwise the adventure will merely skip them and move on to the next stage - for those of you who use high-speed taxiing procedures...<g>. Your speed on the ground at Frankfurt should not exceed a speed of 10-15 kts - firstly because  this gives the adventure time to play back all the WAVS, and secondly, this is the speed jets really do taxi at as you will know if you have ever taxied all the way out to Runway 18 in Frankfurt in real life.

The best time for pressing CTRL+K to hear the captain's first passenger announcement is while taxiing, just after LH 4916 has received ATC clearance and your jet is level with intersection H of taxiway N (look out for the yellow sign "HN" on the left in Skylink's Frankfurt scenery). The second passenger announcement should be activated after the copilot has set the instruments just after passing NKR NDB. The last announcement should be made once you have contacted Ground Control in Stuttgart. By the way, the female Ground Controller who we initially talk to in Stuttgart is relieved by a male colleague when we land - hence the sudden change of voice.

11. Background ATC chatter

These flights are all taken from the relevant airline schedules. They are all flights which you could reasonably expect to find in the air at the same time as ours. Naturally there would also be other flights in the air at this time.

12. Features not implemented

This Adventure has no error recognition. If you fly to Saarbrücken instead of Stuttgart, it is your own fault and you will receive no clearance from ATC. If you head off to the wrong runway, you will not be given takeoff clearance. I would recommend pressing Ctrl+I now and again to check that you are going where you are meant to be, especially if something does not work and you have overflown a waypoint. Should you indeed have missed a waypoint, there is a little trick you can use to get back on course - switch to the slew mode and 'reverse'(!) your aircraft until it is inbound of the last navigation beacon and, using Ctrl+I aim at the beacon once again - the adventure will pick up where it left off. This is certainly better than breaking off the adventure just before finals and starting all over again in Frankfurt.<g>

13. Speakers wanted

I would like to continue to do more such flights in the future.

The more speakers I can recruit for these adventures, the more varied and interesting they will be for everyone concerned. If you always have to listen to the same old crew, these adventures will soon start to get boring too.

It really doesn't involve a lot of work for you - all you need to do is record a maximum if five or six WAV files - but what a difference it makes for the rest of us out there flying and listening to real controller messages. If several hundred people go to the trouble of downloading an adventure like this, why are so few prepared to put their hand up and offer their services? Come on, that could be you piloting a 747 or fielding the traffic at a busy control centre next time round. Just imagine how much more thrilling an adventure will be if you can actually hear yourself participating.<g>

My dream is to find controllers whose accents suit the locality, so that you can tell where you are from the accent of the air traffic controller you are talking to.

I also need more speakers for background ATC chatter, it doesn't matter where you are from - there could easily be an appropriate aircraft in the vicinity at the time. The more the merrier. And this is where we have an opportunity to outshine the commercially available adventures with their limited spectrum of voices and inauthentic accents. So come on, it's up to you - after all you're the ones that benefit in the long run.

So, drop me an email if you liked the flight, then I can tell you just what speakers I'm looking for the  next adventures.

I am grateful for any tips, praise or any other form of constructive feedback...<g>:

Email to Steffen Gerlach  	via Compuserve CIS/address: St_Gerlach

or

via Internet:  St_Gerlach@compuserve.com

P.S.  Please do not take it amiss when I take a bit longer to get around to answering your email.. The more of us there are on board, the longer it takes me to get around to replying - please do not be offended.

14. Credits / the voices

I would like to take this opportunity of thanking

Steffen Rabenau (Frankfurt Ground) for his dulcet tones, his professionalism, his speed... Don't worry Steffen, the next adventures are on their way. He is based in Berlin and runs his own travel/bus company

Thorsten Marx (Frankfurt Tower) for his support and co-operation. Don't worry, you'll get a bigger part next time round, Thorsten <g> He works as a surgeon in a Berlin hospital.

Wolfgang Schulte (Frankfurt Radar) for his immense help, patience and speedy replies to emails. He lives in Münster and has been a real asset for this project because he has a PPL. You may have heard him in a walk-on part in Joachim Kroll's adventures. Thanks for undertaking the test flight, for help with the flight plan and tips regarding approach plates and the Navigator 6 flight plan.

Peter Hiermeier (Stuttgart radar and designer of the Stuttgart scenery airport). He lives in Stuttgart and a big thank-you for helping out even though you didn't really have  enough time. I can't wait for your next scenery. And SPECIAL THANKS for the Swabian accent and, of course, the airport without which the landing would have been far less exciting. I chose this destination deliberately because of your marvellous scenery.

Bernd Hackner (Stuttgart Tower) for his spontaneous reaction and help with the beta version - there should be more users like you...

Martin Häusler (ATIS Stuttgart), Noble Air pilot and in the meantime HUB CAPTAIN ASSISTANT in the new HUB Munich. I would like to recommend his homepage, http://ourworld.compuserve.com/ homepages/Martin_Haeusler in this context. Thanks for your enthusiasm, tips and also for getting your sister (Stuttgart Ground) to participate who I would like to say an extra thank you to here. Also thanks for the FS5.1 test flights and the detailed reports.

Thomas Hulliger (Swiss Air 572 captain), lives in Zurich. Thanks for helping out and recording the wavs.

Sigrid Desczyk: I would also like to say a special work of thanks to my girlfriend Sigi (Frankfurt ATIS) for her immense patience and amazing tolerance regarding the many, many hours and long nights I spend at my PC. 'I'll just finish this one last file off quickly, OK??!'

Stefan Benzinger, a keen flight-simmer and tried and tested speaker in my last adventure to Munich. A doctor in a Clinic in Schongau in Bavaria (yet another surgeon). Thanks for providing the FS5 situation file.

Wilco van Deijl, (and Stephen Coleman) not just for developing his marvellous Ground Proximity Warning Systems (GPWS V7.2) for the entire FS community, but also for generously making it available free of charge.

Joachim Kroll: last but not least, I would like to thank Joachim for help with programming, for revising GPWS 7.2 - who knows whether I would have ever managed it on my own - and for giving me access to his work and answering all my emails. I don't suppose this project would have got off the ground without you.

Claus Kindt: my untiring advisor and former DFS air traffic controller - thank you for all your tips, explanations, and all that you have taught me about flying, giving me an insight into your own immense knowledge of this subject. Thank you, Claus, without you, I would not have achieved this degree of reality.

Jon Shepherd (voice of BA 922) a flight-simmer from Cologne who took the English translation off my hands to give me a bit more programming time.

Wolfgang Milius, who plays the Ramp Agent responsible for the pushback in Frankfurt

Peter Fuß, who took over from the female traffic controller in Stuttgart when she went for her break  - thanks a lot Peter.

André Wieczorek for filtering the Airbus waves.

Ralf Hürst (voice of LH1148)

Alain Nicco from France, (voice of Air France 1466)

Jörg Steinhauser (voice of LH 4816)

Markus Hölscher (voice of LH 4731)

Grisu (voice of LH 4916)

Frank Dunkelberg (voice of LH 1160 )

Tom Cobbley: just for being there to complete the list

And me, Steffen Gerlach, aged 33, a doctor working in - yes, you've guessed it - the  surgical department of a clinic. This time I speak the voice of the pilot on board LH 390.

If you have anything to say regarding this adventure, please do not hesitate to drop me an email - all feedback is welcome!

I would just like to add one last word here [that'll be the day], that this adventure is intended as a small thank you to all those authors of scenery, panels, adventures, tools and other add-ons for Microsoft Flight simulator and from whose hard work I've profited and who have made the flight simulator such an enjoyable experience. A big thank you to you all.

15. Version history

This is the fourth version of this adventure - and definitely the last one...<g>. Promise.

The first version appeared in August 1997: FRA-STR1 to FRA-STR3.ZIP. This version had a simple autopilot routine, no GPWS and no background chatter. It was basically a cheap and cheerful version.

The first upgrade appeared in October 1997 - it included GPWS and background ATC chatter: FRA-STR1 to FRA-STR3.ZIP plus LH390UP1 to LH390UP3.ZIP.

Christmas 1997 saw the publication of a modified version for FS98, expanded to include the Captain's passenger announcements and improved callouts: LH390-98-1 to LH390-98-3.ZIP.

This proved to be rather confusing to a lot of people since this version demanded all nine archived files, even though there were a lot of redundant WAV files etc. in there in the meantime. 

So to cut a swathe through this Gordian knot, this fourth version is complete and no longer relies on any of the old versions. You can, and indeed should, DELETE all the old files on your system relating to LH390. [Whoopee!]

The first version of LH390 in August 1997 with autopilot was very well received. However, a lot of you wanted to hear more radio contact. I too felt rather lonely out there only being able to hear my own radio messages and replies. After all, it's not as if I was flying the only plane over Germany at that time....

Wilco's very fine GPWS module offers the possibility of playing back random ATC chatter from the Flight Simulator/Flightshop repertoire or indeed from any other source for that matter. But this option did not really satisfy me either, because it is perhaps even more bewildering being able to hear a plethora of different controllers from the other side of the Atlantic plying one and the same frequency, making me wonder at times whether I hadn't fallen through a hole in the space-time continuum and finished up in the USA. It was then that I realized what I needed was ONE flight controller on one particular frequency talking to the other aircraft in the airspace under his or her control - as in real life.

I have now had these 'background messages' recorded specially by the participating controllers and indeed managed to recruit some new pilots from my slowly growing community to fly other aircraft in the area. Again, I have tried to be as realistic as possible. For example, we now have a British Airways B757 (Flight number BA 922) which travels daily from London Heathrow to Stuttgart (depart LHR 7:20 a.m. - arrive STU 09:50 a.m. from BA Timetable, 4th Aug. to 25th Oct '97) and a Swiss Air A321 (Flight number SR 572) travelling daily from Zurich to Stuttgart (depart 9:25 a.m. - arrive 10:05 a.m. from Timetable 30th March - 25th Oct 97). Due to a few minor delays, changes in weather conditions and other circumstances beyond my control, we now get to hear these aircraft as well...

16. Guarantee/ legal notice

This adventure is free software / freeware in accordance with the current legal definitions of the terms. Anyone can use it and pass it on to his or her friends (or enemies if you don't want to see them for a time... <g>). I do not mind you uploading it to any mailbox or Internet server which does not incur any extra download charges. In fact, I would actively encourage you to pass it on to other flight-simmers, just as long as no alterations are made.

However, any commercial distribution of this adventure is EXPRESSLY FORBIDDEN, especially publication on data carriers such as diskettes, ZIP, CD-ROM, DVD etc. without the author's prior permission.

This adventure has been tested by other users and myself with regard to functionality and compatibility. However, I can take no responsibility for any damage which you may feel has arisen in connection with this adventure, especially any resulting from lack of sleep or spending too long in front of a computer screen. <g>

As far as it is humanly possible to ascertain, this software was virus-free on publication. It has been tested with several reputable and up-to-date virus scanners. Unfortunately I can undertake no guarantee as to what might have happened to the software since it left my hands.

17. Key summary

Shift-D DECISION HEIGHT: 100 ft. fixed

Shift-G GPWS: g/s-inhibit /flap-inhibit/gear-inhibit/ALT/OFF/ON

R Radio Altitude

Shift-H HDG SELECT ->  enter a heading with keys 0-9

Shift-HH HDG HOLD

Shift-HHH HDG HOLD OFF

Shift-A ALT SELECT ->  enter a Flight Level with keys 0-9

Shift-AA ALT HOLD

Shift-AAA ALT HOLD OFF

Shift-1  HDG  <-  1 deg  Shift-7  ALT DN 100ft

Shift-2  HDG  ->  1 deg  Shift-8  ALT UP 100ft

Shift-3  HDG  <-  10 deg Shift-9  ALT DN 1000ft

Shift-4  HDG  ->  10 deg Shift-0  ALT UP 1000ft

AL  AUTOLAND OFF/ON

AC  AIRCRAFT CAT ->  enter category A-E for autoland

key < WPT SELECT  ->  only in WPT SELECT you can use these keys:

<< WPT NAV ON  0-9 select wpt 1-10 (or +/-)

<<< WPT NAV OFF  ~ select active wpt

| select next wpt

[Space] VIEW WPT INFO  > edit wpt (N/S E/W 0-9)

Shift-D delete wpt

A activate wpt

D direct to wpt

I wpt info

O position

Q quit wpt select

Space quit & view wpt info

< quit & wpt nav on

<< quit & wpt nav off

N NAV1 frequency ->  use keys 0-9 to enter a freq/crs

NC NAV1 course (OBS) ->

M NAV2 frequency ->

MC NAV2 course (OBS) ->

A ADF frequency ->

C COM frequency ->

CR COM RATE ->

CT XPDR code ->

NN swap NAV1 memory NT NAV1 course TO

MM swap NAV2 memory NF NAV1 course FROM

AA swap ADF memory MT NAV2 course TO

CC swap COM memory MF NAV2 course FROM

NM swap NAV1 <-> NAV2 AT ADF QDM (course TO)

AF ADF QDR (course FROM)

on the ground: K TAKE-OFF SPD CALLS ON  ->  use keys 0-9 to enter VR

KK TAKE-OFF SPD CALLS OFF

during flight: K SPD CALLS ON

KK SPD CALLS OFF

Ctrl-U   Exit the GPWS adventure

*German keyboard layout: ~ = ö    | = °    < = ,    > = ;    " = ä