FS2004 MV challenge - ATC deviations

Simulate a real flight from Florida to Chicago, including the many actual ATC deviations from the flight plan

Freeware by Michael Vone - mivone@pacbell.net

Web site http://avsim.com/hangar/utils/mvchallenges/

NOT FOR REAL FLYING:  FOR FLIGHT SIMULATION ONLY

___________________________________________________________________________

Overview

This challenge asks you to simulate a real flight in a B737 or B757, to experience how much Air Traffic Control can make a flight deviate from the flight plan, in this case mostly due to other traffic:  you will follow the flight plan only for a short while.  The flight takes about 2½ hours.

This simulated trip illustrates the limited function of the flight plan, the dominating role of Air Traffic Control in crowded airspace, and the limitations of FS2004's ATC.

Provided are:  a startup FS2004 flight, with preset weather and flight briefing;  a flight plan;  route maps.

Optional:  Add-on B757 of your choice.  See under Installation below.

___________________________________________________________________________

What to do with this document

There is no need to print out this document if you follow the detailed briefing supplied on-line for the flight.

But if you wish to follow your flight relative to the flight plan, you may wish to print out the two maps included in this document.

___________________________________________________________________________

Installation

IMPORTANT:  Carefully follow the installation instructions given below (they are also given in the file installAD.txt).  Because of the variety of ways FS2004 can be organized under different Operating Systems, I could not design a single automatic installation procedure.  You will need to access the folders on your hard drive with, for example, Windows Explorer or My Computer.

1)  Unzip m4atcdev.zip, using paths, to create and fill the two new folders:

C:\temp\ATC deviations

C:\temp\ATC deviations documents

2)  Find the folder "\Flights" within the folder "\Flight Simulator 9".  On many computers, this folder will be

C:\Program Files\Microsoft Games\Flight Simulator 9\Flights

if you have a standard installation of FS2004.

3)  Now MOVE the complete folder "\ATC deviations" with all its contents into FS2004's "\Flights" folder.  On many computers this will result in a folder

C:\Program Files\Microsoft Games\Flight Simulator 9\Flights\ATC deviations

4)  You may move the contents of the folder

C:\temp\ATC deviations documents

anywhere you wish:  FS2004 does not need them, so you can place them where it is convenient for you.  I place them on my computer in a new folder called

C:\My Documents\Flight Simulator Files\ATC deviations documents

5)  OPTIONAL:  For realism, you may download and install a B757 in United Airlines colors (possibly including a panel).  I don't have a specific recommendation for this, but you can search the libraries at http://www.flightsim.com and http://www.avsim.com/.

6)  I recommend maximizing your FS2004 weather settings, to get the full cloud density.

You start the challenge in FS2004 by selecting the flight from the flight category "ATC deviations":  this will load a default FS2004 B737 and preposition it, ready to be flown, with detailed on-line instructions.

___________________________________________________________________________

# Included files

- This document:  "FS2004 MV challenge - ATC deviations.doc" (Word2000)

- FS2004 files positioning a default FS2004 B737 at Orlando, Florida:

ATCdev-KMCO-KORD.FLT, .PLN, .WX & .htm;

- miscellaneous small files.

___________________________________________________________________________

# How we will fly

The goal is to simulate as closely as possible an actual commercial flight from Orlando, Florida, to Chicago, Illinois, including all actual deviations from the flight plan imposed by Air Traffic Control.

Since FS2004's ATC is not programmed for this, we will not use it.  Instead, I will supply on-line all the necessary instructions, as if given by ATC.  You will then execute the required turns, climbs, descents, speed changes, etc.

Initially, the flight plan will be preloaded into the GPS.  However, due to a limitation of FS2004, it will be lost soon after departure, when ATC commands a turn "direct to" a waypoint that skips the next 2 waypoints in the plan.  (The FS2004 GPS erases the flight plan when you use its "direct to" option;  the flight plan can later be reloaded but that requires some very inconvenient operations.)

To see the actual flight track in relation to the flight plan, look at the two maps shown below in this file.

To start the flight of this Challenge, select in FS2004 the category "ATC deviations" and load the flight that you find there.

Many settings, such as frequencies and autopilot settings, are already preset for the departure.

SAVING FLIGHTS EN ROUTE:  You may want to save your flight to your hard disk along the route.  This way you can later quickly restart the flight as those points, without having to fly again from the very beginning of the flight.

(But saving a flight does not save its briefing, so the original briefing will not be loaded when reloading the saved flight, unless you separately make a copy of the *.htm briefing file with the same name as the saved *.flt file and in the same folder - see below.)

To save a flight, press P to pause, then press ";" and fill in a name for it.  You could name such saved flights like this, for example ("Win1" is the round trip, "a" is its first leg):

ATCdev-KMCO-KORD(before descent)

ATCdev-KMCO-KORD(final)

Such saved flights will NOT appear in FS2004 under the "ATC deviations" category, but rather under "My Saved Flights", because they are stored in the folder C:\My Documents\Flight Simulator Files (or similar path, depending on your installation of FS2004).

___________________________________________________________________________

# The actual flight

The flight which we simulate flew on October 20, 2003, from Orlando, Florida (MCO/KMCO) to Chicago O'Hare, Illinois (ORD/KORD).  It was United Airlines flight 445, using a Boeing 757, on which I was a passenger.

The weather was generally clear at first, with some overcast during the second half of the flight:  nothing special.  For the simulated flight, I used the default FS2004 weather theme "Cold fronts", which includes some wind from the west.

The scheduled times of departure and arrival were 14:00 and 15:45, respectively.  The actual flight departed 1 minute late and arrived 2 minutes early.

On board, as is often the case on United flights, the captain fed all ATC communications into channel 9 of the passenger audio system, so that the passengers could follow the conversations between cockpit and ATC.  However, these were not audible during the PA announcements.  This allowed me to take note of most cockpit-ATC conversations, but I had to fill in some missing parts.

From this in-flight information, together with airway maps and departure and arrival plates, I later reconstructed the flight as well as I could, and simulated it in FS2004, as described in this Challenge.

The next two maps show both the flight plan and the actual flight track, as I reconstructed them.  Also shown are the ATC instructions along the track:  these are the instructions you will execute as you try to simulate this flight as closely as possible.  (The flight starts at the bottom of the second map, and then moves north onto the first map, ending at its top.)

On the maps, the ATC instructions are shown in red;  in green are navaids and other waypoints.  The reconstructed flight plan is shown as a dashed green line, while the reconstructed flight track is shown in red.  In orange are shown two pilot's requests, one of which was approved by ATC.  (The underlying map is copied from GNC 2, a Global Navigation and Planning Chart, by the US National Imagery and Mapping Agency.)

The flight progressed as follows:

At 14:12, clearance was given for takeoff:  that is the moment when this simulated flight starts on the runway.

The flight used the Jaguar SID (Standard Instrument Departure), which runs through waypoints GUANO, MATEO, VQQ (Cecil VOR) and YULEE to AMG (Alma VOR).  At AMG, the flight plan presumably joined airway J46 to AHN (Athens VOR) and VXV (Volunteer VOR), then airway J43 to FLM (Falmouth VOR).  From FLM, the flight plan must have gone to OKK (Kokomo VOR), where the Kokomo STAR (Standard Terminal Arrival Route) starts:  it goes via CLEFT, OXI (Knox VOR) and BEARZ to O'Hare.

Right after lift-off the flight was vectored slightly to the right of the SID track and kept at low speed, probably due to other traffic.  Soon, it was told to "resume own navigation", meaning that it should follow the SID.  However, already before waypoint GUANO (not shown on the map:  it lies about midway between KMCO and MATEO), ATC ordered "direct Cecil", so as to skip both GUANO and MATEO.

From Cecil, the flight then followed the SID to YULEE and halfway to AMG.  At that point, ATC ordered "20 degrees to the left for traffic".

From here on, never again would this flight rejoin the flight plan!  Due to other traffic our flight was repeatedly deviated so that it could not come back to the original flight plan.  The closest we got was from Volunteer (VXV) to Falmouth (FLM), but we were then under "direct Falmouth" instructions rather than under "resume own navigation" instructions.

The approach to Chicago was clearly encountering much other traffic:  our flight was made to descend quite early (an hour before touchdown, compared to the normal half hour);  it was also subjected to many turns and slowdowns, obviously designed to delay our arrival at a very busy airport.

Remarkably, despite all this, the flight arrived 2 minutes early at the gate:  this suggests that United had built these delays into their schedule.  Thus, these delays are normal for this flight.

___________________________________________________________________________

###### The simulated flight

All the necessary details for the flight are provided in the on-line briefing.

Use the flight briefing either in printed form (print it out as the flight loads), or on-line with the kneeboard (press F10).

___________________________________________________________________________

###### Deleting this Challenge and liberating hard-disk space

The files for this Challenge occupy about 1 Mb, as installed (and more if you have saved or created flights), mainly because of this document.

If you wish to recover that hard-disk space after completing this Challenge, do the following (your path may vary - see the installation instructions above):

- delete the folder "\ATC deviations" from "…\Flight Simulator 9\Flights\ATC deviations"

(… represents the path to that folder)

- in folder "…\My Documents\Flight Simulator Files", delete:

- the subfolder "ATC deviations", if present

- other files you have created, if you wish.

___________________________________________________________________________

# Copyright and Distribution

This package is released as Freeware.  © Michael Vone 2004.

As freeware you are permitted to distribute this archive subject to the following conditions:

The archive must be distributed without modification to the contents of the archive.  Redistributing this archive with any files added, removed or modified is prohibited.

The inclusion of any individual file from this archive in another archive without the prior permission of the author is prohibited.

No charge may be made for this archive other than a reasonable fee to cover the cost of its distribution.  If a fee is charged it must be made clear to the purchaser that the archive is freeware and that the fee is to cover the distributor's costs of providing the archive.

The author's rights and wishes concerning this archive must be respected.

___________________________________________________________________________

Disclaimer:  I am neither a pilot, nor affiliated with any organization involved in this Challenge.

___________________________________________________________________________