Cockpit Preparation HYD AIR ELEC

EVAG Sw	ARMD

Fuel Quantity	______  / CKD

Oxygen System / Mask	CKD

Status	CKD

PFD	TAXI

FMS	CKD / TAKEOFF

Auto Brake	T/O

T/O Warning System	CKD

FLAP T/O Select	______°

Rudder- / Aileron Trim	ZERO

Before Start AIR FUEL

Doors / Windows	CLOSED & LOCKED

Park Brake	SET

HYD AUX Pump 1	ON

ENG IGN	A or B

BCN Lts	ON

After Start HYD

APU	OFF

Windshield / Anti-Ice	____ / ____

Taxi

Flaps	FLAPS ______

Spoilers	ARMD

Flight Controls	CKD

Stab Trim	SET ______

Before Takeoff HYD AIR

LDG Lts / HI-Int Lts	ON

EAD	CKD / GREEN BOX

TCAS / Transponder	RA/TA / SET

After Takeoff

Gears	UP

Flaps / Slats	UP / RET

Spoiler	DISARMED

Auto Brake	OFF

EAD	CKD

----------------------------------------------------------Seat Belts / LDG Lts	OFF

Baroset	QNE

MD-11 Normal Checklist

Descent / Approach HYD

Windshield / Anti-Ice	____ / ____

Minimums	____ Baro / DH ____

Flap Selector	____

Seat Belt	ON

-----------------------------------------------------------

Altimeters	______QNH

LDG Lts	ON

Before Landing

Gear	DOWN / __GREEN

Spoiler	ARMD

Flaps	______ CKD

Auto Brakes	OFF / MIN / MED / MAX

EAD	CKD / GREEN BOX

After Landing

Spoiler	RET

Flaps / Slats	UP / RET

Stab Trim	3 ANU

Auto Brake	OFF

TCAS / Transponder	STBY

LDG Lts / Hi-Int Lts	OFF

Windshield / Anti-Ice	____ / ____

APU	ON

Parking HYD FUEL

Windshield / Anti-Ice	OFF

Engine Fuel Sws	OFF

Seat Belts	OFF

Emer Pwr	OFF

IRS	OFF

Parking Brake	OFF

Status Page	CKD

Leaving Aircraft

EVAC Sw	OFF

Emer Lt	OFF

EXT Lts	OFF

APU	OFF

Battery	OFF

A few explanations…

The “Quick and Easy”-checklist has some bold printed lines. If you like to go through the checklist as fast as possible you may execute only these lines - quick, but not very professional.

The “Quick and Easy”-checklist can also be used with the panel status “After Engine Start” beginning with the “Taxi Procedure”. But previously you should complete it with all lines showing a + or ++. The ++ means this one and the following.

► This symbol indicates always a condition: if / then do the following.

AIR, ELEC, FUEL, HYD within the headlines of this Normal Checklist have to be checked as well.

Abbreviations:

CHK	Check

CKD	Checked

DISC	Discontinuity

DISCT	Disconnect

EXEC	Execute

EXT	Exterior

EXTD	Extinguished

GUARD	Guarded

LT	Light

LTS	Lights

PERF	Perform

RCV	Receive

RCVD	Received

REQ	Required

RET	Retract(ed)

SW	Switch

SWS	Switches