F/A-18 HUD and panel (2D and 3D) for use with Team FS KBT’s Super Hornet. Built from scratch for FSX, and at higher resolution than previous versions. Works well within the 3-D virtual cockpit. Multiple zooms in 2D also supported. Very little framerate loss in fullscreen. The symbology is now much more realistic (almost identical, pixel for pixel, to the real Hornet’s. See pics and video within.) New HUD features added, including courseline steering arrow (can be linked to NAV1, NAV2, or GPS), multiple clock modes, realistic HUD reject options, altitude options, energy caret, etc. Many pics and vids within, as well as useful and informative documents, including actual carrier pattern specs.

# INSTALL

- If you haven’t already done so, install Team FS KBT’s SuperHornet into FSX. The file name at www.avsim.com is “fa18shexe.zip.”  It’s an FS2004 aircraft, but works fine in FSX. (There is an effects update somewhere out there.) Install into FSX by running the executable that’s within their archive, extracting into a temporary directory, then copying the folders into the relevant FSX folders. (Effects, and SimObject/Airplanes.)

- Make a copy of the entire FSX FA-18E_SuperHornet folder that resides under C:\Program Files\Microsoft Games\Microsoft Flight Simulator X\SimObjects\Airplanes.

- Rename that duplicate aircraft folder to FA-18E_SuperHornet HUD

- Extract the zip file to the FA-18E_SuperHornet HUD directory. It's okay to overwrite, as it's only the panel.cfg and aircraft.cfg files that are being overwritten.

- Put the included thumbnail into all FA-18E_SuperHornet HUD texture subfolders, so that you have an image in the aircraft selection screen.

- Choose a default 2D panel zoom. (0.7 is default, and you need to do nothing if that zoom is fine with you.) If you choose zoom 0.85 or zoom 1.0, rename the appropriate aircraft.cfg and panel.cfg files in the main aircraft folder, and panel subfolder, respectively. (If you fly almost exclusively within the 3D virtual cockpit, you need do nothing with respect to zoom. The HUD is lined up, and adjusts dynamically based on any zoom you choose.)

Note: only the HUD and the engine and fuel display used in the 2D cockpit are mine. All other gauges are from Team FS KBT original Hornet.

# WITHIN THE SIM

Once in the sim, note that there are two 2D cockpit options, default cockpit, and cockpit HUD. Cycle to  “Cockpit HUD”.  This view will have the proper zoom and symbology lineup. (No one yet seems to know how to remove the aircraft.cfg default views from the cycle, though the SDK suggests that it is trivial. It’s not documented.)

I recommend that you fly, even during the day

You can now fly any Superhornet variation included within the original files with this panel / cockpit.

The “Docs, Pics, and Vids” subdirectory has some interesting items, including information about the field and carrier landing patterns.

********************

# THERE IS VERY LITTLE FRAMERATE HIT WHEN USING THIS HUD IN FULLSCREEN MODE, at least not in this particular aircraft model. Framerate loss with realistic HUD’s has been reported elsewhere, in other planes, but I do not see it here, on my very modest machine. Perhaps 5 to 10% at most.

********************

## USE IN OTHER PANELS

When placed in other aircraft panels, this gauge must be used in proportion to its full, original dimensions (659 x 725), or else there will be strange results, distortions or ghost images in the ladder, etc. For accuracy with respect to the outside sim world, zoom must be adjusted accordingly. Note that zoom 1.0 in FS9 corresponds to a zoom of about 0.7 in FSX. (The FSX zoom 1 setting is more realistic with respect to forward angles, perceived distances, etc., but at the loss of peripheral visual input.)

Here’s a small list of possible gauge sizes, for a given zoom.

| zoom | 1.00 | 0.95 | 0.90 | 0.85 | 0.80 | 0.75 | 0.70 | 0.65 | 0.60 | 0.55 | 0.50 | 0.45 | 0.40 | 0.35 | 0.30 | 0.25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X size | 659 | 626 | 593 | 560 | 527 | 494 | 461 | 428 | 395 | 362 | 330 | 297 | 264 | 231 | 198 | 165 |
| Y size | 725 | 689 | 653 | 616 | 580 | 544 | 508 | 471 | 435 | 399 | 363 | 326 | 290 | 254 | 218 | 181 |

I know of no one who flies at less than zoom 0.5, however.

Even if you want to include the gauge in your project and aren’t so concerned about the symbology being “pinned” to the sim world’s, to avoid weird effects the gauge should still be used at its original proportions. Also, for some reason I’ve found that using size_mm= and pixel_size= (within the panel.cfg virtual cockpit section) should be at 512 x 512. It has to do with the gauge size in comparison to those 2 parameters.

Finally, the gauge should be placed in the panel.cfg so that its X coordinate is at  (1024 - desired gauge horizontal dimension) / 2 . The Y coordinate can be any, but you must use and adjust the VIEW_FORWARD_DIRECTION in the panel.cfg to accurately pin the HUD horizon to the sim horizon and scenery.

## SWITCHES PANEL

This panel is not realistic, but is needed to execute the HUD’s realistic features, and was designed with that in mind. Many of its features would normally be accomplished through the HSI and HUD control panel. The Hornet HSI is a very advanced instrument itself, and may be my next project. For now, this panel accomplishes its goal.

A FEW HUD SPECS:

ILS (ICLS) Glideslope needle: +/- 1.4 degrees deflection in elevation

Localizer needle: +/- 6.0 degrees deflection in azimuth

ACLS: tadpole limits are +/- 0.5 degrees in both azimuth and elevation

HUD Reject modes:

Reject 1: removes mach, G, peak G, bank angle scale/pointer (if gear down), energy caret (if gear down), and airspeed / altitude boxes

Reject 2: removes reject 1 items, plus heading scale, caret, pointer, and NAV info

HUD steering (courseline): each dot out = +/- 4 degrees; the dots are not displayed if within 1.25 degrees of the selected course.

Heading pointer: The pointer is NONLINEAR, as per the real F/A-18; The NAV station or GPS waypoint is +/- 30 degrees off nose if at the edge of tape visibility. Near the center of the tape, at +/-5 degrees, the scale does become linear.

AoA bracket: There are about  2.4 degrees of AoA inside the E-bracket.

AoA indexer:  See included doc.

# HUD OPERATION

Velocity Vector / Pitch Ladder:

The velocity vector / flight path marker shows the true flight path of your aircraft. Its position within the pitch ladder shows the aircraft climb angle. The ladder normally rotates around the velocity vector. However, if the true flight path leaves the HUD field of view, the velocity vector blinks at the edge of the HUD glass, and the pitch ladder then rotates around the waterline / longitudinal axis. (This shift is most appreciated when in a stall or during high AoA flight.)  Note that the pitch ladder defaults to on, but can be turned completely off. A switch is on the switches panel. (This is one of the very few unrealistic features of this HUD… but is useful and often desirable in cleaning up the clutter.)

So long as the installation instructions and aircraft setup have been followed, the velocity vector and pitch ladder are accurate with respect to the outside sim world. This is the point of the HUD, really. What this means is that your aircraft is actually traveling to the point the velocity vector overlays. And the +10 degree rung on the pitch ladder, for example, marks the point in the sky that is 10 degrees up. There is no offset or non-linearity.

At the correct zoom, and if you haven’t modified the VIEW_FORWARD_DIRECTION parameter within the panel.cfg, the 2D panel vector and ladder are very accurate. (As in the real HUD, though, the symbology sometimes lags behind by a few milliseconds.)

In the virtual cockpit, there is one and only one eyepoint setting which yields an accurate, precise velocity vector and pitch ladder. This eyepoint value is in the aircraft.cfg, and I recommend that it not be touched. (CTRL-SPACE resets this eyepoint while in flight.) One caveat: most people probably have “dynamic head movement” on, as it is by default, within the VC. What this very neat little feature does is actually shift the eyepoint slightly in response to aircraft accelerations. And what this means for the HUD user is that, during rapid accelerations (takeoff, for example), the ladder and velocity vector become slightly less accurate. They still behave properly in relation to one another, but may be a bit off as to the scenery. You MAY want to slightly reduce the dynamic head movement values the reside in the [DynamicHeadMovement] section within FSX.cfg. Many people, if the forums are any indication, believe these effects are overdone anyway.

# Please note that the HUD horizon SHOULD move up with respect to the “real” horizon as altitude increases. The degree to which this occurs is not a function of this HUD, but of the inner workings of the sim.  It may be slightly overdone, and is not something I or anyone can correct.

Eyepoint:

As mentioned above, there is one and only one eyepoint that yields HUD symbology which is accurate with respect to the outside sim world. In this particular aircraft, for reasons I will explain shortly, this eyepoint is very far forward, which means you have to look significantly down towards the rest of the cockpit instruments, almost as if you’re hovering over them.

“Why is the eyepoint so far forward?”

The HUD glass in this particular aircraft model is artificially a bit small. Also, for some reason related to how the model was made, the HUD gauge actually sits out in front of the HUD glass by a virtual foot or two. (This can be seen by moving the camera outside of the cockpit while in VC mode.) This is not something I’m able to rectify. Only the aircraft modelers can.

As I hope that realistic HUD’s will become the norm in the future, it might be nice if aircraft designers did the following:

- Make the HUD glass the proper size, at the proper distance from the pilot’s “head,” so that the glass occupies the correct field of view when the eyepoint is at an accurate location.

- Make the “gauge-holding” component of the HUD glass completely vertical. Otherwise, as I’ve confirmed with the Aerialfoundry Hornet, Abacus Hornet, and others, the symbology is at different scales at different ends of the HUD glass. Even if this weren’t part of the VISUAL model, perhaps there’s a way to make an invisible surface? I do not design aircraft, so don’t know.

**Remember that CTRL-SPACE resets the eyepoint to its ideal location.

# Courseline Steering:

This is essentially an HSI tacked onto the velocity vector. It can be linked to either NAV1, NAV2, or GPS, via the switches on the HUD controls panel. The desired course can be selected using the dials at the bottom.

In the carrier pattern:

Link the NAV1 radio to the carrier’s ILS system. Link the NAV2 to the carrier’s VOR, or the GPS to the carrier’s GPS waypoint. (Nav 2 is preferred, as the position of its needle is updated more frequently than is that of the needle linked to the GPS.) Tune the NAV 1 course to the carrier runway’s heading (not essential, but a good reminder.) Tune the NAV 2 or GPS course to the CARRIER’S heading, called the Base Recovery Course (BRC), which is about 8 degrees to the right of the runway heading (depending on the carrier or the carrier model designer.) Turn the HUD steering switch to NAV2 or GPS, as appropriate.

These dots / needle are useful because they give an instant reference as to your heading with respect to the BRC. They’re especially nice when in the turn from base to final, as the needle crosses the center of the vector at the instant you’ve crossed the ship’s stern. This tells you whether you need to tighten your turn or widen it in order to roll out on the runway’ extended centerline.  Though you should be primarily visual at this point.

See the “HUD_courseline_steering.jpg” in the Docs folder for what the symbology will look like.

HUD Clock:

There’s a switch on the HUD control panel which allows you to adjust what the HUD clock displays. The following are available:

- Local time

- Zulu time

- ET, or Elapsed Time (Basically a stopwatch, which runs from the moment the ET Reset button’s pressed.)

- ETE, or Estimated Time En Route (Time until arrival at navaid / waypoint)

CARRIERS

To get carriers in FSX, download the addon scenery, filename “carr2006.zip,” from www.flightsim.com. It can also be found elsewhere, though I don’t think it’s on AVSIM. Also do a search for “rcbco-30.zip,” the updated version. I forget where I found this, but thank goodness for Google….

ISSUES:

- THERE IS ONLY ONE EYEPOINT SETTING THAT KEEPS THE VELOCITY VECTOR AND PITCH LADDER COMPLETELY ACCURATE. (I have already put it in the aircraft config file. You should not change it.) This leads to 2 problems:

- the eyepoint seems much too far forward, close to the cockpit dash, meaning you must look down at a very steep angle to see the other instruments. This had to be this way, because the model appears to have an artificially small HUD glass, with an artificially small field of view (FOV). Ilso, the gauge “floats” out a foot or so in front of the HUD glass, for reasons again related to the model. To get the correct FOV, and keep all HUD symbology accurate with respect to the outside sim world, this had to be done. The HUD is the primary flight instrument in a modern tactical jet, so this “solution” should seem less painful.

- With the wonderful “Dynamic Head Movement” feature in FSX, the eyepoint changes a bit when significant accelerations occur. This means that, when rapidly speeding up or slowing down, the vector / ladder become a bit less accurate. It’s not much, but it is noticable.

- In VC mode, there is now way to place an AoA Indexer where it belongs. I believe this is a limitation of the model

- Energy caret is actually an “acceleration caret,” not a speed trend indicator. It’s less jumpy this way, though is too responsive to added G’s. It’s basically the same when in landing config at approach speeds and little load.

- Eyepoint needs to be precise in VC mode. CTRL-SPACE resets it back to its precise default.

- The ladder’s switching between rotation points (vector vs. waterline) is too abrupt. smoothing that out is next on my list, and may be included in an update. I wanted to get this file out now, however, and have had enough of trigonometry for a few weeks. ;-)

- With this plane, rudder control and yaw are a little questionable. Perhaps it’s the port to FSX? This is not a function of the HUD, but the .air file.

- There is no look ahead radar. The recovery arrow / pull up cue bases its calculation on current radar altitude, using the terrain height directly underneath the aircraft. Therefore, in hilly or mountainous areas, it’s less reliable.

- In one “practice” version I tried, I did the top half of the pitch ladder using vector graphics, like this:

<Element><Clip Top="100" Left="105" Right="570" Bottom="670"/>

<Position X="330" Y="287"/>

<Text X="42" Y="25" Bright="Yes" Length="2" Font="tahoma" Color="#FFFFFF" Adjust="Right" VerticalAdjust="Center" Multiline="No" Fixed="No">

<String>5</String>

<Axis X="132" Y="161"/>

</Text>

</Element>

<Element><Clip Top="100" Left="105" Right="570" Bottom="670"/>

<Position X="330" Y="287"/>

<Polyline Color="#FFFFFF" LineWidth="2" Bright="Yes">

<Point X="85" Y="-139"/>

<Point X="85" Y="-155"/>

<Point X="36" Y="-153"/>

<Pivot X="0" Y="0"/>

</Polyline>

</Element>

<Element>

<Position X="330" Y="287"/>

<Polyline Color="#FFFFFF" LineWidth="2" Bright="Yes">

<Point X="-36" Y="-153"/>

<Point X="-85" Y="-155"/>

<Point X="-85" Y="-139"/>

<Pivot X="0" Y="0"/>

</Polyline>

</Element>

<Element><Clip Top="100" Left="105" Right="570" Bottom="670"/>

<Position X="330" Y="287"/>

<Text X="42" Y="25" Bright="Yes" Length="2" Font="tahoma" Color="#FFFFFF" Adjust="Left" VerticalAdjust="Center" Multiline="No" Fixed="No">

<String>5</String>

<Axis X="-91" Y="161"/>

</Text>

</Element>

…followed by the apropriate shift and rotate commands…

Unfortunately, there was a SIGNIFICANT framerate hit. I’m not sure there’s supposed to be. Anyone who knows why he above method doesn’t work well, please let me know! Vector graphics do improve the look of the pitch ladder. The performance hit was not worth it, though, so… bitmaps it is.

The same goes for using formatted text in the heading tape. Not so much performance, but those terrible dark anti-aliasing jaggies appeared, as the do with some other text. Why?

THANKS:

- Team FS KBT for their SuperHornet

- The very knowledgeable people on Avsim’s panel and gauge design forum.

- All whose XML gauge work I peaked into to get the syntax down.

- The FSX team at Microsoft, for a nice sim, and a GREAT Service Pack 1.

LEGAL:

This gauge is Copyright, me, 2007.

However, freeware authors are free to use it, modify it, repost it, etc. in their freeware projects. Payware authors should contact me before including it in their projects.

Thanks,

Scott Printz

saprintz@gmail.com