Piasecki H-21C & Vertol V-44 For Microsoft Flight Simulator X By Mick Posch 6-3-2007 Checklist NOTE: For more detailed information, use Checklist_Annotated.doc Camera View Keystrokes These need to be assigned; see the manual, under Assigning Camera Views. 4 – Engine Gauges 5 – Overhead Panel 6 – Center Console 7 – Pilot's View 8 – Floor - Right 9 – Floor - Left Useful Default Keystrokes Other useful keystrokes – already assigned by default – include: (Shift)2 – Radios (Shift)3 – GPS A – Toggle cameras within category S – Toggle between camera categories (cockpit , outside, and tower views) (Shift)E – Open/close front door (avail. only on civilian versions) (Shift)E then 2 – Open/close rear door (avail. only on civilian versions) F9 – Return to default eyepoint

This aircraft's real-world checklists have been modified for use with Flight Simulator

| Preliminary | Master shutdown button: ACTIVATE (Shift 4) |
| --- | --- |

| Interior Check | Seat: ADJUST |
| --- | --- |
|  | Safety belt & shoulder harness: FASTEN |
|  | Directional pedals: ADJUST (to comfortable position) |
|  | Flight Controls CHECK (for freedom of movement) |
|  | Battery switch: ON |
|  | Dome light switch: ON (night flights) |
|  | Collective Pitch Lever: DOWN & LOCKED |
|  | Nosewheel UNLOCKED |
|  | Parking Brake: ON |

| Console Check | Cyclic Trim: ZERO |
| --- | --- |
|  | Fuel mixture lever: IDLE CUTOFF |
|  | Carburetor heat lever: COLD |
|  | Oil temperature control switch: AUTO |
|  | Friction clutch switch: DISENGAGED (with warning light out) |
|  | Supercharger switch: LOW |
|  | Fuel Booster Pump Switch: OFF |
|  | Ignition switch: OFF |

| Overhead Panel Check | Inverter switch: ON |
| --- | --- |
|  | Generator switch: ON |
|  | Fire detection switch: TEST |
|  | Engine fluids switch: NORMAL |
|  | Pitot heat switch: OFF |
|  | Hoist switch: OFF |
|  | Heater switch: OFF |
|  | Blower switch: OFF |
|  | Navigation lights: OFF |
|  | Instrument lights: OFF |
|  | Windshield wipers: TEST |

| Starting Engine | Fuel mixture lever: NORMAL |
| --- | --- |
|  | Fuel booster pump switch: ON |
|  | Friction & jaw clutch switches: CHECK WARNING: Damage to the rotor system will result if starter is activated with friction or jaw clutch engaged |
|  | Starter switch: ENGAGE |
|  | Ignition switch: BOTH |
|  | Primer switch: AS NEEDED |
|  | Fuel mixture lever: RICH |
|  | Oil pressure & temperature: CHECK (stop engine if the oil pressure does not register within 10 seconds or reach 40 psi within 20 seconds |
|  | Navigation lights: ON |

| Starting Rotors | Parking Brake: ON |
| --- | --- |
|  | Nosewheel UNLOCKED |
|  | Collective Pitch Lever: DOWN & LOCKED |
|  | Directional pedals: Neutral position |
|  | Friction clutch: ENGAGE Lock switch in this position with the guard for all rotor operations |
|  | Rotor speed: VERIFY IN SYNC with engine speed |
|  | Jaw clutch switch: ENGAGE CAUTION: Do not engage the jaw clutch switch before engine & rotor rpm have synchronized |

| Ground Tests | Hydraulic pressure control valve: ON Check pressure 1000 psi |
| --- | --- |
|  | Generator output: CHECK (27 1/2 volts) |
|  | Carburetor heat control lever: ADJUST as needed |
|  | All temperature and pressure gauges: CHECK within green zone |
|  | Radio: ON |

| Taxiing Instructions | Parking brake: OFF |
| --- | --- |
|  | Use the cyclic to move the helicopter forward or backward; use the pedals to provide directional control. Differential brakes should not be used to provide taxiing control except in emergency. To slow or stop the helicopter, displace the cyclic rearward, while increasing collective pitch. |

| Prior to Take-off | Hydraulic pressure: CHECK 1000 psi |
| --- | --- |
|  | Fuel booster pump switch: ON |
|  | Supercharger switch: LOW |
|  | Fuel mixture lever: RICH |
|  | Cyclic Trim: CHECK |
|  | Crew & passengers: Alerted and ready for take-off |
|  | Area: CHECK (clear for take-off) |
|  | Parking brake: OFF |

| Take-off | Collective: UNLOCKED |
| --- | --- |
|  | Heading: Helicopter should be headed into the wind, when practical |
|  | Increase collective until airborne |
|  | All temperature and pressure gauges: CHECK within green zone |
|  | Carburetor air temperature: ADJUST if necessary |

| After Take-off | Cyclic trim: AS NEEDED |
| --- | --- |
|  | Fuel mixture lever: NORMAL |
|  | Carburetor heat control lever: ADJUST if necessary |

| Traffic Pattern, Approach, & Landing | Landing light switch: ON (night flights) |
| --- | --- |
|  | Landing area: CLEAR |
|  | Heading: Into the wind, when practical |
|  | Nosewheel: UNLOCKED for taxiing |

| Shutdown | Transmission temperature and pressure gauges: CHECK within green zone |
| --- | --- |
|  | Jaw clutch switch: DISENGAGE |
|  | Friction clutch switch: DISENGAGE |
|  | Oil dilution: AS NEEDED CAUTION: Do not dilute oil when oil temperature is above 50 degrees C. |
|  | Cylinder head temperature: CHECK Below 150 degrees C |
|  | Fuel mixture lever: IDLE CUTOFF |
|  | Ignition switch: OFF |
|  | Center console - All switches: OFF |
|  | Hydraulic pressure control valve: OFF |
|  | Parking brake: ON |
|  | Friction clutch switch: ENGAGE |
|  | Overhead panel- All switches: OFF |