Modify Weight Distribution for Project Open Sky’s Boeing 757-200

Background:

Some problems were found with the default weight and balance figures for this aircraft.  Discrepancies with the numbers caused flight problems and in many cases the aircraft was not easily able to climb to cruising altitude above FL270 without excessive pitch and the resulting stall.

Changes:

Using the scale model diagrams and seat layout (2-class seating) provided on Boeing’s website, accurate numbers and locations of weights were pinpointed on the aircraft.  These changes are seen in the information below.  The attached graphic shows locations and distances of each region.

Installation:

- Open the “Aircraft.cfg” file in the POSKY 757-200 folder.

- Scroll down to the section that says “[WEIGHT_AND_BALANCE]”.

- Delete all information in that section (down to “[flight_tuning]”).

- Copy and Paste the following lines in place of what was just deleted.

max_gross_weight = 256000.000

empty_weight = 130875.000

reference_datum_position     = 0.000, 0.000, 0.000

empty_weight_CG_position     = 0.000, 0.000, 0.000

max_number_of_stations =50

station_load.0 =    170.0,  70.4, -3.0,  2.0

station_load.1 =    170.0,  70.4,  3.0,  2.0

station_load.2 =   1440.0,  46.4, -3.0,  2.0

station_load.3 =   1440.0,  46.4,  3.0,  2.0

station_load.4 =  17460.0, -10.0, -3.0,  2.0

station_load.5 =  17460.0, -10.0,  3.0,  2.0

station_load.6 =   7900.0, -33.6,  0.0, -5.5

station_load.7 =   3300.0,  32.0,  0.0, -5.5

station_name.0 = "Pilot"

station_name.1 = "Co-Pilot"

station_name.2 = "First Class A-C"

station_name.3 = "First Class D-F"

station_name.4 = "Coach A-C"

station_name.5 = "Coach D-F"

station_name.6 = "Rear Cargo"

station_name.7 = "Front Cargo"

;Moments of Inertia

empty_weight_pitch_MOI  = 2995645.000

empty_weight_roll_MOI   = 1873855.000

empty_weight_yaw_MOI    = 5171429.000

empty_weight_coupled_MOI = 0.000

CG_forward_limit= 0.000

CG_aft_limit= 1.000

- Save file and enjoy!

This was created by Justin N. Sen.  If you have any questions about this, please email me at lasallecrew2@gmail.com