PROFESSIONAL          APPROACH GUIDE

INTO TNCM

By Lewis Duncan.

USE THE CHART ALWAYS WHEN COMING INTO TNCM.

The approach starts at 3500 feet, over the ST. MARTEEN VOR (PJM - 113.0). Intercept and track radial 318 until 7 DME from the VOR, where you can start turning left to enter the 9 DME ARC. The concept behind DME Arcs is simple: imagine that you, with a compass, draw a circle with its center over the VOR and with a radius of 9 nautical miles. Then, follow the part of this circle required by the plate and you're done. In this case the DME ARC goes from from radial 318 to 276; exit this circle so that you reach radial 276 as far as 7 nautical miles from the station. Once you have visual contact, align visually, at your 7.0 DME from the VOR you should then descend, I find that -700 fpm works when your 7.0 DME from the VOR you start that. At 2.0 DME you should think if you’re too high or something’s wrong you should then think about a go-around at that point. When you come closer to about 1.0 DME you should then adjust the fpm because the vasi lights always change at around 1.0 DME in TNCM. Then at 100 feet drop your speed by going to idle and flare at 30 feet. Easy! You can find the chart at www.fscharts.com