GNS530 Default Nav Page

Com Active and Standby Frequencies

Nav Active and Standby Frequencies

Power Button

Com Active/Standby Frequency Switch

Nav Active/Standby

Frequency Switch

Ident Switch

Nav1 Ident

Nav1 Radial

Nav1 Distance

Com/Nav

Set Switch

Decrease Frequency (MHz)

Decrease Frequency (KHz)

Radio Frequency

Set Switch

Increase Frequency (MHz)

Increase Frequency (KHz)

Nav/GPS Switch

Nav Bar

Switch

- The power button turns the unit on and off.

- The top number in each radio box correspond to the active frequency, the bottom number correspond to the standby frequency.

- The active/standby switch toggles the active frequency to standby and the standby frequency to active.

- Nav1 ident readout is the 3 letter code of the Nav1 beacon tuned in on the Nav1 frequency.

- Click on VLOC knob to ident. ID appears in the upper right corner.

- Nav1 radial is the radial degrees from the Nav1 beacon. Convert to “direct  to” by adding 180.

- Nav1 distance is the distance from the Nav1 beacon.

- The radio frequency set switch is used to tune the radios. The click areas are shown in the figure above.

- The Com1/Nav1 set toggle switch is used to toggle between tuning the Com and Nav radios.

- The CDI switch ( Nav/GPS switch) toggles between GPS and Nav1 navigation modes.

- The Nav bar switch displays flight information in a boxed rectangle along the right side of the screen. The bar can be displayed in the default or map pages. See the accompanying file “530 Nav Bar Page”.

- The CDI displays course deviation information in either Nav1 or GPS navigation modes. In the Nav1 mode, the CDI works exactly like the standard CDI display found on most aircraft. The deflection distance of the CDI needle corresponds to ~1 mile on each side of the desired track in the Nav1 mode. It corresponds to a ~5 mile distance on each side when tracking a waypoint in the GPS mode.

- All other buttons and their functions are the same as in the FS2004 default GPS500 unit and are described in the help section of the simulator. Please refer to this section for information on the functions.

- Com/Nav standby frequencies are highlighted in cyan when setting frequency.