## Piper PA-28-181 Archer II PH-KAX KLM Aeroclub

Aircraft by Rien Cornelissen

Panel and repaint by Arjan Harmans

# Panel information

All the gauges in the 2D panel are located at the same location in the virtual cockpit.

The virtual cockpit has no magnetic compass so I suggest you open the compass window in virtual cockpit view (icon in bottom left corner or shift+3).

- Press-to-test annunciator lights will light up

- Panel lights switch activates panel lights and nav lights

- com1 radio standby window Use your mouse to change standby frequency, pushbutton will swap to active. Top left unit is com1, top right nav1. Bottom left unit is com2, bottom right nav2.

- Transponder Use your mouse to change numbers.

- Directional gyro and (5a) magnetic compass. Notice the wrong indication of the directional gyro compared to the magnetic compass? It will do so everytime you start your flight with the Archer. Use button 5 to align (or press <D> on your keyboard). Check gyro heading against magnetic heading in level flight regularly, the gyro will drift with time and in steep turns.

- Rudder trim. You will need this one if you have your realism settings set to the right because the engine and prop forces will induce left or right turns under different power settings and airspeeds.

- Primer.

- Carburetor heater.

- Fuel selector. Don’t forget to change to other tank every 30 minutes of flight or you will have  a problem with balance.