Socata TB20GT Trinidad Public Release v1.0

NOTE!  This is a very advanced panel, with many custom features! Please read the [Usage] instructions carefully, so you will avoid unpleasant surprises... :)

# Credits

Bill Leaming: XML gauge programming and panel design (n4gix@attbi.com)

Warren Tucker: graphics artist

Bob Samuelson: GMax model

Bill Leaming Virtual Cockpit in GMax

Fabio Miguez: preliminary flight dynamics

Tom Goodrick: final tweaking of flight dymanics

Special Credits

Many of the gauges were made possible only because of the kind efforts and mentoring of Karl Peterson, Arne Bartells, Dai Griffiths and Bill Morad.  Bill Morad's programming of a C++ gauge was critical to getting this to work, because Microsoft "omitted" certain key k_events from this version of XML!

Also, in one sense this entire project was the result of collaborative efforts and sharing of knowledge from darn near everyone on the Panels & Aircraft Design Forum at AVSIM.com!!! :)

Commercial use or making money from any included file is prohibited.

OPEN GNU program for the SOCATA TB20 Trinidad aircraft, a 4-5 seats training, business and travel machine.  GMax, retractable gear with Aircraft Animator moving flaps, prop, rudder and elevator, opening gull-wing doors & cargo door, and variable pitch propeller.

# Introduction

This model is the culmination of over 5000 man-hours of work, and represents a significant contribution, in spite of being the "first aircraft" to roll off the assembly line of the Factory Aircraft Rebuilders Technology Systems team.

Avionics include the following:

KMD550 MFD (mask) w/Radar Gauge by Eric Marciano

KLN94B GPS

KCS55 Bendix-King slaved HSI (NAV1)

Bendix-King VOR2 ILS/GS (NAV2)

Bendix-King dual-needle RMI (ADF+NAV1/2)

KR87 Bendix-King ADF

KN63 Bendix-King DME (NAV1/2 selectable)

KT76C Bendix-King Transponder

2x KX155A Bendix-King COM/NAV radios

KMA 28 Audio Control Panel

MAP/FuelFlow on P2

Tachometer on P2

The "copilot" panel is accessed using the cleverly designed "P" Icon located under the RMI...

MAP/FuelFlow

Tachometer

#2 Electric Horizon

#2 Altimeter

EDM 700 EGT/CHT

Shadin Fuel Management Computer (customized for Socata)

Likewise, the Yoke (with fitted Astrotech LC2 Timer) is accessed using the "Y" Icon on the left side of the panel.

Installation - Complete Kit!

Simply unzip the TB20GT.zip file to your FS2002 folder.  That's it! :)

# Usage

When you first load the a/c, you must follow these steps to properly initialize the avionics!

1) Open the Throttle Quadrant and turn ON the Master Battery and Alternator.

2) Turn on the first four 'pushbuttons.' These are Fuel Pump, Recognition, Strobe and Navigation Lights.

3) Close the Throttle Quadrant.

4) Click on the "P" Icon to switch to the copilot's panel.

5) Turn ON all the avionics, beginning with the KLN94B "GPS" unit, followed by COM/NAV1 and then COM/NAV2.

5a) Clicking on the center of the ON/OFF knob on the radios will put them into "Test Mode." Make sure all LED segments are lighted, then click the center again to place the radio in ON mode.

6) Click on the "P" Icon to return to the pilot's panel.

This panel is sized to be moved by grabbing the panel with your cursor, clicking the left button and sliding it up and down as needed for IFR/VFR flight.  When down moved down to where the lower portion is hidden, the view out the windscreen is virtually unobstructed, since there is no center pillar to block your view!

Even with the panel fully visible, the view is still very, very broad.

As a practical matter though the center pedestal does partially block the view of the runway on descent, but on the flip side it provides a dandy 'marker' for aligning the aircraft with the runway! If the runway is just left of and parallel to the center pedestal, your aircraft is also lined up and parallel to the runway... :)

There are two light switches located on the far left side of P1.  The bottom switch will turn on the panel lights.  The upper switch will turn on the wonderfully backlit instruments!  The backlighted gauges in the Virtual Cockpit are tied to the landing lights, so if you fly at night, you must leave the landing light on!  The actual landing light will turn off when the gear is retracted.

The normal assortment of 'simicons' are provided, including one to display the yoke and throttle quadrant.  A view of the GPS is also selectable from the usual icon.

The KMD550 will display Eric Marciano's excellent Radar Gauge. There are "clickable" mouse points on the large buttons at the bottom of the display, in the following order.  Simply play with them a bit to see what they do! :)

Mode Select-   Mode Select+     Range-    Range+

The KLN94B has several 'mouse points' as well:

1) The middle button on the bottom row will toggle the GPS display.

2) The rightmost button on the bottom row will toggle NAV/GPS lateral guidance.

## EDM 700 and Shadin Fuel Management Computer

The EDM 700 is an advanced gauge that will display a bargraph of Exhaust Gas Temperature for each of the six cylinders, as well as a bargraph of the Cylinder Head Temperature. At the bottom of the gauge, there is a digital readout of EGT/CHT.

The Shadin Fuel Management Computer will allow you to 'fine tune' the Mixture control to optimize fuel usage (economy vs. performance). It will also provide a digital reading of the Fuel Flow, the Fuel On Board, the Fuel Used, and Endurance (approximate flight time remaining).

These two gauges are used together to lean/enrichen the mixture (make *sure* that you have "Automixture" turned off in FS2002, otherwise these will not work properly!

1) When starting the engine, open Mixture Full Rich.

2) After rotation, and during climbout, slowly reduce Mixture until Fuel Flow is approximately 18 GPH.

3) You may need to lean a bit more, if you are climbing > 8,000' or are operating from high altitude airport.

4) After leveling out at cruise altitude, slowly pull the throttle back to MAP 20.

5) Slowly lean Mixture, while watching the EDM 700's EGT display. Note where the 'peak EGT' is reached.

6) Slowly enrichen the Mixture to ~20 degrees RICH of the 'peak EGT' point.

For example, if 'peak EGT is 1640, set the Mixture until EGT reads ~`1620.

7) Fuel Flow on the Shadin should read between 12 and 13 GPH between 6500' and 8500'.

8) For best "Performance," set the Mixture to ~60 degrees RICH of peak. Fuel Flow will be ~15 to 18 GPH.

------------------------------------------------------------------------------

## KMA 28

Audio Amplifier/Intercom/Marker Beacon Receiver

------------------------------------------------------------------------------

### KMA 28 Operation

#### Receive Audio Selectors

Receiver audio is selected through two momentary and six latched pushbutton, backlit switches.  Com1 and Com2 are the momentary switches.

Users can identify which receivers are selected by noting which of the green switch LEDs are illuminated.  Pushbuttons labled Nav1, Nav2, DME, MKR  (Marker), DME and SPK (Speaker) are “latched” type switches.  When one of these buttons is pressed, it will stay in the “in” position.  Press the switch again and it will be in the “out” position and remove that receiver from the audio.

The KMA 28 is equipped with an electronic “click” to provide additional feedback for button operation  (see Installation Notes below).

To monitor both COM1 and COM2, press the SPK button.  Please note that COM1 and COM2 select which will transmit, they are mutually exclusive; you cannot have both selected at the same time!

------------------------------------------------------------------------------

## Astrotech LC2

Timer/Chronometer/Clock

------------------------------------------------------------------------------

### LC2 Operation

#### Mode Button

- The red mode button will switch between the Timer-Clock Mode and the SimRate Mode.

- Clicking “Timer” will decrease SimRate; “Clock” will increase SimRate.

#### Time/Date

- Clicking “DT/AV” will cycle through Time/Day-Month/Year displays.

#### Clear/Start/Stop

- Clicking on the red button above “ST/SP” will cycle through the Timer’s Clear/Start/Stop functions.

#### Chronometer

- Clicking on “SET” will toggle the Chronometer (Accumulated Time) function On/Off.

- Clicking on “RST” will clear the Chronometer’s Accumulated Time.

# Disclaimer

There is no reason to suppose that use of this panel will harm your computer system in any way, but in the unlikely event that it does, neither I nor my boss are liable.

Bill Leaming

n4gix@attbi.com

http://catholic-hymns.com/frbill/FS2002_Socata_Project/