Tu-144  Glideslope Autoland

Preconditions for the Autoland System:

- Fuelquantity of the full loaded Aircraft: 10 – max. 60% (Dump Fuel if more!)

- The COG must be adjusted before activating the AL system.

- Maximum Crosswind (Surface Wind / Windshear) during aproach: 16 kts.

- Airport must have NAV Station with Localizer and Glideslope Signal.

- NAV1 Signal with DME recommendet.

Einleiten von AUTOLAND:

1

Fly the plane normally making a normal descent. If you are flying IFR and wish to use the auto-

land feature, cancel the IFR at about 7000 feet, as the approach instructions given to you by the

FS ATC will not be the same as the approach that the autoland system will make.

2

set the ILS frequency in NAV1 (OBS Course at HSI Gauge)

3

set the airport DME, if availiable, in NAV2.

Remark:

Some Airports have the DME signal together with a DME. A usable DME signal can be determined on the DME receiver on the overhead panel. Autoland tries to find a DME on both NAV receivers.Autoland needs DME signal to set the correct airspeed. If theres no DME availiable, Autoland will set the Airspeed by calculating the Altitude. However, without DME, Autoland is much less precise.

4

your heading should not differ more than +-30 degrees from the runway

5

Between 25 – 20 Nm distance, maintain 4000 Ft at  250 Knots.

6

Switch (1) to  „armed“ . A centering monitor pops up (4).

7

The Joystick has to stay in ZERO position to activate Autoland

and has to stay there until the final touchdown. The colour changes from red over yellow to green when the joystick is moved into its neutral position.

8

As soon as (3) starts flashing, NAV1-Sig of the Localizer has

been captured. Press (2) to activate Autoland

From now on until Touchdown the Aircraft is under total Computer Control.

Computer Procedures :

- Altitude is reduced to 2800 Feet.

The gauge tracks the plane’s height via the radar

altimeter so that you will always be targeting the set height above ground level, regardless of the

altitude of the airport. This will mean that the plane may rise and fall as the land rises and falls.

This is normal.. Canards will be deployed

- At 18 Miles ILS DME:

Sets speed to 220 Knots IAS (without DME, 220Knots will be set at 3000 ft

Lower Nose to 11 deg.

- At 14 Miles ILS DME:

Sets Speed to 200 Knots IAS. (At 2600 ft without DME)

Lower Nose to 17 deg,

Flaps set to 15 deg..

- At Localizer captured

Lamp (2) blinks green,  Autopilot + Approach hold activated.

Flaps set to 30 deg.

- At Glideslope captured

the indicator lamp is solid GREEN.

reduces airspeed for approach to 152 - 186 kts IAS (weight-dependent) to get a pitch of 8-10 deg.

Flaps set to 45 deg.

Autobrake active, Chutes armed

- at 900 feet above the runway:

lowers landing gear.

- at 700 feet above the runway:

Switches on the landing lights.

- at 220 feet above the runway:

Sets trim for landing

- 50-80 feet:

Throttle to 10%. Last Trims.  “Wing Leveler” active.

- At touchdown:

Throttle to idle, Chutes deployed, Autobrake active

- at 100 kts:

switches on the taxi lights.

- at 55-60 kts:

switches of the landing lights.

releases autobrakes.

decreases nose and flaps.

switches off the pitot heat and ice detection.

switches off the autopilot master.

switches itself off.

At this point, you are slowing down on the runway below 55 kts, with the autoland system switched

off and all controls except one returned to you so that you can taxi off the runway.

PAPI and VASI

The Glidepad can be controled by Lights on the right side of the runway. Depending on the Airport, PAPI or VASI systems can be used.

During Autoland aproach, you should see always „On Glide Path“ signals. Just in case of strong wind there can be some deviation.

AUTOLAND and ILS (Instrumential Landing System)

ILS-Signal contains a horizontal signal (Glidepath) and a vertikal signal (Localizer). Both signals create a kind of a 35 deg cone which narrows closer to the runway. The cone is approx. 18nm long and 10nm wide.

If an aircraft enters tjis cone (intercept), it will be captured by the glidepath and the localizer (APR Approach-Mode) and follows this cone until touchdown. The Glidepath is 2,5-3,5° average and the localizer is usualy 20-25nm (depends on airport and surrounding terrain).

Go Around! (Special Function: TO/GA)

Sometimes, depending on the airport, the aircraft fails to reach the runway. If you notice problems during the final aproach, you have to Go-Around at once!

You have two options to start a goaround maneuver.n! Es gibt zwei Optionen:

1

Click the TO/GA button on the lower left AP Quad (Autothrottle-Unit).

2

The Tu144 offers a unique XML-Gauge which activates TO/GA function as soon as the Brake/Fire Button on the Joystick is pressed. (see picture).

Very helpful if you dont have the time to use the mouse. TOGA

also raises the gear and activates the afterburner.

TOGA only works when the aircraft is in the air. As soon as the aircraft is on ground, the joystickbutton is set back to its usual brake function.

Aproach-Precondition:

- The difference between localizer-course and aircraft-couzrse should not exceed 30deg.That should prevent a „overshooting“ which would be fatal on airports with two paralel runways. For example: Runway 26 = OBS corse 260deg – Intercept course MUST be between 230deg and 290deg.

- To „capture“ the glidepath, it is necessary to enter the glidepath from beneath. Means that the glidepath-needle is above the center.

- Check the Aproach chart of the airport to ensure the correct „capture-altitude“. If you dont have charts, you can try an altitude of 3000ft at a distance of 10nm to the runway.

Original XML programming by Nick Pike, 2001/2003 –

XML Code modified for Tu144 delta behaviours by Claudio Mussner, 2007

Text by Martyn Becker, November 2005 –

Text modified for Tu144 by Claudio Mussner + Thomas Ruth, 2007