# Tu-144  Sonic boom

When an aircraft breaks the sound barrier, an unusual cloud sometimes forms in its wake.

A drop in pressure, due to shock wave formation, causes a sharp drop in temperature,  leading

water vapor to condense into droplets and form the cloud.

Cause of sonic booms

As an object moves through the air it creates a series of pressure waves in front and behind it,

similar to the bow and stern waves created by a boat. These waves travel at the speed of sound,

and as the speed of the aircraft increases the waves are forced together because they cannot

"get out of the way" of each other, eventually merging into a single shock wave at the speed of

sound. This critical speed is known as Mach 1 and is approximately 1190 km/h (740 mph) at sea

level.

In smooth flight, the shock wave starts at the nose of the aircraft and ends at the tail. There is a

sudden rise in pressure at the nose, decreasing steadily to a negative pressure at the tail, where

it suddenly returns to normal. This "overpressure profile" is known as the N-wave due to its shape.

We experience the "boom" when there is a sudden rise in pressure, so the N-wave causes two

booms, one when the initial pressure rise from the nose hits, and another when the tail passes

and the pressure suddenly returns to normal.

This leads to a distinctive "double boom" from supersonic aircraft. When maneuvering the pressure distribution changes into different forms, with a characteristic U-wave shape.

Since the boom is being generated continually as long as the aircraft is supersonic, it traces out a

path on the ground following the aircraft's flight path, known as the boom carpet.

Characteristics

The "length" of the boom from front to back is dependent on the length of the aircraft, although to

a factor of 3:2 not 1:1. Longer aircraft therefore "spread out" their booms more than smaller ones,

which leads to a less powerful boom.

The nose shockwave compresses and pulls the air along with the aircraft so that the aircraft behind

its shockwave sees subsonic airflow.

However, this means that several smaller shock waves can, and usually do, form at other points

on the aircraft, primarily any convex points or curves; and especially the inlet to engines.

These secondary shockwaves are caused by the subsonic air behind the main shockwave being

forced to go supersonic again by the shape of the aircraft (for example due to the air's acceleration

over the top of a curved wing).

These shock waves add to the main shockwave at some distance away from the aircraft to create

a much more defined N-wave shape. This maximizes both the magnitude and the "rise time" of

the shock, which makes it seem louder.

On most designs the characteristic distance is about 40,000ft, meaning that below this altitude the

sonic boom will be "softer".

However the drag at this altitude or below makes supersonic travel particularly inefficient, which

poses a serious problem.

Abatement

In the late 1950s when SST designs were being actively pursued it was thought that although the

boom would be very large, they could avoid problems by flying higher. This premise was proven

false when the North American B-70 Valkyrie started flying and it was found that the boom was a

very real problem even at 70,000ft (21,000m).

It was during these tests that the N-wave was first characterized. Richard Seebass and his friend

Albert George at Cornell University studied the problem extensively, and eventually defined a

"figure of merit", FM, to characterize the sonic boom levels of different aircraft. FM is proportional

to the aircraft weight divided by the three-halves of the aircraft length, FM = W/(3/2·L) = 2W/3L.

The lower this value, the less boom the aircraft generates, with figures of about 1 or lower being

considered acceptable. Using this calculation they found FM's of about 1.4 for the Concorde, and

1.6 for the Tu-144. This eventually doomed most SST projects as public resentment, some-

what blown out of proportion, mixed with politics eventually resulted in laws that made any such

aircraft impractical (flying only over water for instance).

Ideally this would raise the characteristic altitude from 40,000ft to 60,000, which is where most

SST designs fly.

The design required some fairly sophisticated shaping in order to achieve the dual needs of re-

ducing the shock and still leaving an aerodynamically efficient shape, and therefore had to wait

for the advent of computer-aided design before being able to be built.

Hearing sonic booms

A sonic boom is the audible component of a shock wave in air. The term is commonly used to

refer to the air shocks caused by the supersonic flight of military aircraft or passenger transports

such as the Concorde (Mach 2.2) Tu-144  (Mach 2.4) and the Space Shuttle (Mach 27).

Sonic booms generate enormous amounts of sound energy, sounding much like an explosion,

typically the shock front may approach 100 megawatts per square meter, and may exceed 200

decibels.

The sound of a sonic boom depends largely on the distance between the observer and the aircraft

producing the sonic boom. A sonic boom is usually heard as a deep double "boom" as the aircraft

is usually some distance away. However, as those who have witnessed landings of space shuttles

have heard, when the aircraft is nearby the sonic boom is a sharper "bang" or "crack".

The sound is much like the "aerial bombs" used at firework displays.

The power, or volume, of the shock wave is dependent on the amount of air that is being sped up,

and thus the size and weight of the aircraft. As the aircraft increases speed the shocks grow "tighter"

around the craft, and do not become much "louder". At very high speeds and altitudes the cone

does not intersect the ground, and no boom will be heard.