Tu-144  Engineer Panel

Electrical System (fs9 aircraft)

The Tu-144 is supplied with main AC power at 115 volts and 400 Hz, secondary AC power at 36 volts

and 400 Hz, and DC power at 27 volts. Each engine is connected to its respective Integrated Drive Generator (IDG).

Each of the four AC generators is rated at 120 kV-A and provides independent AC power to its

respective bus. There is no parallel generator operation under normal circumstances. Each IDG is managed by a Generator Control Unit to maintain quality of the power supply.

Additionally, there are left and right Electrical Generator Logic Units for power control.

Most systems can be powered from more than one bus, and one generator can provide all of the electrical power

requirements except for the canard and inlet anti-ice.

The many fuel tank boost pumps are the main electrical power consumers. The high capacity fuel

transfer pumps are hydraulically driven and controlled with DC power.

Other important electrically driven systems are the canard and the retractable nose.

36 volt AC power is provided by two main and one back-up transformer.

This power is used for the aircraft's flight instruments, and the total draw is typically on the order of 1 kW out of a normal 200 kV-A main AC load.

The DC system consists of three transformer/rectifiers and three batteries. The normal DC load is 12 kW.

APU   Auxiliary Power Unit  (fs9 aircraft)

The APU is started from battery power, and DC power is used for communication units, relays,

and signaling devices. An Essential Bus is supplied by the 3 aircraft's batteries and provides

power to an inverter for driving essential flight instruments.

The APU generator rated at 60 kV-A at 400 HZ and provisions for external AC power.  In an

emergency the APU may also be used to supply 115 volt, 400 Hz AC power, but can only be

operated below 5 km altitude and cannot be started above 3 km altitude.

Controls (for a detailed description please look under “Quickstart Manual”):

1	APU Main switch 	(Sets APU to operational mode, Lamp 6 )

2	APU Door close/open	(After activated, Lamp 7 starts to blink until Exit is open.)

3	APU Fuel pump switch	(When operational preasure is reached, Lamp 8 activates)

4	BleedAir switch		(open/closes bleedair-vent)

5 	Control Lamps		(shows different conditions and problems)

6	Start APU		(Starts APU Turbine)

7	Stop APU		(Stops APU Turbine)

8	N1/EGT Instrument	(Shows N1 RPM and Exaust Gas Temperature)

9	Controllspeaker		(Activates control speaker.)

Control Lamps:

1 	Low oil level

2	High oil preasure

3 	Turbine temp reaches limit

4	Turbine RPM reaches limit

5	TCA6 failure

6	APU main power on

7	APU door open

8	APU fuel reasure nominal

9	APU ready

10	Bleedair ready

TakeOff Fuel-Settings (fs9 aircraft)

Important to keep the correct COG.

Left main 	(left wing tank): 		100 %

Right main 	(right wing tank):	100 %

Center 1 	(front tank): 		100 %

Center 2 	(aux aft tank): 	  	  10 %

Fuel Transfer System (fs9 aircraft)

The Instruments for the fuel levels of the left and right wing tanks are the top-mid and top-right gauges.

The two gauges (right-mid) are the fuel level instruments of the forward and rear ballance tanks.

The turning switch for the ballance control has four settings:

AUTO: fuel transfer is controled by the onboard computer.

NB: fuel transfer manual to the front ballance tank.

NT: fuel transfer manual to the rear ballance tank.

0: fuel transfer off

The fourth gauge (bottom-right) shows the overall fuel quantity.

A computer within the COG indicator system

continuously calculates and displays the CG

location.

The hydraulic fuel transfer pumps,  automatically or manually operated by the flight engineer,

provide fuel balancing using the following transfer routings:

to move the CG aft for

1 climb to Supersonic Cruise

fuel can be pumped from tanks

1 and 2  to tanks 4, 6, or 8

2 trim during Supersonic Cruise

fuel can be pumped from tank

2 to tank 8

to move the CG forward for

3 emergency deceleration

fuel can be pumped from tank

8 to tank 1 and 2

4 descend

fuel can be pumped from tank

8 to tanks 1, 2, 4 or 6.

Fuel Tank System (fs9 aircraft)

This Tu144 uses, same like the default FS2002 Concorde, special „airframe-files“ which are part of the sim.dll. This Files support only up to four Tanks

1   Center1  	(Front ballance)

2   LeftMain  	(Left main)

3   RightMain 	(Right main)

4   Center2  	(Rear ballance)

These four Tanks are enough for the Simulation because its ony important to simulate the movement of the COG during supesonic flight and to compensate it by pumping fuel from one tank to another.

Tank 1 / 4 crossfeed:

Can be selected automatic /manual.

Tank 2 / 3 selection:

Automatic-Modus (Auto Fuel Equalization Mode) is availiable which switches between Tank 2 and Tank 3 as needed.

X-feed between the two wing-tanks is not supported by FS9.

Tank 1, 2, 3, 4:

Manual Tank selection. Overrides automatic selection

Fuel Tank System (real aircraft)

The fuel system is comprised of 8 fuel storage

areas composed of 17 separate tanks containing

a total operating capacity of 95,000 kg.

The nomenclature refers to fuel tanks 1 through

8, but only tanks 6, 7 and 8 are single units.

Tanks 1, 2, and 8 are balance tanks used to

maintain the proper center of gravity location

through high capacity fuel transfer pumps.

These transfer pumps are hydraulically driven

and controlled by DC power. Fuel boost pumps

located in each tank are powered by the main

AC electrical systems.

Tank system 4 consists of 6 total tanks, four of

which provide fuel directly to the engines.

A crossfeed capability exists to control lateral

balancing.

Emergency fuel dumping can be accomplished

from all fuel tanks. All fuel system information is

displayed on the flight engineer panel, and all

fuel system controls are accessible only to the

flight engineer.

Numerous fuel quantity probes are used to pro-

vide individual tank system quantity indications

and to provide inputs to the CG indicator on the

flight engineer panel.

Quickstart Manual

| Battery / External Power |  |  |  |  |
| --- | --- | --- | --- | --- |
| Open Flight Engineer Panel |  |  |  |  |
| 1. | Battery Master Switch | ВНИМАНИЕ! СЕТЬ ОТ АККУМ | switch #4 | ON (UP) |
|  | - - Battery light |  |  | VERIFY ON |
|  | - - Voltage indicator |  |  | CHECK 24 V |
| If available, external power may be used to preserve battery power: |  |  |  |  |
| 2. | 200V APU/OFF/EXT PWR | ВСУ-ВЫКЛ-РАП | switch #1 | EXTERNAL (DOWN) |
|  | - - Voltage indicator |  |  | CHECK 24 V |
| ## APU Start |  |  |  |  |
| 3. | APU Warning panel test button | КОНТРОЛЬ ЛАМП | button #30 | PRESS |
|  | - - APU warning lights |  |  | VERIFY ALL ON |
| 4. | APU Master switch | ЗАПУСК | switch #5 | ON (UP) |
|  | - - Green warning light (APU door open) | ЗАБОРНИК ОТКРЫТ |  | VERIFY ON |
| 5. | APU Start/Rotate switch | ЗАПУСК/ХОЛ.ПРОКР | switch #6 | ON (UP) |
|  | - - Green warning light (APU fuel pressure) | Р ТОПЛИВА |  | VERIFY ON |
| 6. | APU Fuel Shutoff Valve | ПЕРЕКРЫВН. КРАН ВСУ | switch #7 | OPEN (UP) |
|  | - - Green warning light (APU ready to start) | ГОТОВ К ЗАПУСКУ |  | VERIFY ON |
| 7. | APU Start button | ПУСК | switch #8 | PRESS |
|  | - - Verify APU RPM gauge |  | APU RPM | min. 90% RPM |
|  | - - Green warning light | ГОТОВ К ЗАПУСКУ |  | VERIFY OFF |
| 8. | APU Bleed Air switch | ОТБОР ВОЗДУХА | switch #10 | ON (UP) & HOLD 8 sec |
|  | - - Green warning light | ВЫХОД НА РЕЖИМ |  | VERIFY ON |
| 9. | 200V APU/OFF/EXT PWR | ВСУ-ВЫКЛ-РАП | switch #1 | APU (UP) |
|  | - - Battery light | ВНИМАНИЕ! СЕТЬ ОТ АККУМ |  | VERIFY OFF |
| 10. | Avionics Master Power switches |  | switches #2+3 | ON (UP) |
| ## Before Engine Start |  |  |  |  |
| 11. | Engine Warning panel test button | КОНТРОЛЬ ЛАМП | button #31 | PRESS |
|  | - - Engine warning lights |  |  | VERIFY ALL ON |
| 12. | Fire Shutoff Valves | ПЕРЕКРЫВНЫЕ КРАНЫ | switches #16 | OPEN (4 UP) |
| 13. | Fuel gauges power switch | ТОПЛИВОМЕР | switch #17 | ON (UP) |
| 14 | Left Wing Tanks Fuel Valves |  | switch #12 | OPEN (UP) |
| 15. | Right Wing Tanks Fuel Valves |  | switch #13 | OPEN (UP) |
| 16. | Center Tank Fuel Valves |  | switch #14 | OPEN (UP) |
| 17. | Auto Fuel Equalization Mode | АВТОМАТ ВЫРАВНИВАНИЯ | switch # 18 | ON (UP) |
| 18. | Auto Fuel Supply System | АВТ. РАСХ | switch #19 | ON (UP) |
| 19. | Auto Fuel Supply System | АВТОМАТ/РУЧНОЕ | switch #20 | АUTO (UP) |
| 20. | Fuel Transfer Pumps knob |  | switch #28 | AUTO |
| Engine Start |  |  |  |  |
| Open Overhead Panel |  |  |  |  |
| 21. | Beacon switch |  |  | ON |
| 22. | Seatbelts, No Smoking, Exit light switches |  |  | ALL ON |
| Open Flight Engineer Panel |  |  |  |  |
| 23. | Fuel boost pump switches | РАСХОДНЫЙ БАК 1 | switches #15 | ON (4 UP) |
| 24. | Engine Start Master switch | ЗАПУСК | switch #21 | START |
| 25. | Engine Igniters switch | ЗАПУСК | switch #22 | START |
| 26. | Fuel heaters power switch | ОБОГРЕВ ЗАПАЛЬНОГО УСТРОЙСТВА | switch #23 | ON (UP) IF OAT < +5ºC |
|  | Note: Engines will shut down when the throttles are moved to idle if the outside air temperature is lower than 5ºC. To avoid accidental shutdown, you may have the fuel heaters turned ON. |  |  |  |
| 27. | Engine start selector knob - - Starting order (#2,#3,#4,#1) | 1 ДВИГ, 2 ДВИГ, 3 ДВИГ, 4 ДВИГ | switch #24 pos. 1-4 | SELECT ENG. TO START (2,3,4,1) |
| 28. | Engine starter button |  | switch #27 | ENGAGE |
| 29. | HP Fuel Valves - - Monitor engine RPM and EGT to verify startup |  | levers #11 | OPEN (4 UP) after RPM>20% |
| 30. | Engine Generator switches |  | switches #29 | ON (UP) |
|  | Repeat starting procedure steps 27-30 for remaining engines. |  |  |  |
| After Engine Start |  |  |  |  |
| 31. | Engine start selector knob |  | switch #24 | NEUTRAL (0) |
| 32. | 200V APU/OFF/EXT PWR | ВСУ-ВЫКЛ-РАП | switch #1 | OFF (MIDDLE) |
| 33. | APU Bleed Air switch | ОТБОР ВОЗДУХА | switch #10 | OFF (DOWN) 8 sec |
| 34. | APU Stop button | СТОП |  | PRESS |
| 35. | APU Fuel Shutoff Valve switch |  | switch #7 | CLOSE (DOWN) |
| 36. | APU Start/Rotate switch | ЗАПУСК/ХОЛ.ПРОКР | switch #6 | OFF (DOWN) |
| 37. | APU Master switch - - Monitor engine RPM | ЗАПУСК | switch #5 | OFF (DOWN) after RPM 0% |
| 38. | Center of Gravity (C.G.) Indicator |  | CG | VERIFY C.G. |
|  | IMPORTANT: Do NOT begin taxi unless C.G. is within limits!!! Advice, fill your tanks with: 100% in the wing tanks, 100% in the front tank and 50% in the rear tank. |  |  |  |
| Switch to Main & Overhead Panels |  |  |  |  |
| 39. | Instrument power switches (Attitude + Backup Attitude Indicator , Radio Altimeter,) |  |  | ON (4) |
|  | Refer to attached documentation bmp files for location of these switches |  |  |  |
| 40. | Flight Director/Localizer/ Glideslope indicators |  |  | ON (3) |
|  | These 3 toggle switches are located on the autopilot panel, below the autopilot master on/off switches. |  |  |  |
| 41. | Navigation/Panel lights |  |  | AS REQUIRED |
| In-flight Engine Start |  |  |  |  |
| In the event an engine needs to be restarted in-flight, use the following procedure. Note that in this case the APU is not required. |  |  |  |  |
| 42. | Engine Start Master switch | ЗАПУСК | switch #21 | START |
| 43. | Engine Igniters switch | ЗАПУСК | switch #22 | START |
| 44. | Fuel heaters power switch | ОБОГРЕВ ЗАПАЛЬНОГО УСТРОЙСТВА | switch #23 | ON (UP) |
| 45. | Engine air start button | ЗАПУСК В ВОЗДУХЕ 1 ДВ, 2 ДВ, 3 ДВ, 4 ДВ | buttons #27 (eng. 1-4) | ENGAGE |
| 46. | HP Fuel Valve - - Monitor engine RPM and EGT to verify startup |  | levers #11 | OPEN after RPM>20% |
| 47. | Engine Generator switch |  | switches #26 | ON (UP) |