Airbus A320 MCDU Data Insertion

This flight plan will use an airway which radiates from a waypoint close to Gatwick Airport UK called BIG and can be used to fly all the way to BEGAR close to Zurich.  So the route is EGKK (Rwy any) [BIG, L15 BEGAR], LSZH Rwy ILS16 Via AMIKI and takes approximately 2min to install.

To enter the flight plan into the MCDU first open the INIT Page 1.  Anything that

is red will need data inserted, so begin by inserting the Departing Airport and the Destination Airport, EGKK/LSZH.                     LSK = Line Select Key.

| PAGE | DATA | Insert | MCDU Button |
| --- | --- | --- | --- |
| INIT 1 | From / To | EGKK/LSZH | LSK 1R Enter |
| Type [EGKK/LSZH] on the MCDU Keypad and Press LSK Next To From / To |  |  |  |

Page [EGKK/LSZH] Press LSK Next To [<RETURN] for Page [INIT] ‎‎

Page [INIT] Open [F-PLAN] page. Press LSK Next To [FROM EGKK]

Page [LAT REV FROM EGKK] Press LSK Next To [<DEPARTURE]

Page [DEPARTURES FROM EGKK] Press LSK Next To [26L]

LAV REV = Lateral Revision

‎

Page [DEPARTURES FROM EGKK] Press LSK Next To [INSERT]

Page [FROM EGKK26L] Press LSK Next To [FROM EGKK26L]

Page [LAT REV FROM EGKK] Type BIG and Press LSK Next To [NEXT WPT]

Page [SELECT DESIRED WPT 1/1] Press LSK Next To the TOP CO-ORDINATE

Depending on the spec of your PC it may take several seconds to generate the co-ordinates of BIG.

Please be patient.

When the MCDU is ready choose the top co-ordinate and if it installs below the DECEL point wait until it shuffles above where it should be.

Page [FROM EGKK26L] Press LSK Next To [TMPY INSERT] Press LSK Next To [BIG]

Page [LAT REV FROM BIG] Press LSK Next To [AIRWAYS]

Page [AIRWAYS FROM BIG] Type in Keypad L15 and Press LSK Next To [VIA]

Page [AIRWAYS FROM BIG] Type in Keypad [BEGAR] and Press LSK Next To [TO]

Page [AIRWAYS FROM BIG] Press LSK Next To [< RETURN]

Page [FROM EGKK26L] = FLIGHT PLAN Press LSK Next To [DEST LSZH]

Page [LAT REV FROM LSZH] Almost done. Press LSK Next To [<ARRIVAL]

Page [ARRIVAL TO LSZH] Press LSK Next To [ILS 16]

Page [ARRIVAL TO LSZH] Press LSK Next To [TMPY INSERT]

Before closing go to INIT page 2 and enter the BLOCK FUEL 7.5

Back to Flight Plan page and move on.

COMPLETED FLT-PLAN