AERO COMMANDER 500 PROCEDURES

For suggested speeds, see the Reference page of the Kneeboard.

Note that most actions can also be performed using the mouse.

NOTE: This aircraft's real-world checklists have been modified for use with Flight Simulator.

| To... | Press... |
| --- | --- |
| Display/hide Overhead | SHIFT+2 |
| Display/hide GPS | SHIFT+3 |
| Display/hide engine Zoom Panel | SHIFT+4 |
| Display/hide Radio | SHIFT+5 |

| BEFORE STARTING |  |
| --- | --- |
| [ ] Parking Brake | SET (press CTRL+PERIOD key) |
| [ ] All Avionics | OFF |
| [ ] Landing Gear Handle | DOWN (press G to toggle) |
| [ ] Fuel Selectors | CHECK ON (press SHIFT+4 to display throttle quadrant) |
| [ ] All Switches and Equipment Controls | CHECK |
| [ ] Battery and Alternator Switches | ON |
| [ ] Fuel Quantity Indicators | CHECK QUANTITY |
| [ ] Landing Gear Position Lights | CHECK |
| STARTING |  |
| Press CTRL+E to initiate engine autostart sequence, or: |  |  |
| LEFT ENGINE: |  |
| (press E, then 1 to move focus to left engine) |  |  |
| [ ] Mixture Control | FULL RICH (press CTRL+SHIFT+F3 until fully forward) |
| [ ] Propeller Control | HIGH RPM (press CTRL+F3 until fully forward) |
| [ ] Throttle | FULL OPEN (press F3 until fully forward) |
| [ ] Fuel Boost Pump Switch | HI (until fuel flow peaks, then OFF) |
| [ ] Throttle | CLOSE; THEN OPEN APPROX. 1/2 INCH (press F3 until fully in, then press F2 as necessary) |
| [ ] Magneto/Start Switch | START position; release to BOTH position when engine starts (press M+PLUS key to R, L, Both, Start) |
| [ ] Throttle | 900 to 1000 RPM after start |
| [ ] Oil Pressure | 10 PSI WITHIN 30 SECONDS |
| [ ] Warm-up | 900 to 1000 RPM |
| [ ] Alternator Switch | ON |
| [ ] Loadmeters and Voltmeter | CHECK FOR BATTERY CHARGE |
| [ ] Red "START" Annunciator Light | CHECK (should be illuminated during start and extinguished after start) |
| RIGHT ENGINE: |  |
| (press E, then 2 to move focus to left engine) |  |  |
| [ ] Mixture Control | FULL RICH (press CTRL+SHIFT+F3 until fully forward) |
| [ ] Propeller Control | HIGH RPM (press CTRL+F3 until fully forward) |
| [ ] Throttle | FULL OPEN (press F3 until fully forward) |
| [ ] Fuel Boost Pump Switch | HI (until fuel flow peaks, then OFF) |
| [ ] Throttle | CLOSE; THEN OPEN APPROX. 1/2 INCH (press F3 until fully in, then press F2 as necessary) |
| [ ] Magneto/Start Switch | START position; release to BOTH position when engine starts (press M+PLUS key to R, L, Both, Start) |
| [ ] Throttle | 900 to 1000 RPM after start |
| [ ] Oil Pressure | 10 PSI WITHIN 30 SECONDS |
| [ ] Warm-up | 900 to 1000 RPM |
| [ ] Alternator Switch | ON |
| [ ] Loadmeters and Voltmeter | CHECK FOR BATTERY CHARGE |
| [ ] Red "START" Annunciator Light | CHECK (should be illuminated during start and extinguished after start) |
| (press E, then 1, then 2 to move focus to both engines) |  |  |
| AFTER STARTING AND TAXI |  |
| [ ] Avionics | ON, AS REQUIRED (press SHIFT+2 to display radio stack) |
| [ ] Exterior Lights | AS REQUIRED |
| [ ] Brakes | RELEASE AND CHECK (press PERIOD key) |
| BEFORE TAKEOFF |  |
| [ ] Parking Brake | SET (press CTRL+PERIOD key) |
| [ ] Fuel Boost Pumps | OFF (if ambient temperature is 32 degrees C or above, use LOW pressure boost) |
| [ ] All Instruments | CHECKED |
| [ ] Fuel Indicators | CHECK QUANTITY INDICATED |
| [ ] Mixture | ADJUST AS REQUIRED BY FIELD ELEVATION WHEN SETTING FULL POWER FOR TAKEOFF |
| [ ] Fuel Selectors | CHECK ON |
| [ ] Red "START" Annunciator Light | CHECK (should be illuminated during start and extinguished after start) |
| [ ] Throttles | 2200 RPM (press F2 and F3 as necessary) |
| [ ] Propellers | EXERCISE (to obtain 200-300 RPM drop, then return to high RPM) (press CTRL+F2 and CTRL+F3 as necessary) |
| [ ] Throttles | 1700 RPM (press F2 and F3 as necessary) |
| [ ] Magnetos | CHECK <150 RPM drop on each; <50 RPM drop between (Press M+MINUS SIGN and M+PLUS SIGN to L, R, L, Both) |
| [ ] Throttles | 1500 RPM (press F2 and F3 as necessary) |
| [ ] Propellers | FEATHERING CHECK (Do not allow an RPM drop of more than 300 RPM) |
| [ ] Throttles | IDLE (note RPM) (press F2 as necessary) |
| [ ] Throttles | 900 to 1000 RPM (press F2 and F3 as necessary) |
| [ ] Trim | AS REQUIRED FOR TAKEOFF (with Num Lock off, press Num Pad 1 and Num Pad 7 as necessary) |
| [ ] Flaps | CHECK AND SET FOR TAKEOFF (press F6 and F7 as necessary) |
| [ ] Flight Controls | CHECK PROPER DIRECTION AND FREEDOM OF MOVEMENT |
| [ ] Ice Protection Systems | AS REQUIRED |
| [ ] Parking Brake | OFF (press PERIOD key) |
| TAKEOFF |  |
| [ ] Minimum Takeoff Oil Temperature | 24 degrees C |
| [ ] Throttles | FULL (press F3 until fully forward) |
| [ ] Propellers | MAX RPM (press CTRL+F3 until fully forward) |
| [ ] Mixture | FULL RICH (press CTRL+SHIFT+F3 until fully forward) |
| [ ] Airspeed | ACCELERATE TO AND MAINTAIN RECOMMENDED SPEEDS (see Reference tab) |
| [ ] Landing Gear | RETRACT (when positive rate of climb is established) (press G) |
| [ ] Airspeed | ESTABLISH DESIRED CLIMB SPEED (when clear of obstacles) |
| CLIMB |  |
| [ ] Throttles | FULL (press F3 until fully forward) |
| [ ] Propellers | 2500 RPM (press CTRL+F3 and CTRL+F2 as necessary) |
| [ ] Mixture | LEAN AS REQUIRED (press CTRL+SHIFT+F2 and CTRL+SHIFT+F3 as necessary) |
| [ ] Airspeed | 110 MPH |
| CRUISE CLIMB |  |
| [ ] Throttles | FULL (press F3 until fully forward) |
| [ ] Propellers | 2400 RPM (press CTRL+F3 and CTRL+F2 as necessary) |
| [ ] Mixture | LEAN AS REQUIRED (press CTRL+SHIFT+F2 and CTRL+SHIFT+F3 as necessary) |
| [ ] Airspeed | 150 MPH |
| [ ] Engine Temperatures | MONITOR |
| [ ] Fuel Boost Pumps | AS REQUIRED |
| CRUISE |  |
| [ ] Power | SET (press F2 and F3 as necessary) |
| [ ] Fuel Boost Pumps | AS REQUIRED |
| [ ] Mixtures | SET USING EGT |
| DESCENT |  |
| [ ] Altimeter | SET (press B to autoset to current setting) |
| [ ] Mixture | ENRICH AS REQUIRED (press CTRL+SHIFT+F3 and CTRL+SHIFT+F2 as necessary) |
| [ ] Flaps | AS REQUIRED (press F6 and F7 as necessary) |
| [ ] Power | AS REQUIRED (avoid prolonged idle settings) (press F2 and F3 as necessary) |
| BEFORE LANDING |  |
| [ ] Fuel Selector Valves | CHECK ON |
| [ ] Fuel Boost Pumps | OFF OR LOW AS PER AMBIENT TEMPERATURE |
| [ ] Mixture Controls | AS REQUIRED BY FIELD ELEVATION |
| [ ] Flaps | APPROACH (10 degrees; when below 180 MPH) (press F6 and F7 as necessary) |
| [ ] Landing Gear | DOWN (below 180 MPH) |
| [ ] Flaps | 20 degrees at 150 30 degrees at 135 FULL (40 degrees; when below 125 MPH) |
| [ ] Airspeed | ESTABLISH NORMAL APPROACH SPEED (approx. 110 MPH) |
| [ ] Propellers | HIGH RPM (press CTRL+F3 until fully forward) |
| AFTER LANDING (CLEAR OF RUNWAY) |  |
| [ ] Landing and Taxi Lights | AS REQUIRED |
| [ ] Flaps | UP (press F6 as necessary) |
| [ ] Trim Tabs | SET TO ZERO (with Num Lock off, press Num Pad 1 and Num Pad 7 as necessary) |
| [ ] Fuel Boost Pumps | AS REQUIRED |
| SHUTDOWN |  |
| [ ] Parking Brake | SET (press CTRL+PERIOD key) |
| [ ] Propellers | HIGH RPM (press CTRL+F3 until fully forward) |
| [ ] Throttles | 1000 RPM (press F2 and F3 as necessary) |
| [ ] Fuel Boost Pumps | OFF |
| [ ] Electrical and Avionics Equipment | OFF |
| [ ] Mixture Controls | IDLE CUT-OFF (press CTRL+SHIFT+F2 until fully back) |
| [ ] Magneto/Start Switches | OFF AFTER ENGINES STOP (press M+MINUS key to L, R, OFF) |
| [ ] Battery and Alternator Switches | OFF AFTER ENGINES STOP |