Model 500

Type Certificate:	6A1

Certificated:	July 24th 1958

PCLM:	7  (2 at +94; 2 at +128; 3 at +168)

Number Built:	101

Certificated by:	Aero Design & Engineering Company

Serial numbers 618 through 656 	(16 aircraft)

Aero Design & Engineering Company (subsidiary of Rockwell-Standard Corp)

Serial numbers 657 through 852 	(85 aircraft)

Engines:	Lycoming O-540-A1A5

Serial number 618	(1 aircraft)

Lycoming O-540-A2B

Serial numbers 635 through 852	(100 aircraft)

Propellers (hub/blade): 	Hartzell HC-82XK-2/8433-4

Serial number 618	(1 aircraft)

Hartzell HC-82XK-2B/8433-4

Serial numbers 635 through 740 	(54 aircraft)

Hartzell HC-82XK-2B1/8433-4

Serial numbers 742, 838 & 846	(3 aircraft)

Hartzell HC-82XK-2C1/8433-4

Serial numbers 748 through 836, 840, 842, 848 & 852 	(43 aircraft)

Fuel:	91/96 minimum grade aviation gasoline

Fuel consumption:	11.9 U.S. gallons per engine per hour at 2,300r.p.m.

Fuel capacity:	158.5 U.S. gallons (600 litres) total	[5 inter-connected tanks]

156.0 U.S. gallons (591 litres) usable

or	225.5 U.S. gallons (854 litres) total	(with optional 67 U.S. gallons auxiliary fuel)

223.0 U.S. gallons (844 litres) usable

Oil capacity:	6.000 U.S. gallons (22.71 litres) total (3 gallons each engine)

4.625 U.S. gallons (17.51 litres) usable

| Weights & Balance: |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | lb | kg |  |  |
| Ramp Weight |  |  |  |  |
| MTOW | 6,000 | 2,722 |  |  |
|  | 6,500 | 2,948 | per STC SA377SO |  |
| MLW |  |  |  |  |
| ZFW |  |  |  |  |
| Empty Weight (typical) | 3,850 | 1,746 | (Factory Empty Weight, with standard equipment) |  |
| Useful load | 2,150 | 975 |  |  |
| Basic Operating Weight |  |  |  |  |
| Payload |  |  |  |  |
| Maximum baggage | 350 | 159 |  |  |
| Maximum Wing Loading | 23.53 lb/sq.ft. | 114.88 kg/m2 | 6,000lb. / 255 sq.ft. |  |
|  | 25.49 lb/sq.ft. | 124.45 kg/m2 | 6,500lb. / 255 sq.ft. |  |
| Maximum Power Loading | 12.00 lb/h.p. | 5.44 kg/h.p. | 7.30 kg/kW | 6,000lb. / 500 h.p. |
|  | 11.21 lb/h.p. | 5.08 kg/h.p. | 6.82 kg/kW | 6,500lb. / 580 h.p. |
| C.G. range | (+166.0) to (+175.1) - gear extended |  |  |  |
| Effect of retracting gear | +6210 in./lb. |  |  |  |
| Empty Weight C.G. range | None |  |  |  |
| Datum | Located 152 inches (12' 8") forward of wing leading edge at center section. |  |  |  |
| Levelling means | Longitudinal | Top of fuselage centerline aft of wing trailing edge. |  |  |
|  | Lateral | Transverse beams at front or rear of baggage compartment floor. |  |  |

| Control Surface Movements: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | o up | o down | o right | o left | Remarks |
| Aileron | 23 2 | 15 +2 |  |  |  |
| Flaps |  | 40 +2 |  |  |  |
| Elevator | 30 +1/-0 | 10 +2/-0 |  |  |  |
| Elevator tab | 2.5 +2/-0.5 | 20 +2/-0 |  |  |  |
| Rudder |  |  | 20 +2/-0 | 20 +2/-0 |  |
| Rudder tab |  |  | 26 +2/-0 | 26 +2/-0 |  |

| Dimensions: |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Feet/Inches | Inches | Meters | Remarks |
| Wing span | 49' 0.56" | 588.56 | 14.949 | Wing section is NACA 23012 modified |
| Wing chord at root | 8’ 4.00” | 100.00 | 2.540 |  |
| Wing chord at tip | 2’ 1.42” | 25.42 | 0.646 |  |
| Wing - ground to wingtip height |  |  |  |  |
| Wing Aspect Ratio | 9.45 |  |  |  |
| Wing Taper Ratio | 3.93 |  |  |  |
| Wing dihedral | 4 degrees |  |  |  |
| Wing incidence at root | 3 degrees |  |  |  |
| Wing incidence at tip | minus 3.5 degrees |  |  |  |
| Wing - sweep forward at trailing edge | 14 degrees |  |  |  |
| Wing area - inc. Flaps & Ailerons | 255 sq.ft. |  | 23.690 m2 |  |
| Total Flap area | 21.20 sq.ft. |  | 1.970 m2 |  |
| Total Aileron area | 20.52 sq.ft. |  | 1.906 m2 |  |
| Fuselage Length | 35' 1.25" | 421.25 | 10.700 |  |
| Overall Length | 35’ 5.50” | 425.50 | 10.808 |  |
| Overall Height | 14' 6.00" | 174.00 | 4.420 |  |
| Height - ground to top of fuselage |  |  |  |  |
| Height - ground to bottom of fuselage | 0’ 5.50” | 5.50 | 0.140 |  |
| Total Overall Fin area | 39.40 sq.ft. |  | 3.660 m2 |  |
| Fin fixed surface area | 24.00 sq.ft. |  | 2.230 m2 |  |
| Rudder area | 15.40 sq.ft. |  | 1.430 m2 |  |
| Tailplane span | 16’ 8.95” | 200.95 | 5.104 |  |
| Ground to tailplane tip height |  |  |  |  |
| Tailplane dihedral | 10 degrees |  |  |  |
| Tailplane - angle of incidence | 0.75 degrees |  |  |  |
| Total Overall Tailplane area | 53.60 sq.ft. |  | 4.980 m2 |  |
| Tailplane fixed surface area | 33.06 sq.ft. |  | 3.072 m2 |  |
| Elevators area | 20.54 sq.ft. |  | 1.908 m2 |  |
| Wheel track (tread) | 12’ 11.00” | 155.00 | 3.937 |  |
| Wheelbase (nose to main) | 14’ 0.00” | 168.00 | 4.267 |  |
| Minimum ground turning radius |  |  |  |  |
| Cabin Length (Total) | 10’ 7.50” | 127.50 | 3.239 |  |
| Cabin Height | 4’ 5.00” | 53.00 | 1.346 |  |
| Cabin Width | 4’ 4.00” | 52.00 | 1.321 |  |
| Maximum external fuselage width |  |  |  |  |
| Cabin door Height | 3’ 7.00” | 43.00 | 1.092 |  |
| Cabin door Width | 2’ 4.00” | 28.00 | 0.711 |  |
| Cabin door, Height to sill | 1’ 6.50” | 18.50 | 0.470 |  |
| Cabin volume | 177 cu.ft. |  | 5.012 m3 |  |
| Baggage compartment Length | 2’ 7.00” | 31.00 | 0.787 |  |
| Baggage compartment Height | 3’ 5.00” | 41.00 | 1.041 |  |
| Baggage compartment Width | 3’ 11.00” | 47.00 | 1.194 |  |
| Baggage door Height | 1’ 11.50” | 23.50 | 0.597 |  |
| Baggage door Width | 1’ 7.50” | 19.50 | 0.495 |  |
| Baggage door, Height to sill |  |  |  |  |
| Baggage compartment volume | 32 cu.ft. |  | 0.906 m3 |  |

| Performance: |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Feet | Meters |  | Remarks |
| Takeoff distance, over 50ft obstacle | 1,400 | 427 |  |  |
| Takeoff run |  |  |  |  |
| Short-field takeoff, over 50 ft obstacle |  |  |  |  |
| Short-field takeoff run |  |  |  |  |
| Landing distance, over 50ft obstacle | 1,350 | 410 |  |  |
| Landing run |  |  |  |  |
| Short-field landing distance, over 50ft obstacle |  |  |  |  |
| Short-field landing run |  |  |  |  |
| Maximum Operating Altitude |  |  |  | FAA Operational Ceiling |
| Service ceiling | 21,375 | 6,515 |  | RCAH says 22,500ft |
| Single-engine ceiling | 8,150 | 2,485 |  | RCAH says 8,000ft |
|  | nm | sm | km |  |
| Range, maximum cruising power | 956 | 1,100 | 1,770 | with 30 minutes reserve |
| Range, economy cruising power |  |  |  |  |
| Range, maximum cruising power, max payload |  |  |  |  |
| Range, economy cruising power, max payload |  |  |  |  |
|  | mph | kts | km/hr |  |
| Manoeuvring | 160 | 139 | 257 | True Indicated |
| Max. structural cruising (Max. Operating Speed) (Vmo) | 210 | 182 | 338 | True Indicated |
| Never Exceed (Vne) | 270 | 235 | 435 | True Indicated |
| Flaps extended – Half (Vfe) | 150 | 130 | 241 | True Indicated |
| Flaps extended – Full | 130 | 113 | 209 | True Indicated |
| Landing gear extended | 180 | 156 | 290 | True Indicated |
| Maximum level speed (TAS) | 218 | 189 | 351 |  |
| Maximum cruising speed (TAS) - at 70% power | 205 | 178 | 330 | at 10,000ft / 3,048m |
| Economy cruising speed (TAS) - at 65% power | 199 | 173 | 320 | at 10,000ft / 3,048m |
| Takeoff speed | 87 | 76 | 140 |  |
| Landing speed | 82 | 71 | 132 |  |
| Best rate-of-climb speed (Vy) | 110 | 96 | 177 |  |
| Best Angle of Climb speed (Vx) |  |  |  |  |
| Minimum Control speed (Vmc) |  |  |  |  |
| Stall Speed (Clean) (Vsi) |  |  |  |  |
| Stall Speed (Dirty) (Vso) - power off | 63 | 55 | 101 |  |
|  | fpm | mpm |  |  |
| Initial Rate of Climb |  |  |  |  |
| Max. Rate of Climb at Sea Level | 1,400 | 427 |  |  |
| Max. Rate of Climb at Sea Level, single engine | 310 | 95 |  |  |

| Engines (1): |  |  |  |  |
| --- | --- | --- | --- | --- |
| Characteristics | Piston, air-cooled, with 6 horizontally Opposed cylinders |  |  |  |
| Manufacturer | Lycoming |  |  |  |
| Engine Type | O-540 |  |  |  |
| Engine Variant | -A1A | -A1A5 | -A2B |  |
| Type Certificate Number | E-295 |  |  |  |
| Type Certificate Date | October 31st 1957 | June 18th 1958 | July 24th 1958 |  |
| Applicable under STC No. | n/a |  |  |  |
| Maximum continuous HP | 250 |  |  |  |
| Maximum continuous RPM | 2,575 |  |  |  |
| Takeoff HP (5 minutes) | 250 |  |  |  |
| Takeoff RPM | 2,575 |  |  |  |
| Weight (lb.) | 374 |  |  |  |
| Reduction Gear Ratio | n/a |  |  |  |
| Supercharging Ratio | n/a |  |  |  |
| Bore & Stroke (inches) | 5.125 x 4.375 |  |  |  |
| Displacement (cu.in.) | 541.5 |  |  |  |
| Compression Ratio | 8.50 : 1 |  |  |  |
| Carburetor Make & Model | PAC MA-4-5 (formerly Marval Schebler/Facet) |  |  |  |
| Notes: | The O-540-A1A has two 6th order counterweights (dampers). |  |  |  |
|  | The O-540-A1A5 has one 5th and one 6th order counterweights (dampers). |  |  |  |
|  | The -A2B is an -A1A with short propeller governor studs and propeller locating bushing re-located 60 degrees counter-clockwise. |  |  |  |

| Engines (2): |  |  |  |  |
| --- | --- | --- | --- | --- |
| Characteristics | Piston, air-cooled, Fuel Injected, with 6 horizontally Opposed cylinders |  |  |  |
| Manufacturer | Lycoming |  | Continental |  |
| Engine Type | IO-540 |  | IO-470 |  |
| Engine Variant | -A1A5 | -B1A | -E |  |
| Type Certificate Number | 1E4 | n/a ? | 3E1 |  |
| Type Certificate Date | June 30th 1961 | n/a ? | November 26th 1958 |  |
| Applicable under STC No. | SA257CE | n/a | n/a |  |
| Maximum continuous HP | 290 |  | 260 |  |
| Maximum continuous RPM | 2,575 |  | 2,625 |  |
| Takeoff HP (5 minutes) | 290 |  | 260 |  |
| Takeoff RPM | 2,575 |  | 2,625 |  |
| Weight (lb.) | 414 | 411 | 429 |  |
| Reduction Gear Ratio | n/a |  |  |  |
| Supercharging Ratio | n/a |  |  |  |
| Bore & Stroke (inches) | 5.125 x 4.375 |  | 5.00 x 4.00 |  |
| Displacement (cu.in.) | 541.5 |  | 471 |  |
| Compression Ratio | 8.70 : 1 |  | 8.60 : 1 |  |
| Injector Make & Model | PAC RS10ED1 |  | TCM |  |
| Notes: | The IO-540-A1A5 has one 5th and one 6th order counterweights (dampers). |  |  |  |
|  | The IO-540-B1A is the same as the -A1A5 except for updraft exhaust cooling. |  |  |  |
|  | The IO-470-E has four 6th order dampers. |  |  |  |

| Propellers (1): |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Characteristics | Constant speed, hydraulic, with feathering and unfeathering features. |  |  |  |  |  |  |  |
| Manufacturer | Hartzell |  |  |  |  |  |  |  |
| Hub Model | HC-82XK |  |  |  |  |  | HC-82VK |  |
| Hub Variant | -2 | -2B |  |  | -2B1 | -2C1 | -2B | -2B1 |
| Type Certificate Number | P-878 |  |  |  |  |  | P23EA |  |
| Type Certificate Date | October 8th 1952 |  |  |  |  |  | March 4th 1966 |  |
| Hub Material | Alloy Steel |  |  |  |  |  |  |  |
| Blade Model | 8433-4 |  | 8433-6 | 8433-4 |  |  | V8433-4 |  |
| Blade Material | Aluminum Alloy |  |  |  |  |  |  |  |
| Number of Blades | 2 |  |  |  |  |  |  |  |
| Diameter (inches) | 80 |  | 78 | 80 |  |  |  |  |
| Diameter (meters) | 2.032 |  | 1.981 | 2.032 |  |  |  |  |
| Cut-off permitted | none |  |  |  |  |  |  |  |
| Prop tip ground clearance (inches) | 23.29 |  | 22.29 | 23.29 |  |  |  |  |
| Prop tip fuselage clearance (inches) | 12.00 |  | 11.00 | 12.00 |  |  |  |  |
| Distance between propeller centers | 153” |  |  |  |  |  |  |  |
| Pitch setting at 30" Station - Low | 14o |  |  |  |  |  |  |  |
| - High |  |  |  |  |  |  |  |  |
| - Feathered | 82o to 84o |  |  |  |  |  |  |  |
| Max. Hub & Blade weight (lb.) | 67 |  |  |  |  |  |  |  |
| Maximum continuous - HP | 260 |  |  |  |  |  |  |  |
| Maximum continuous - RPM | 2,625 |  |  |  |  |  |  |  |
| Maximum Takeoff - HP | 260 |  |  |  |  |  |  |  |
| Maximum Takeoff - RPM | 2,625 |  |  |  |  |  |  |  |

| Propellers (2): |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Characteristics | Constant speed, hydraulic, with feathering and unfeathering features. |  |  |  |  |  |  |  |
| Manufacturer | Hartzell |  |  |  |  |  |  |  |
| Hub Model | HC-A2XK |  | HC-A2VK |  |  | HC-A3VK |  |  |
| Hub Variant | -2C1 |  | -2B | -2C1 |  | -2B |  |  |
| Type Certificate Number | P-908 |  | P24EA |  |  | P6EA |  |  |
| Type Certificate Date | Oct 7th 1958 |  | Apr 1st 1966 |  |  | August 15th 1962 |  |  |
| Applicable under STC No. | n/a |  | n/a |  |  | SA1547WE / SA1735SO |  |  |
| Hub Material | Alloy Steel |  |  |  |  |  |  |  |
| Blade Model | 8433-4 |  | V8433-4 |  | V7636D-4 |  | V7636D |  |
| Blade Material | Aluminum Alloy |  |  |  |  |  |  |  |
| Number of Blades | 2 |  |  |  |  | 3 |  |  |
| Diameter (inches) | 80 |  |  |  |  | 72.625 |  | 76.625 |
| Diameter (meters) | 2.03 |  |  |  |  | 1.84 |  | 1.95 |
| Cut-off permitted | none |  |  |  |  |  |  |  |
| Prop tip ground clearance (inches) | 23.29 |  |  |  |  |  |  |  |
| Prop tip fuselage clearance (inches) | 12.00 |  |  |  |  |  |  |  |
| Distance between propeller centers | 153” |  |  |  |  |  |  |  |
| Pitch setting at 30" Station - Low | 14o |  |  |  |  |  |  |  |
| - High |  |  |  |  |  |  |  |  |
| - Feathered | 82o to 84o |  |  |  |  |  |  |  |
| Max. Hub & Blade weight (lb.) | 67 |  |  |  |  | 90 |  |  |
| Maximum continuous - HP | 260 |  |  |  |  | 336 |  |  |
| Maximum continuous - RPM | 2,625 |  |  |  |  | 2,850 |  |  |
| Maximum Takeoff - HP | 260 |  |  |  |  | 336 |  |  |
| Maximum Takeoff - RPM | 2,625 |  |  |  |  | 2,850 |  |  |

| Engine & Propeller combinations: |  |  |
| --- | --- | --- |
| Engines | Propellers (Hub / Blade) | Factory or other |
| Continental IO-470-E | ? | Factory (tests - serial 654 only) |
| Lycoming O-540-A1A | ? | Factory (tests - serial 654 only) |
| Lycoming O-540-A1A5 | Hartzell HC-82XK-2 / 8433-4 | Factory (tests - serial 618 only) |
| Lycoming O-540-A2B | Hartzell HC-82XK-2B / 8433-4 | Factory |
|  | Hartzell HC-82XK-2B1 / 8433-4 | Factory |
|  | Hartzell HC-82XK-2C1 / 8433-4 | Factory |
|  | Hartzell HC-A3VK-2B / V7636D | Field - STC SA1735SO |
|  | Hartzell HC-A3VK-2B / V7636D-4 | Field - Kensair STC SA1547WE |
| Lycoming O-540-A2B (assumed) | Hartzell HC-82VK-2B / V8433-4 | Field |
|  | Hartzell HC-82VK-2B1 / (unquoted) | Field - probably blade V8433-4 |
|  | Hartzell HC-82XK-2B / 8433-6 | Field |
|  | Hartzell HC-A2VK-2B / V8433-4 | Field |
|  | Hartzell HC-A2XK-2C1 / (unquoted) | Listed on TCDS - no trace of usage |
|  | Hartzell HC-A2VK-2C1 / V8433-4 | Listed on TCDS - no trace of usage |
| Lycoming IO-540-A1A5 | Hartzell HC-A3VK-2B / (unquoted) | turbo-charged engines (SA257CE) |
| Lycoming IO-540-B1A | ? | Factory (tests - serial 654 only) |

Notes:

The FAA Type Certificate Data Sheet quotes the HC-82XK-2C1 propeller hub as “HC-82XX-2C1.”

The FAA Type Certificate Data Sheet quotes the HC-A2XK-2C1 propeller hub as “HC-A2XX-2C1.”

The Model 500 also was equipped with the outboard wing panel extensions even though it didn't have the 'E' prefix on its model number.

-------------------------------------------------------------------oooOOOooo-------------------------------------------------------------------