# Boeing 767 Normal Procedure Checklists

PRE-FLIGHT

(ELECTRICAL POWER NOT ESTABLISHED)

Battery Switch 									ON

Standby Power Selector  								AUTO

Verify that battery DISCH Light illuminated and Standby Bus OFF Light extinguishes.

Hydraulic Electric Pump Switches							OFF

Hydraulic Demand Pump Selectors						OFF

Landing Gear Lever 								DOWN

Alternate Flaps Selector								NORM

Electrical Power									ESTABLISHED

APU Selector, -or-									START

External Power Switch   								PUSH ON

APU GEN Switch 									ON

(ELECTRICAL POWER ESTABLISHED)

IRS											OFF, -then- 
NAV

Verify ALIGN Lights illuminated.

YAW Damper Switches								ON

Electronic Engine Control Switches						ON

Hydraulic System									SET

Primary Engine Pump Switches							ON

Primary Electric Pump Switches						OFF

Demand Pump Selectors								OFF

Electrical System									SET

Battery Switch									ON

Verify DISCH Light extinguished.

Standby Power Selector								AUTO

Verify Standby Power Bus OFF Light extinguished.

Bus Tie Switches									AUTO

Verify AC BUS OFF and Utility Bus OFF Lights extinguished.

Generator Control Switches							ON

Utility Bus Switches								ON

Ram Air Turbine UNLKD Light							EXTINGUISHED

CAUTION: SWITCH ACTIVATION WILL CAUSE DEPLOYMENT OF RAM AIR TURBINE!

Engine Ignition Selector								SINGLE

Set on position 1 or 2

Fuel System										SET

Fuel Pump Switches								OFF

Crossfeed Valve Switches							OFF

Verify VALVE Lights extinguished.

Wing and Engine Anti-Ice Switches						OFF

Passenger Signs Selectors							SET

No Smoking Selector								AUTO, -or- ON

Seatbelts Selector									AUTO, -or- ON

Air Conditioning System								SET

APU and Engine Bleed Air Switches						ON

Verify APU VALVE Light extinguished.

Left, Center and Right Isolation Switches					ON

Verify VALVE lights extinguished.

Pack Control Selectors								AUTO

Verify PACK OFF lights extinguished.

Trim Air Switch 									ON

Recirculation Fan Switches 							ON

Verify INOP Lights extinguished.

Compartment Temperature Controls 						AUTO

Set as desired and verify INOP Lights extinguished.

Flight Director Switch 								ON

EICAS Display 									CHECK

Secondary Engine Display 							ON

Verify that primary and secondary engine indicators display existing conditions,

oil quantity is adequate for flight and no excessive values are displayed.

Status Display 									ON

Verify hydraulic quantity indications do not display RF

FMC 											ROUTE SET

Flight Instrument Control Panel 							SET

HSI Range Selector 								AS DESIRED

HSI Mode Selector 								MAP

Map Switches 									AS DESIRED

Speedbrake Lever 									DOWN

Thrust Levers 									IDLE

Flap Lever 										SET

Position flap lever to agree with flap position indicator.

Parking Brake 									SET

Verify PARK BRAKE Light illuminated and matching indication on upper EICAS.

Stabilizer Trim Cutout Switches 							NORM

Fuel Control Switches 								CUT OFF

Fire Extinguishing System 								CHECK

Engine and APU Fire Switches 							IN

Cargo Fire Switches 								OFF

Engine, Cargo, and APU Fire Bottle Discharge Lights 			EXTINGUISHED

Reserve Brake and Steering Switch 						OFF

Standby Flight Instruments 							SET

Attitude Indicator									CHECK

Verify proper attitude.

Airspeed Indicator 								CHECK

Verify proper speed.

Altimeter 										SET

Set barometric indication. Verify Proper altitude.

Standby Engine Indicator Selector 						AUTO

Autobrake Selector 									OFF

VHF Comms 										SET

Instrument Sources 								CHECK

Flight Director Source Selector 							L *

NAV Source Selector 								FMC L *

Alternate Source Switches 							OFF *

Flight Instruments 									SET

Altimeter 										SET

Set barometric indication. Verify proper altitude and no flags.

Vertical Speed Indicator 								CHECK

Verify indication zero and OFF flag not in view.

Clock 										SET

ADI 											CHECK

Verify proper attitude, flight director command bars, flight mode displays

T0, T0, FD, proper airspeed, and no flags.

HSI 											CHECK

Verify  magnetic  track,  present  heading,  appropriate  route  and  no  flags.

Airspeed Indicator 								CHECK

Verify proper speed and no flags.

RDMI 										CHECK

Position  ADF/VOR  selectors,  as  desired.  Verify present heading and no

inappropriate flags.

Autoland Status Annunciator 							CHECK

Press on Test 1 then Test 2, verify indications then blank.

Alternate Gear Extend Switch 							OFF

Alternate Flaps 									CHECK

Alternate Flaps Selector 								NORM

Alternate Flaps Switches 							OFF

Ground Proximity / Configuration Gear Override Switch 			OFF

Ground Proximity Flap Override Switch 						OFF

COCKPIT PREPARATION FINAL

(This procedure is accomplished after papers are aboard and crew is ready for engine start.)

Fuel Loading 									CHECK

Takeoff Thrust Reference   							SET

Verify correct thrust reference mode displayed.

FMC 											SET

Wind (If required) 								ENTER

Slope/Condition (If required) 							ENTER *

Acceleration Height 								CHECK

Command Airspeed Bugs 								SET

Start Clearance 									OBTAIN

Hydraulic System 									SET

Electric Primary Pump Switches 						ON

Verify PRESS Lights extinguished.

Demand Pump Selectors 							AUTO

Verify PRESS Lights extinguished.

Fuel Pump Switches 								ON

Position  switches ON for all tanks containing fuel and verify that all left and right

Pump PRESS Lights are extinguished.

PAC  Control Selectors    								OFF

Trim 											SET

Flight Controls 									CHECK

Secondary Engine Display 							ON

# PRE-FLIGHT CHECKLIST COMPLETED

----------------------------------------------------------------------------------------------------

# BEFORE START

RED ANTI-COLL. LIGHTS . . . . . . . . . .	ON

IRS . . . . . . . . . . . . . . . . . . . . . . . . . . . . 	ALIGNED

CDU . . . . . . . . . . . . . . . . . . . . . . . . . . . 	SET

MCP . . . . . . . . . . . . . . . . . . . . . . . . . . .	SET

FLIGHT INSTRUMENTS . . . . . . . . . . .	SET

SPEED BUGS . . . . . . . . . . . . . . . . . . . .	SET

STAB TRIM . . . . . . . . . . . . . . . . . . . . . .	SET

# BEFORE START CHECKLIST COMPLETED

[PROCEED WITH ENGINE START]

----------------------------------------------------------------------------------------------------

# AFTER START

ENG PARAMS . . . . . . . . . . . . . . . . . . .	CKD

APU . . . . . . . . . . . . . . . . . . . . . . . . . . . .	OFF

ENG / WING ANTI-ICE . . . . . . . . . . . . .	A/R

PACKS . . . . . . . . . . . . . . . . . . . . . . . . .	AUTO

L & R ISOL SWITCHES . . . . . . . . . . . . 	OFF

GROUND EQUIPMENT . . . . . . . . . . . . 	CLEAR

# AFTER START CHECKLIST COMPLETED

# ----------------------------------------------------------------------------------------------------

# TAXI

AUTOBRAKE . . . . . . . . . . . . . . . . . . . .	RTO

FLIGHT CONTROLS . . . . . . . . . . . . . . 	CKD

# RECALL . . . . . . . . . . . . . . . . . . . . . . . .	CKD

# TAXI CHECKLIST COMPLETED

# ----------------------------------------------------------------------------------------------------

# BEFORE TAKEOFF

TAKEOFF BRIEFING . . . . . . . . . . . . . .	DONE

COCKPIT WINDOWS . . . . . . . . . . . . .	LOCKED

FLAPS . . . . . . . . . . . . . . . . . . . . . . . . .	SET

AFDS T/O MODE . . . . . . . . . . . . . . . . .	ACTIVE

LDG LTS . . . . . . . . . . . . . . . . . . . . . . . .	ON

# BEFORE TAKEOFF CHECKLIST COMPLETED

# ----------------------------------------------------------------------------------------------------

# AFTER TAKEOFF

LANDING GEAR . . . . . . . . . . . . . . . . . .	UP & OFF

FLAPS . . . . . . . . . . . . . . . . . . . . . . . . . .	UP

AFDS LAT/VERT MODES . . . . . . . . . .	ACTIVE

CLB THRUST . . . . . . . . . . . . . . . . . . . .	SET

# AFTER TAKEOFF CHECKLIST COMPLETED

# ----------------------------------------------------------------------------------------------------

# CLIMB

SEAT BELTS . . . . . . . . . . . . . . . . . . . . 	AUTO

LDG LTS . . . . . . . . . . . . . . . . . . . . . . . .	OFF

ALTIMETERS . . . . . . . . . . . . . . . . . . . .	STD

# CLIMB CHECKLIST COMPLETED

# ----------------------------------------------------------------------------------------------------

# DESCENT

SEAT BELTS . . . . . . . . . . . . . . . . . . . . 	ON

FUEL QUANTITY/BALANCE	. . . . . . . .	CKD

MCP/MSA ALT . . . . . . . . . . . . . . . . . .	SET/CKD

AUTOBRAKE . . . . . . . . . . . . . . . . . . . .	SET

DESCENT CHECKLIST COMPLETED

# ----------------------------------------------------------------------------------------------------

# APPROACH

APP BRIEFING . . . . . . . . . . . . . . . . . . .	DONE

CDU . . . . . . . . . . . . . . . . . . . . . . . . . . .	SET

SPEED BUGS . . . . . . . . . . . . . . . . . . . .	SET

LDG LTS . . . . . . . . . . . . . . . . . . . . . . . .	ON

ALTIMETERS . . . . . . . . . . . . . . . . . . . .	SET

RECALL . . . . . . . . . . . . . . . . . . . . . . . .	CKD

# APPROACH CHECKLIST COMPLETED

# ----------------------------------------------------------------------------------------------------

# LANDING

LANDING GEAR . . . . . . . . . . . . . . . . . .	DOWN/ GREEN

FLAPS . . . . . . . . . . . . . . . . . . . . . . . . . .	SET

SPEEDBRAKE . . . . . . . . . . . . . . . . . . .	ARMED

MAP, ALT & G/A BRIEFING . . . . . . . . .	SET &

DONE

LDG CLEARANCE….. . . . . . . . . . . . . .	RCVD

EMERGENCY LANDING BRIEFING…..	DONE

# LANDING CHECKLIST COMPLETED

# ----------------------------------------------------------------------------------------------------

# AFTER LANDING

APU . . . . . . . . . . . . . . . . . . . . . . . . . . . .	A/R

AUTOBRAKE . . . . . . . . . . . . . . . . . . . .	OFF

SPEEDBRAKE . . . . . . . . . . . . . . . . . . .	DOWN

FLAPS . . . . . . . . . . . . . . . . . . . . . . . . . .	UP

LDG LTS . . . . . . . . . . . . . . . . . . . . . . . .	OFF

STAB TRIM . . . . . . . . . . . . . . . . . . . . . .	4 UNITS

AFTER LANDING CHECKLIST COMPLETED

# ----------------------------------------------------------------------------------------------------

# PARKING

PARKING BRAKE . . . . . . . . . . . . . . . . .	SET

ELECTRICAL POWER . . . . . . . . . . . . .	ESTD

LIGHTS . . . . . . . . . . . . . . . . . . . . . . . . .	ALL OFF

FUEL CONTROL SWITCHES . . . . . . .	CUTOFF

IRS . . . . . . . . . . . . . . . . . . . . . . . . . . . .	CHECK

HYDRAULIC SYSTEM . . . . . . . . . . . . .	SET

PASS SIGNS . . . . . . . . . . . . . . . . . . . . 	OFF

FUEL PUMPS . . . . . . . . . . . . . . . . . . . .	OFF

ENG / WING ANTI-ICE . . . . . . . . . . . . .	OFF

PACKS . . . . . . . . . . . . . . . . . . . . . . . . .	A/R

L & R ISOL SWITCHES . . . . . . . . . . . .	ON

FLIGHT DIRECTORS . . . . . . . . . . . . . .	OFF

# PARKING CHECKLIST COMPLETED

# ----------------------------------------------------------------------------------------------------

# SECURE

IRS . . . . . . . . . . . . . . . . . . . . . . . . . . . .	OFF

PACKS . . . . . . . . . . . . . . . . . . . . . . . . .	OFF

APU . . . . . . . . . . . . . . . . . . . . . . . . . . . .	OFF

If APU is OFF and no external power selected:

STBY PWR . . . . . . . . . . . . . . . . . . . . . .	OFF

BATTERY SW . . . . . . . . . . . . . . . . . . . .	OFF

SECURE CHECKLIST COMPLETED

# Boeing 767 Abnormal Procedure Checklists

ELECTRICAL

AC BUS(ES) OFF

Message: 	L or R AC BUS OFF

Light: 		L and/or R BUS OFF

GENERATOR CONTROL SWITCH(ES) ……………………………………………. OFF THEN ON

Attempt one reset of the generator control switch(es).

APU (If Available) …………………………………………….………………………… START

After APU RUN Light illuminates and APU GEN OFF Light remains extinguished:

BOTH BUS TIE SWITCHES…………………………………………………………… OFF THEN AUTO

Attempt one reset of the Bus Tie switches.

If both BUS OFF Lights were illuminated and AC power is restored:

Reactivate FMC route and re-enter performance data. Select ATT mode on IRS(s)

with ALIGN Light(s) illuminated.

If one BUS OFF Light remains illuminated:

Flight in icing conditions may result in some erroneous flight instrument

indications. [Associated probe heat inoperative.]

Left BUS OFF Light illuminated:

L and C autopilots inoperative. L and C flight directors inoperative.

Right BUS OFF Light illuminated:

R autopilot/flight director inoperative.

If both BUS OFF Lights remain illuminated:

If EEC Inoperative Lights illuminated:

THRUST LEVERS…………….……………………………………………………… RETARD TO MID

ELECTRONIC ENGINE CONTROL SWITCHES ………………………………… OFF*

Turn both EEC switches to OFF*.

Accomplish this action methodically, one switch at a time*.

If engine inoperative:

FUEL CROSSFEED VALVE SWITCHES ………………………………………… OPEN

FLIGHT DECK TEMPERATURE CONTROL …………………………………….. MAN*

Master caution inoperative.

Antiskid for outboard wheels inoperative braking effectiveness reduced, brake

with caution.

Auto speedbrake inoperative - deploy speedbrakes manually after Landing.

STANDBY BUS OFF

Message: 	STANDBY BUS OFF

Light: 		OFF

STANDBY POWER SELECTOR …………………………………………………… BAT

The battery will provide standby bus power for approximately 30 minutes.

If standby power bus OFF light remains illuminated and right AC bus power is

Available.

STANDBY POWER SELECTOR …………………………………………………… AUTO

GENERATOR DRIVE

Message: 	L or R GEN DRIVE

Light: 		DRIVE

GENERATOR DRIVE DISCONNECT SWITCH …………………………………… PUSH

APU (If Available) ……………………………………………………………………… START

GENERATOR OFF

Message: 	L or R GEN OFF or APU GEN OFF

Light: 		OFF

H

GENERATOR CONTROL SWITCH (APU/Engine) ……………………………… OFF THEN ON

If engine generator control OFF light remains illuminated:

# ------------------------

HYDRAULIC

HYDRAULIC OVERHEAT

E

Message: 	L, C, or R DEM HYD OVHT, L or R PRIM HYD OVHT,C HYD 1 or C HYD 2 OVHT

Light: 		OVHT

PUMP SWITCH/SELECTOR ………………………………………………………… OFF

HYDRAULIC PUMP

Message: 	L, C or R HYD DEM PUMP,L or R HYD PRIM PUMP,C HYD PRIM 1 or 2

Light: 		PRESS

If primary pump PRESS Light illuminated:

PRIMARY PUMP SWITCH …………………………………………………………… OFF

If demand pump PRESS light illuminated:

DEMAND PUMP SELECTOR ………………………………………………………... ON

If PRESS light remains illuminated:

DEMAND PUMP SELECTOR ………………………………………………………... OFF

HYDRAULIC SYSTEM(S) PRESSURE

Message: 	L, C or R HYD SYS PRESS

Light: 		SYS PRESS

DEMAND PUMP SELECTOR(S) (Affected System(s)) …………………………… ON

If SYS PRESS light remains illuminated:

PUMP SWITCH(ES) & SELECTOR(S)(Affected Pumps) ………………………… OFF

Turn off pumps with PRESS lights illuminated.

If L system only inoperative:

L autopilot inoperative

If C system only inoperative:

Automatic speedbrake inoperative

C autopilot inoperative

R autopilot stabilizer trim inoperative.

Use flaps 20 and VREF 20 for landing Accomplish C SYSTEM INOPERATIVE

LANDING PREPARATION

If R system only inoperative:

Auto brakes inoperative

R autopilot inoperative

If L and R systems inoperative:

Auto brakes inoperative

L and R autopilots inoperative

Crosswind limit 20 knots.

Use flaps 20 and VREF 30 + 20 knots for landing

GROUND PROXIMITY FLAP OVERRIDE SWITCH …………………………………OVRD

If L and C systems inoperative:

Automatic speedbrake inoperative

Elevator feel inoperative

ALL autopilots inoperative

Crosswind Limit 20 knots

Use flaps 20 and VREF 30 + 20 knots for landing

Accomplish C SYSTEM INOPERATIVE LANDING PREPARATION

If C and R systems inoperative:

Automatic speedbrake inoperative

Normal brakes inoperative

Primary source to alternate brakes inoperative

Auto brakes inoperative

C and R autopilots inoperative

Crosswind limit 20 knots

Use flaps 20 and VREF 30 + 20 knots for landing.

Accomplish C SYSTEM INOPERATIVE LANDING PREPARATION

C SYSTEM INOPERATIVE LANDING PREPARATION

ALTERNATE FLAP OPERATION:

GROUND PROXIMITY FLAP OVERRIDE SWITCH ……………………………………… OVRD

ALTERNATE FLAPS SELECTOR …………………………………………………………... SET

Position to agree with flap Lever.

ALTERNATE FLAPS SWITCHES …………………………………………………………... ALTN

ALTERNATE FLAPS SELECTOR ……………………………………………………………SET

Extend or retract flaps as required.

ALTERNATE GEAR EXTENSION:

LANDING GEAR LEVER ………………………………………………………………………OFF

ALTERNATE GEAR EXTENSION SWITCH (Max 250K-.75M) ………………………….. DN

After gear down Lights illuminate:

LANDING GEAR LEVER ……………………………………………………………………... DN

RESERVE BRAKES/STEERING SWITCH ……………………………………………..….. ON

NOTE: If No. 1 Center Primary Hydraulic pump PRESS Light remains

illuminated, Reserve Brakes and Nose Wheel Steering are inoperative.

ANTISKID (FAULT)*

Message: 	ANTISKID*

Light: 		ANTISKID*

Braking effectiveness is reduced, brake with caution*.

BRAKE SOURCE

Message: 	BRAKE SOURCE

Light: 		BRAKE SOURCE

RESERVE BRAKES/STEERING SWITCH ……………………………………………… ON

If BRAKE SOURCE light remains illuminated:

AUTO BRAKES SELECTOR ……………………………………………………………… OFF

Only accumulator pressure available for brakes; apply steady increasing

brake pressure and hold to full stop.

GEAR DISAGREE

Message: 	GEAR DISAGREE

Light: 		GEAR

NOTE: Do not use FMC fuel predictions with Landing gear extended If gear Lever UP:

bserve EXTEND OR EXTENDED Limit speed (270K.82M).

If gear Lever DN and all gear down (green) Lights illuminated:

Accomplish normal Landing.

If gear Lever DN and any gear down (green) Light(s) not illuminated:

LANDING GEAR LEVER ………………………………………………………………… OFF

ALTERNATE GEAR EXTENSION SWITCH (Maximum 250K-.75M) …………….… DN

If all gear down Lights illuminated:

LANDING GEAR LEVER ………………………………………………………………… DN

If any gear down (green) light(s) not illuminated:

- Use flaps 30 for Landing.

- Use manual speedbrake after landing.

LANDING GEAR LEVER ………………………………………………………………… DN

GROUND PROXIMITY/CONFIGURATION GEAR OVERRIDE SWITCH ……….… OVRD

At pattern altitude:

PACK CONTROL SELECTORS ………………………………………………………… OFF

FUEL PUMP SWITCHES ………………………………………………………………… OFF

# ----------------------------------------

FLIGHT CONTROLS

LEADING EDGE SLAT DISAGREE

Message: 	LE SLAT DISAGREE

Light: 		LEADING EDGE

GROUND PROXIMITY FLAP OVERRIDE SWITCH …………………………………… OVRD

If indicated flap position greater than 20:

Use existing flaps and VREF 20 for Landing.

If indicated flap position 20 or less:

Use flaps 20 and VREF 20 for landing.

ALTERNATE FLAPS SELECTOR …………………………………………………………. SET

Position to agree with flap Lever.

LEADING AND TRAILING EDGE ALTERNATE FLAPS SWITCHES …………………. ALTN

CAUTION: IF, AFTER SELECTING ALTN, THE LEADING EDGE OR TRAILING

EDGE LIGHT REMAINS ILLUMINATED, POSITION THE ASSOCIATED ALTERNATE

FLAPS SWITCH OFF AND ACCOMPLISH THE APPLICABLE ASYMMETRY PROCEDURE.

ALTERNATE FLAPS SELECTOR …………………………………………………………. SET

Extend or retract flaps as required.

TRAILING EDGE FLAP DISAGREE

Message: 	TE FLAP DISAGREE

Light: 		TRAILING EDGE

GROUND PROXIMITY FLAP OVERRIDE SWITCH ……………………………………. OVRD

If indicated flap position greater than 20:

Use existing flaps and VREF 20 for landing

If indicated flap position 20 or less:

Use flaps 20 and VREF 20 for Landing.

ALTERNATE FLAPS SELECTOR ………………………………………………………… SET

Position to agree with flap Lever.

LEADING AND TRAILING EDGE ALTERNATE FLAPS SWITCHES ………………… ALTN

CAUTION: IF, AFTER SELECTING ALTN, THE LEADING EDGE OR TRAILING

EDGE LIGHT REMAINS ILLUMINATED, POSITION THE ASSOCIATED ALTERNATE

FLAPS SWITCH OFF AND ACCOMPLISH THE APPLICABLE ASYMMETRY PROCEDURE.

ALTERNATE FLAPS SELECTOR ……………………………………………………… SET

Extend or retract flaps as required.

LEADING EDGE SLAT ASYMMETRY

Message: 	LE SLAT ASYM

Light: 		LEADING EDGE

CAUTION: DO NOT ARM ALTERNATE LEADING EDGE FLAPS

GROUND PROXIMITY FLAP OVERRIDE SWITCH ………………………………… OVRD

NOTE: Do not use FMC fuel predictions with flaps extended.

If indicated flap position greater than 20:

Use existing flaps and maintain VREF 30 + 20 knots minimum for landing.

If indicated flap position 20 or less:

Use trailing edge flaps 20 and VREF 30 + 30 knots for landing.

ENGINE START SELECTORS ………………………………………………………… CONT

ALTERNATE FLAPS SELECTOR ……………………………………………………… SET

Position to agree with flap Lever.

ALTERNATE FLAPS SWITCH (Trailing Edge) ……………………………………..… ALTN

LTERNATE FLAPS SELECTOR ……………………………………………………….. SET

Extend or retract trailing edge flaps as required.

TRAILING EDGE FLAP ASYMMETRY

Message: 	TE FLAP ASYM

Light: 		TRAILING EDGE

CAUTION: DO NOT ARM ALTERNATE TRAILING EDGE FLAPS.

GROUND PROXIMITY FLAP OVERRIDE SWITCH ………………………………… OVRD

NOTE: Do not use FMC fuel predictions with flaps extended.

If indicated flap position 5 or Less, use VREF 30 + 30 knots for landing.

If indicated flap position between 5 and 20, use VREF 30 + 20 knots for Landing.

If indicated flap position at or greater than 20, use VREF 20 for Landing.

NO FLAP AND NO SLAT LANDING

This procedure is to be used when no flaps and no slats are extended (ALTNextension failed).

ENGINE START SELECTORS ……………………………………………………...… FLT

GND PROX FLAP OVRD SWITCH …………………………………………………… OVRD

The Too Low Terrain and Too Low Flaps advisories are inhibited.

APPROACH PREPARATION

Maneuver at Flaps 30 VREF + 80 knots until on final approach.

Use longest runway available. Recommend a 10-mile final.

If ILS available, the autopilot may be used for approach. Autoland is not

recommended.

LANDING PREPARATION

Use Flaps 30 VREF + 50 knots for landing. Autobrakes, if operative, must be

used when braking action is reported to be less than good, or on wet/slippery

runways, or on runways 7,000 feet or less in length. On dry runways greater

than 7,000 feet in length, the autobrake system is not required but may be

used at Captain's discretion.Arm Speed Brakes.Extend landing gear at first indication of 	G/S movement.When landing is assured (100 feet AFL), retard throttles to idle.

# ----------------------------

PNEUMATICS

BLEED DUCT LEAK

Message: 	L or R BLD DUCT LEAK

Light: 		DUCT LEAK

Condition: Bleed air Leak detected in the Left or right pneumatic duct.

ENGINE BLEED AIR SWITCH (Affected side) ………………………………………… OFF

ISOLATION SWITCH (Affected side) …………………………………………………… OFF

WING ANTI-ICE SWITCH ………………………………………………………………… OFF

Avoid icing conditions.

Do not use wing anti-ice.

CAUTION: Flight beyond 6 hours with a DUCT LEAK Light illuminated may result in structural damage.

ENGINE BLEED OFF

Message: 	L or R ENG BLEED OFF

Light: 		OFF

If wing anti-ice required:

ONE PACK CONTROL SELECTOR …………………………………………………… OFF

ISOLATION SWITCH (Affected Side) ………………………………………………….. ON

Return to OFF when wing anti-ice is no Longer required.

Thrust reverser (affected side) may be inoperative.

ENGINE BLEED OVERHEAT*

Message: 	L or R ENG BLD OVHT*

Light: 		OVHT*

ENGINE BLEED AIR SWITCH (Affected side) ........................................................OFF THEN ON*

Select to ON after thrust is reduced for cruise.

If OVHT Light remains illuminated or re-illuminates*:

ENGINE BLEED AIR SWITCH (Affected side) ....................................................... OFF*

If wing anti-ice required*:

ONE PACK CONTROL SELECTOR ...................................................................... OFF*

ISOLATION SWITCH (Affected side) ..................................................................... ON*

Return to OFF when wing anti-ice is no longer required*.

Thrust reverser (affected side) inoperative*.

# -----------------------------------------------------------------------------------------------

RAPID DEPRESSURIZATION /CABIN ALTITUDE

NOTE: CABIN ALTITUDE EICAS message and CABIN ALT light will display if cabin

altitude exceeds 10,000 feet.

C ISLN VALVE ................................................................................................. …	CLOSE

ENGINE BLEED VALVES ……………………………………………………………	CHECK OPEN

PACK CONTROL SELECTORS ……………………………………………………..	CHECK AUTO

L AND R ISLN VALVES ………………………………………………………………	CHECK CLOSED

If not arrested by 10.000 ft cabin altitude:

INITIATE EMERGENCY DESCENT

EMERGENCY DESCENT

AUTOTHROTTLE ………………………………………………………………………AS DESIRED

THROTTLES …………………………………………………………………………… CLOSE

SPOILER HANDLE ……………………………………………………………………..FULL AFT

NO SMOKING AND SEAT BELT SIGN ………………………………………………ON

Descent straight ahead or initiate moderate bank (max 30°)

TARGET SPEED ……………………………………………………………………….MAX Vmo

ENGINE START SELECTORS ……………………………………………………… FLT

- Advise passengers to: Use oxygen; Fasten seat belts; No smoking. Make the

following announcement: "Ladies and gentlemen, pull down and use your

oxygen masks. Fasten your seatbelts and no smoking please."

- When time permits: Advise Flight Attendants if cabin altitude remains above

10,000 feet for an extended period. Advise Flight Attendants when cabin

altitude is below 10,000 feet.

NOTES:

1. The above procedure assumes structural integrity of the aircraft. If

structural integrity is in doubt, limit airspeed as much as possible and avoid

high maneuvering loads until structural integrity has been verified.

2. Level-Off (If not in FL CH mode)

- Start reducing rate of descent approximately 2,000 feet above desired

level-off altitude.

- Retract speed brakes approximately 1,000 feet above desired level-off

altitude, or as desired to control airspeed and level-off.

3. Level-Off (Using FL CH mode)

- Retract speed brakes approximately 1,000 feet above desired level-off

altitude, or as desired to control airspeed and level-off.

4. After Level-Off

- Speed Brakes - Check retracted

- Engine Start Selectors - As Required

# -------------------

ENGINES

ENGINE FIRE, SEVERE DAMAGE OR SEPARATION

Message: 	L or R ENGINE FIRE

Light: 		LEFT or RIGHT engine fire switch and associated fuel control switch.

Condition: Airframe vibrations with abnormal engine indications.

THRUST LEVER ………………………………………………………………………CLOSE

Disengage autothrottle if on.

FUEL CONTROL SWITCH …………………………………………………………..CUT OFF

ENGINE FIRE SWITCH ………………………………………………………………PULL

If engine fire warning light remains illuminated:

ENGINE FIRE SWITCH ………………………………………………………………ROTATE

Rotate to the stop and hold for 1 second.

After 30 seconds, if engine fire warning Light remains illuminated:

ENGINE FIRE SWITCH …………………………………………. ROTATE TO REMAINING BOTTLE

Rotate to the stop and hold for 1 second.

If high airframe vibration occurs and continues after engine is shut down:

Without delay, reduce airspeed and descend to a safe altitude which results in

an acceptable vibration level. If high vibration returns and further airspeed

reduction and descent is not practical, increasing the airspeed may reduce the

vibration.

APU (If Available) ………………………………………………………………………START

GROUND PROXIMITY FLAP OVERRIDE SWITCH ……………………………….OVRD

If wing anti-ice required:

ONE PACK CONTROL SELECTOR …………………………………………………OFF

ISOLATION SWITCH (Affected Side) ………………………………………………..ON

Return to OFF when wing anti-ice is no longer required.

LANDING PREPARATION

Use flaps 20 and VREF 20 for landing and flaps 5 for go-around.

Condition: - Engine failure or flameout

- Engine parameters cannot be maintained within Limits

- When prescribed by other non-normal procedure

THRUST LEVER ……………………………………………………………………….CLOSE

Engine conditions permitting, operate at idle for 2 minutes to allow engine

to cool and stabilize. Disengage autothrottle if on.

FUEL CONTROL SWITCH ……………………………………………………………CUT OFF

APU (If Available) ………………………………………………………………………START

GROUND PROXIMITY FLAP OVERRIDE SWITCH ……………………………….OVRD

If wing anti-ice required:

ONE PACK CONTROL SELECTOR …………………………………………………OFF

ISOLATION SWITCH (Affected Side) ………………………………………………..ON

Return to OFF when wing anti-ice is no longer required.

LANDING PREPARATION

Use flaps 20 and VREF 20 for landing and flaps 5 for go-around.

LOSS OF THRUST ON BOTH ENGINES

Condition: Thrust or thrust lever response is Lost from both engines.

ENGINE START SELECTORS ………………………………………………………FLT

NOTE: Takeoff EGT limit is in-flight start EGT Limit.

RAM AIR TURBINE ……………………………………………………………………UNLKD

AIRSPEED …………………………………………………………………….….. ___KTS

Above 30,000 ft - 240 kts

30,000 ft or below - 220 kts minimum

NOTE: OVSPD light and associated aural warning will indicate Vmo/Mmo exceedances.

If engines do not recover:

THRUST LEVERS …………………………………………………………………….CLOSE

FUEL CONTROL SWITCHES ……………………………………………… CUT OFF then RUN

Repeat until engine start achieved (Remain approximately 30 seconds in each position).

APU (If Available) ……………………………………………………………………..START

After restoring AC power, reactivate FMC route and re-enter performance data.

Select ATT mode on IRS(s) with ALIGN lights) illuminated.

NOTE: Cabin altitude warning may occur during descent.

ENGINE IN-FLIGHT START

Condition: Following a shutdown or flameout when fire or apparent damage has not occurred.

ALTITUDE AND AIRSPEED ………………………………………………………..CHECK

Starts not assured outside EICAS envelope.

If X-BLD not displayed:

ENGINE START SELECTOR ………………………………………………………FLT

FUEL CONTROL SWITCH …………………………………………………………RUN

Abort start if EGT fails to rise within 45 seconds.

If X-BLD displayed:

ONE PACK CONTROL SELECTOR ………………………………………………OFF

ISOLATION SWITCH (Affected Side) ……………………………………………..ON

IGNITION SELECTOR ………………………………………………………………BOTH

ENGINE START SELECTOR ………………………………………………………GND

FUEL CONTROL SWITCH ………………………………………………………….RUN

Select when N2 exceeds Fuel On Command bug [15%]. Abort start if EGT

fails to rise within 45 seconds.

After engine started:

ENGINE START SELECTOR ………………………………………………………AUTO

PACK CONTROL SELECTORS ……………………………………………………AUTO

LEFT AND RIGHT ISOLATION SWITCHES ……………………………………..OFF

GROUND PROXIMITY FLAP OVERRIDE SWITCH …………………………….OFF

# ---------------------------------------------------

TAKE OFF AND LANDING

PASSENGER EVACUATION

Condition: The situation requires the evacuation of the passengers and crew.

PARK BRAKE …………………………………………………………………….…SET

ALL ENGINE AND APU FIRE SWITCHES ………………..…… OVERRIDE, PULL & ROTATE

Rotate engine fire switches in opposite directions

PASSENGER EVACUATION ……………………………………………………..INITIATE

Initiate the evacuation by informing the tower and flight attendants of

passenger evacuation.

DITCHING PREPARATION

Condition: The flight conditions require airplane ditching and evacuation

DISTRESS SIGNAL …………………………………………………………….….TRANSMIT

CREW AND PASSENGERS ………………………………………………………ADVISE

Below 5,000 feet:

GROUND PROXIMITY/CONFIGURATION GEAR OVERRIDE SWITCH ……OVRD

PACK CONTROL SELECTORS …………………………………………………..OFF

NO SMOKING SIGNS SELECTOR ……………………………………………….ON

Final Approach (Omit LANDING CHECKLIST):

PASSENGER CABIN ……………………………………………………………….SECURED

LANDING GEAR LEVER …………………………………………………………...UP & OFF

FLAPS .………………………………………………………………………………. 30

Advise cabin crew of imminent touchdown.

Maintain airspeed at VREF. Flare airplane to achieve minimum rate of descent

at touchdown.