MD-80

Microsoft Flight Simulator X

Panel: Pablo Contouris

Pre-Start Checklist

- Parking Brake 	SET

- Throttle 	IDLE

- Fuel Flow 	CUTOFF

- BATT Master Switch 	ON

- Panel Lights 	ON if required

- Hydraulic Pump Switches 	ON

- Landing Gear Lever 	CHECK DOWN

- Flaps 	UP

- Spoiler 	RETRACTED

- APU 	START / CHECK RUN

- APU 	Gen ON / CHECK VOLTS

- Fuel Quantity 	CHECK

- Pitot Heat / De-Ice 	OFF

- Aircraft Lighting 	OFF

- Flight controls 	FREE AND CORRECT

- Passenger Signs 	OFF

- Check Weather 	(Atis, Flight Services)

- De-Ice 	TEST/CHECK

- Request Clearance

- Transponder 	STANDBY

- Beacon / Anti Collision Lights 	ON

Startup Checklist

- Thrust Levers 	IDLE

- Engine Area 	CLEAR

- Engine Start Switches 	GND / IGN BOTH

- Fuel Pump Switches 	ON

- Left Engine

  - Master Starter Switch 	ON

  - At N2 > 20% Fuel Flow 	CHECK ON

  - N1 increasing as N2 incr. 	CHECK

  - Oil Pressure 	CHECK

  - Generator Switch 	ON

- Repeat for right Engine

- Fuel Pump Switches 	OFF

- APU Gen, APU 	OFF

Before Taxi Checklist

- Nav Lights 	ON

- Taxi Lights / Runway Turnoff Lights	ON

- Heading Indicator/Altimeters 	SET

- Standby Instruments 	SET

- Radios and Avionics 	SET FOR DEPARTURE

- Autopilot 	ON, SET, don‘t activate

- Autothrottle 	ON, SET, don‘t activate

- F/D 	ON

- Autobrake 	RTO

- Elevator Trim 	SET for takeoff

- Request Taxi Clearance

Taxi Checklist

- Parking Brake 	RELEASE

- Taxi to assigned runway 	SPEED Max. 20 knots

- Brakes/Gyro/Turn Coordinator 	CHECK during taxi and turns

Before Take-off Checklist

- Parking Brake 	SET

- Throttle 	IDLE

- Elevator Trim 	SET for takeoff

- Flaps 	5 deg

- Spoilers 	RETRACTED

- Flight Instruments 	CHECK

- Engine Instruments 	CHECK

- Takeoff Data (V1, VR, V2) 	CHECK

- Nav Equipment 	CHECK

- Landing Lights 	ON

- Taxi Lights 	OFF

- Strobe Light 	ON

- Pitot Heat 	ON

- De-Ice 	AS REQUIRED

- Transponder 	ON

- Request Takeoff Clearance

Take-off Checklist

- Smoothly increase thrust to 	60% N1 let spool up

- Takeoff Thrust 	FULL or TO/GA

- Brakes 	RELEASE

- V1 = 	135 KIAS (descision)

- Vr = 	140 KIAS (rotate)

- Pitch 	10 deg. nose up

- V2 = 	145 KIAS (safety speed)

- At Positive Climb Rate 	Touch Brakes

- Landing Gear 	RETRACT

- At 1000' AGL 	RETRACT Flaps to 1 deg

- At 210 KIAS 	RETRACT flaps up

Climb-out Checklist

- Throttle 	AS REQUIRED

- Trim for 	250 KIAS / 1800 fpm

- Autopilot/Autothrottle 	CHECK and ACTIVATE

- Autobrake 	OFF

- Below 10000' max. speed 	250 KIAS

- ATC 	AS REQUIRED

- Fasten Seat Belts 	OFF

- No Smoking Sign 	OFF

- Landing Lights 	OFF

- Above 10000' speed 	280 KIAS

Cruise Checklist (above FL 180)

At Transition Altitude (FL180) set Altimeter to 	29,92" (1013mb)

- Accelerate to cruise speed 	FL180 240 KIAS

FL240 0.76 mach

FL280 0.76 mach

FL330 0.76 mach

- Engine+Instruments 	CHECK

- Fuel Quantity 	CHECK

- Radios 	TUNED and SET

- Autopilot 	CHECK and SET

- Lights 	as required

Descent Checklist

- Atis/Airport Information 	CHECK

- Altimeter 	CHECK

- Radios 	SET

- De-Ice 	AS REQUIRED

- Descent Speed 	to FL240 0.74 mach

to FL180 0.74 mach

At Transition Altitude (FL180) reset Altimeter to local

to FL120 290 KIAS

below 10'000ft 250 KIAS

- Fuel Quantities and Balance 	CHECK

- Flaps / Landing Gear 	CHECK UP

- Check Weather 	(ATIS, Flight Services)

Approach Checklist

Localizer Level Flight :

- Fasten Seat Belts 	ON

- No Smoking Sign 	ON

- APU 	START / CHECK RUN

- APU 	Gen ON / CHECK VOLTS

- Avionics + Radios 	SET

- Speed: Establish 	210 KIAS

- Landing Lights 	ON

- Auto Spoilers 	ARM

- Autobrake 	SET

- Flaps 	5-10 degrees

- Speed: Establish 	180 KIAS

- Flaps 	15-20 degrees

- Speed: Establish 	160 KIAS

- Landing Gear 	DOWN

- Turning toward runway: set flaps 	30 degrees or FULL

Final Glideslope Descent :

- Speed Establish 	155 KIAS

- Parking Brake 	VERIFY OFF

- De-Ice 	AS REQUIRED

Landing Checklist

- Landing Gear 	CHECK DOWN

- Autopilot and Autothrottle 	OFF

- Landing Speed 	150 KIAS

- After touchdown 	Apply Reverse Thrust, at

60 kts: Cancel Reverse Thrust

- Spoilers 	VERIFY EXTENDED

- Brakes 	AS REQUIRED

Taxi To Ramp

- Strobe Light 	OFF

- Autobrake	OFF

- Flaps 	UP

- Spoilers 	RETRACTED

- Taxi Lights 	ON

- Landing Lights 	OFF

- Speed 	Max. 20 knots

- Transponder 	1200 / OFF

- Elevator Trim 	TAKEOFF SETTING

Shutdown Checklist

- Parking Brake 	SET

- Throttle 	IDLE

- Passenger Signs 	OFF

- Pitot Heat / De-Ice 	OFF

- Taxi Lights 	OFF

- Nav Lights 	OFF

- F/D 	OFF

- Fuel Flow	OFF

- Master Starter Switches 	OFF

- Beacon / Anti Collision Lights 	OFF

- Hydraulic Pump Switches 	OFF

- APU Gen, APU 	OFF

- BATT Master Switch	OFF

Securing Aircraft

- Parking Brake 	Verify SET

- Throttle 	Verify IDLE

- All Switches 	Verify OFF