GARMIN GNS530 Default Nav Page

Com Active and Standby Frequencies

Nav Active and Standby Frequencies

Power Button

Com Active/Standby Frequency Switch

Nav Active/Standby

Frequency Switch

Ident Switch

Nav1 Ident

Nav1 Radial

Nav1 Distance

Com/Nav

Set Switch

Decrease Frequency (MHz)

Decrease Frequency (KHz)

Radio Frequency

Set Switch

Increase Frequency (MHz)

Increase Frequency (KHz)

Nav/GPS Switch

Nav Bar

Switch

1.   The power button turns the unit on and off.

2.   The top number in each radio box correspond to the active frequency, the bottom

number correspond to the standby frequency.

3.   The active/standby switch toggles the active frequency to standby and the standby

frequency to active.

4.   Nav1 ident readout is the 3 letter code of the Nav1 beacon tuned in on the Nav1

frequency.

5.   Click on VLOC knob to ident. ID appears in the upper right corner.

6.   Nav1 radial is the radial degrees from the Nav1 beacon. Convert to “direct to” by

adding or subtracting 180.

7.   Nav1 distance is the distance from the Nav1 beacon.

8.   The radio frequency set switch is used to tune the radios. The click areas are shown in

the figure above.

9. The Com1/Nav1 set toggle switch is used to toggle between tuning the Com and Nav radios.

10. The CDI switch ( Nav/GPS switch) toggles between GPS and Nav1 navigation modes.

11.  The Nav bar switch displays flight information in a boxed rectangle along the right

side of the screen. The bar can be displayed in the default or map pages. See the

accompanying “530 Nav Bar Page”.

12.  The CDI displays course deviation information in either Nav1 or GPS navigation

modes. In the Nav1 mode, the CDI works exactly like the standard CDI display

found on most aircraft. The deflection distance of the CDI needle corresponds to

~1 mile on each side of the desired track in the Nav1 mode. It corresponds to a ~5

mile distance on each side when tracking a waypoint in the GPS mode.

13.   All other buttons and their functions are the same as in the FS2004 default GPS500

unit and are described in the help section of the simulator. Please refer to this section

for information on the functions.

14.   Com/Nav standby frequencies are highlighted in cyan when setting frequency.

GARMIN GNS530 Nav Map Page

Digital Display of GPS

Waypoint Information

Toggle Waypoint

Information Bar

On/Off

GPS Waypoint

Information Bar

1.   Mostly self-explanatory.

2.   A bar on the right side displays specific information about the aircraft and its

geographical relative to the waypoint listed in the WPT box.

3.   The bar is toggled on and off by clicking on the “DEFAULT NAV” logo.