GARMIN GNS430 Default Map Page

Com Active and Standby Frequencies

Nav Active and Standby Frequencies

Power Button

Com Active/Standby Frequency Switch

Nav Active/Standby

Switch

Ident Toggle

Switch

Com/Nav

Set Switch

Decrease Frequency (MHz)

Decrease Frequency (KHz)

Radio Frequency

Set Switch

Increase Frequency (MHz)

Increase Frequency (KHz)

Nav/GPS Switch

Default/Map Page

Switch

Position Page

Toggle Switch

CDI Display

To/From Pointer

CDI Needle

Digital Display of

GPS Waypoint

Information

- The power button turns the unit on and off.

- The top number in each radio box corresponds to the active frequency, the bottom number corresponds to the standby frequency.

- The active/standby switches toggle the active frequency to standby and the standby frequency to active.

- The radio frequency set switch is used to tune the radios. The click areas are shown in the figure above.

- The Com/Nav set toggle switch is used to toggle between tuning the Com and Nav radios.

- The CDI switch ( Nav/GPS switch) toggles between GPS and Nav1 navigation modes.

- The map page bar switch displays a moving map and various flight information in a boxed rectangle along the right side of the screen. See the accompanying file “430 Nav Map Page”.

- Clicking on the GPS icon will bring up the position page which contains information boxes displaying information relating to the position of the aircraft and its relative position to the next waypoint. A clock displays local or Greenwich mean time. See the “430 Nav Pos Page” below.

9. The digital display of GPS waypoint information shows values only for GPS waypoints (excluding TRK and GS). If no waypoint is set in the GPS, the readout will display ----. The values will be displayed whether in the Nav1 or GPS navigation mode.

The CDI at the top of the page shows course deviation information in either Nav1 or GPS navigation modes. In the Nav1 mode, the CDI works exactly like the standard CDI display found on most aircraft. The deflection distance of the CDI needle corresponds to ~1 mile on each side of the desired track in the Nav1 mode. It corresponds to a 5 mile distance on each side when tracking a waypoint in the GPS mode.

10. All other buttons and their functions are the same as in the FS2004 default GPS500 unit and are described in the help section of the simulator. Please refer to this section for information on the functions.

11. Com/Nav standby frequencies are highlighted in cyan when setting frequency.

GARMIN GNS430 NAV Map Page

Digital Display of GPS

Waypoint Information

GPS Waypoint

Information Bar

Toggle Between  Default

and Map Pages

- Clicking on the “DEFAULT NAV” logo toggles the Map page on or off.

- The information bar lists the aircraft ground speed, and the distance and direction (magnetic) of the GPS waypoint identified in the WPT box.

3.   The Nav bar is only available on the map pages.

4.   The Map page is only available from the default Nav page. It is not available directly from the position page.

GARMIN GNS430 Position Page

Position Page

Toggle Switch

Graphical Readout

of Aircraft Trek

Aircraft Latitude and

Longitude Position

Local Time

(click for GMT)

GPS Waypoint

Information Boxes

Flight Information Boxes

- Flight information boxes contain information relating to the aircraft position in the real world.

- Trek graphical readout is self-explanatory. Shows the same information given in the TRK box as a sliding scale representation.

- The position box lists the latitude and longitude of the aircraft.

- Local or mean time listed, default is local time.

- The waypoint boxes contain information relating to the aircraft position relative to the next GPS waypoint identified.

6.   The position page is toggled on or off by clicking on the GPS logo as shown.

7.   The position page can be toggled directly from the default Nav or Map pages

8.   The default Nav and Map pages cannot be toggled on directly from the position page.

You must exit the position page before toggling between these 2 pages (re-click on GPS).