1.   GENERAL  AIRSPEED GUIDE

Rotate          140 kts

Take0ff         160  kts

Cruise           350KIAS   to   2.6M

Approach      160  kts   or  ONSPEED

Touchdown   135-150  kts  or  ONSPEED

CONFIGURATION   SPEEDS:

Slats-         Normal MAX         295KIAS

Structural MAX     330KIAS or 0.62M   whichever is less

Flap 25’-    Normal MAX         250KIAS or 0.62M   whichever is less

Structural MAX     330KIAS or 0.62M           “

Flap 34’-    Normal MAX         220KIAS or 0.48M           “

Structural MAX     300KIAS or 0.56M            “

Wing  16’-        Takeoff / Landing

Slow airspeeds below   330KIAS or 0.62M

26’-         Up to   0.80M

38’-         Up to   0.80M

47’-         Up to   0.80

Transonic    0.80M   to   1.10M

58’-         Transonic    0.80M    to   1.10M

1.10M   and   greater

72.5’-      Transonic     0.80M     to   1.10

1.10M    and    greater

Landing  gear -    Normal  MAX      295KIAS or 0.62M   whichever is less

Structural  MAX   330KIAS or 0.62M              “

Airbrake -             2.2M

GENERAL LOWER  ALTITUDE  OPERATIONS

~350KIAS   to   600KIAS  is a reasonable airspeed range .

~350 – 420KIAS  suggested for extreme low level manoeuvring

(slow enough to give  the pilot ‘time’ )

Below 350KIAS  control is not crisp and positive .

The  F-111  flies best at  350 , or 350 plus knots .

Note ; Energy and consequent airspeed bleed off during hard manoeuvring  , manipulate the throttle as appropriate to maintain desired airspeed .

2.   RATE 1  TURNS

The standard rate of turn in IFR instrument flying is a  3 degree change of heading per second .

Resulting in the following ‘timed’ heading changes:-

90  degrees   in     30 seconds

180      “            “      60     “   = 1 minute

270      “            “      90     “

360      “            “     120    “   = 2 minutes , as seen on Turn and Bank indicators .

The following  Table provides data relevant to  RATE  1 turns in the F-111 .

It lists data for straight and level ( S/L )  and then data for when stabilized in the  Rate 1 turn ( TURN ) .

The data was compiled in FS9, in flight at an altitude of 4000’ . Whilst the figures have not been verified in FSX  they should still represent a very good guide .

Example:-

From the table below , in level flight  at  350 knots with wing sweep at 26 degrees commence turn by smoothly banking with aileron to 52.5 degrees , increase power from 58 to 65% and increase trim from 0.5 to 3.1 , maintain bank angle , and keep the velocity vector on the horizon bar . If a  180 degree  ‘U turn’ is required , hit start button on timer at start of turn , and ensure a smooth rollout  to wings level  exactly as timer reaches  60 seconds . You are now on the reciprocal  of the initial heading . Setting the heading bug on the ADI prior to commencement is a helpful cross check .

3.                                 TABLE  OF  RATE  1  TURNS

| CONDITION | KIAS | AoB | PWR % | TRIM | WING | Gs | AoA |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S/L TURN | 200 “ | 30 | 61 64 | 0.8 2.9 | 16’+flap25’ w/o gear | 1.0 1.1 | 0.4 1.1 |
| S/L TURN | 200 “ | 30 | 64 66 | 0.8 2.7 | 16’+flap25’ w. gear | 1.0 - | 0.4 1.8 |
| S/L TURN | 300 “ | 45 | 54 62 | 1.0 3.3 | 26’ “ | 1.0 1.4 | 1.7 3.3 |
| S/L TURN | 350 “ | 52.5 | 58 65 | 0.5 3.1 | 26’ “ | 1.0 1.6 | 0.7 2.6 |
| S/L TURN | 400 “ | 60 | 61 67 | 0.2 3.3 | 26’ “ | 1.0 2.0 | 0.0 2.4 |
| S/L TURN | 450 “ | 67.5 | 66 75 | -0.1 3.8 | 26’ “ | 1.0 2,5 | -0.6 2.6 |
| S/L TURN | 450 “ | 67.5 | 65 74 | 0.3 4.1 | 38’ “ | 1.0 2.5 | 0.1 3.2 |
| S/L TURN | 500 “ | 75 | 72 86 | -0.2 5.0 | 26’ “ | 1.0 3.5 | -1.1 3.8 |
| S/L TURN | 500 “ | 75 | 69 87 | 0.1 5.4 | 38’ “ | 1.0 3.7 | -0.3 4.1 |
| S/L TURN | 500 “ | 75 | 69 87 | 0.3 5.5 | 47’ “ | 1.0 3.8 | 0.2 4.8 |
| S/L TURN S/L TURN S/L TURN S/L TURN S/L TURN | 550 “ “ “ “ “ “ “ “ “ | 82.5 | 79 97 74 97 73 97 73 97 73 97 | -0.4 8.2 0.0 8.7 0.2 8.9 0.4 9.0 0.7 9.4 | 26’ “ 38’ “ 47’ “ 58’ “ 72.5’ “ | 1.0 7.0 1.0 7.1 1.0 7.1 1.0 7.1 1.0 7.1 | -1.4 5.7 -0.6 6.5 0.0 7.0 0.5 7.4 1.1 8.1 |

4.  TABLE  OF  ONSPEED  AIRSPEEDS

The following table lists both approach and touchdown airspeeds for given

total fuel percentages and weights , as well as the applicable aircraft weight .

| FUEL % | FUEL WEIGHT | AIRCRAFT WEIGHT | APPROACH IAS | TOUCHDOWN IAS |
| --- | --- | --- | --- | --- |
| 16 | 5,420 | 68,290 | 126 | 121 |
| 24 | 8,050 | 70,940 | 128 | 123 |
| 32 | 10,650 | 73,540 | 131 | 125 |
| 39 | 13,200 | 76,100 | 133 | 127 |
| 48 | 16,170 | 79,050 | 136 | 130 |
| 58 | 19,400 | 82,250 | 139 | 133 |
| 66 | 22,230 | 85,100 | 142 | 135 |
| 73 | 24,750 | 87,630 | 144 | 137 |
| 82 | 27,750 | 90,640 | 145 | 139 |
| 93 | 31,470 | 94,350 | 149 | 142 |

5.

6.

7.

8.  SLATS/FLAPS LIMIT SPEEDS.

The flaps and slatS have been structurally cleared to the following limits during travel, or when fully extended:

• Slats only                 330 KIAS or mach 0.62   whichever is less

• Flaps 0'·30           330 KIAS or mach 0.62   whichever is less

• Flaps 31 '·Full       300 KIAS or mach 0.56   whichever is less

However, to ensure slat/flap extension, and from component life and handling considerations, the following limitations should be observed:

Flaps:

During Extension

• Flaps-l0 to 25 degrees 	          250 KIAS or 0.62 mach   whichever is less

• Flaps-26 degrees to full down	 220 KIAS or 0.48 mach   whichever is less

Static Extended Condition or During Retraction

• Flaps-0 to 25 degrees                270 KIAS or 0.62 mach   whichever is less

• Flaps-26 degrees to full down    245 KIAS or 0.48 mach   whichever is less

•  Slat limit speed is 295 KIAS or 0.62 mach which· ever is less.

9.   GLIDESLOPE  DISTANCES

Final approaches to land utilizing ILS radio navigation aids guidance are

made at a 3 degree downwards slope .

The following list details the horizontal distance covered from various heights

above the runway threshold during that  3 degree glideslope .

Basically it gives an idea of when to commence the 3 degree descent .

HEIGHT                      DISTANCE

1000'                              3.14 NM

2000'                              6.28 NM

2500'                              7.85 NM

3000'                              9.42 NM

4000'                            12.55 NM

5000'                            15.69 NM

6000'                            18.83 NM

7000'                            21.97 NM

NOTE 1 :  The above are heights above the threshold , to determine altitude

add the ground elevation of the airport  to the height .

Airport elevation can be determined from (A) a chart or plate (B) in the

Nearest airports , airports detail page of LH MFD  (C) the

SALS -Satellite Assisted Landing System instrument

towards the lower right of the panel , but the airport must

be selected  by clicking the enter button twice  , see the

instruments notes in DOCS folder .

Distance  to the threshold are best obtained obtained from  the same

instrument ( SALS )  as it measures to a point  5% down the length

of the runway that you are approaching , ie ; about your

touchdown point.

Other instruments such as the HUD ,GPS and ADI give distances to

points such as NAVAID antennas and Aerodrome reference points

which are scattered about the airport and are sometimes  several

nautical miles past the touchdown point .

The  SALS instrument is by Glenn Copeland , it is well worth having it on during

ILS approaches and becoming aquainted with its operation . This instrument

enables you to conduct an ILS like approach at airports that do not have radio NAVAIDS. , very handy to say the least .

10.   DIVE  BOMBING DISTANCES

Distance covered horizontally in Dive Bombing scenarios .

Table of dive bombing runs .

Distance NM  given ; Dive angle in Degrees , Height in 000s  above target.

| HEIGHT x 1000 | 10 | 15 | 20 | 30 | 40 |
| --- | --- | --- | --- | --- | --- |
| 1 2 3 4 5 | 0.93 1.87 2.80 3.73 4.66 | 0.61 1.23 1.84 2.46 3.07 | 0.45 0.90 1.36 1.81 2.26 | 0.28 0.57 0.85 1.14 1.42 | 0.20 0.39 0.59 0.78 0.98 |
| 6 7 8 9 10 | 5.60 6.53 7.46 8.39 9.33 | 3.68 4.30 4.91 5.52 6.14 | 2.71 3.16 3.62 4.07 4.52 | 1.71 1.99 2.28 2.56 2.85 | 1.18 1.37 1.57 1.76 1.96 |
| 11 12 13 14 15 | 10.26 11.19 12.13 13.06 13.99 | 6.75 7.37 8.00 8.59 9.21 | 4.97 5.42 5.87 6.33 6.78 | 3.13 3.42 3.70 4.00 4.27 | 2.16 2.35 2.55 2.74 2.94 |
| 16 17 18 19 20 | 14.92 15.86 16.79 17.72 18.66 | 9.82 10.44 11.05 11.66 12.28 | 7.23 7.68 8.13 8.59 9.04 | 4.56 4.84 5.13 5.41 5.70 | 3.14 3.33 3,53 3.72 3.92 |

Example : In the case highlighted in bold  on the above table the aircraft must

have its nose pointing down at the 20 degree dive angle at 15,000' at a horizontal distance of  6.78 NM before the target .

NOTE 1 :  The altitudes nominated in the table are above the target .

If the ground elevation of the target is 4,000'  , then that 4,000'

would be added to the  15,000' and the aircraft altimeter would read

19,000' in the above situation .

The table would still be entered at 15,000' as that is the aircraft

height above the target .

NOTE 2  :  No allowances are made for ordinance ballistics in the above table .