FLARES OPTIONS

1. Ensure that the package has been installed using 'INSTALLATION GUIDE' word document . This installation process has already installed  all the effects .

2. There are 4 options available , each option governs the number of Flares that will be issued per switch activation  , 1 ,4 , 8 , or 60 seconds continuous , flare issue can be discontinued by off switch . The single shot is the best , the others being a different type of effect do not have as good a quality of smoke . Switching on/off too rapidly interupts the time cycle and can result in unsightly fragments.

For those that do not wish to assign the 'strobe' function to a controller button the

60 second option would be best as it gives time to select an outside view .

3. Changing options does require the editing of the 'aircraft cfg'   [LIGHTS]

section which is located after the aircraft descriptions and the camera views .

The editing process is described below  and  for accuracy sake is best done by copy and paste ..

REMEMBER always  SAVE after any changes .

4. Below is an extract of the  [LIGHTS] section  included in the 'aircraft cfg'  .

You will see that the FLARES are listed at LIGHT.6 and LIGHT.7 .

[LIGHTS]

//TYPES: 1=BEACON, 2=STROBE, 3=NAVIGATION, 4=COCKPIT, 5=LANDING, 6=TAXI, 7=RECOGNITION, 8=WING, 9=LOGO 10= CABIN

LIGHT.0 = 4, 25.00, 0, 1.58, FX_VCLIGHT

LIGHT.1 = 3, -27.136, 0, 10.06, FX_NAVWHI

LIGHT.2 = 3, 14.911, -5.088, 1.922, FX_NAVRED

LIGHT.3 = 3, 14.911, 5.088, 1.922, FX_NAVGRE

LIGHT.4 = 1, 7.114, -0.665, 3.131, FX_BEACONH

LIGHT.5 = 1, 6.064, -0.665, -3.048, FX_BEACONH

LIGHT.6 = 2, -22.6, -14.64, -10.1, FX_FLARE_PigPt           // 1 shot

LIGHT.7 = 2, -22.56, 14.66, -10.18, FX_FLARE_PigStbd    // 1 shot

//LIGHT.6 = 2, -22.6, -4.64, 0.1, FX_FLARE_DipPt

//LIGHT.7 = 2, -22.56, 4.66, 0.18, FX_FLARE_DipStbd

LIGHT.8 = 9, -20,  -2.50,  0.5, SR_BURNER_PORT

LIGHT.9 = 9, -20,   2.50,  0.5, SR_BURNER_STARBOARD

LIGHT.10 = 10, -10.0,  0.0,  0.0, FX_RCB_MACHENVLOP,1   //MACH ONE VAPOR CLOUD

LIGHT.11 = 7, -39.000, -2.500, 0.500, FX_EXHAUST_PIG                    // ENGINE 1

LIGHT.12 = 7, -39.000, 2.500, 0.500, FX_EXHAUST_PIG                     // ENGINE 2

//LIGHT.11 = 7, -39.000, -2.500, 0.500, FX_LTBRWN15EXHAUST_PIG            // ENGINE 1

//LIGHT.12 = 7, -39.000, 2.500, 0.500, FX_LTBRWN15EXHAUST_PIG              // ENGINE 2

LIGHT.13 = 8, -9.56, -13.16, -1.60, FX_MISS_BEST

//LIGHT.14 = 8, -9.66, 13.26, -1.60, FX_MISS_TEST

//LIGHT.14 = 8, -9.66, 13.26, -1.60, FX_FLARE_miss

LIGHT.14 = 8, -9.66, 13.26, -1.60, FX_MISS_BEST

LIGHT.15 = 6, -31.000, 0.00, 0.0, FX_WWF111RIVDNCE

5. Its location is:

Go to FSX,

then  simobjects

then  airplanes

then   ALPHA F-111 PIG HUD

then  aircraft cfg , open it with  notepad

scroll down to  [LIGHTS]

6. You can now make a choice of which Flare option you wish to employ .

The options are based on the number of Flares issued per click of the switch .

The options are :

per switch activation .

- Single Flare shot

-  4 Flare shot

-  8 Flare shot

- 60 seconds of continuous shots .

The above  [LIGHTS]  section has the  '1 Flare shot' option listed as shown below

LIGHT.6 = 2, -22.6, -14.64, -10.1, FX_FLARE_PigPt

LIGHT.7 = 2, -22.56, 14.66, -10.18, FX_FLARE_PigStbd

7. Following below are the relevant template lines that need to be highlighted and copied then pasted  into the  [LIGHTS]  section by highlighting the relevant lines   LIGHT.6 and   LIGHT.7  .

8. Options are :

- Single Flare shot

LIGHT.6 = 2, -22.6, -14.64, -10.1, FX_FLARE_PigPt

LIGHT.7 = 2, -22.56, 14.66, -10.18, FX_FLARE_PigStbd

-  4 Flare shot

LIGHT.6 = 2, -22.6, -14.64, -10.1, FX_FLARE4_PigPt

LIGHT.7 = 2, -22.56, 14.66, -10.18, FX_FLARE4_PigStbd

-  8 Flare shot

LIGHT.6 = 2, -22.6, -14.64, -10.1, FX_FLARE8_PigPt

LIGHT.7 = 2, -22.56, 14.66, -10.18, FX_FLARE8_PigStbd

- 60 second continuous shot

LIGHT.6 = 2, -22.6, -14.64, -10.1, FX_FLARE60_PigPt

LIGHT.7 = 2, -22.56, 14.66, -10.18, FX_FLARE60_PigStbd

You will notice that the only difference above is the number following FLARE

in each line  .