EMBRAER 110 (Bandeirante)

ESPAÑOL / ENGLISH

PROCEDIMIENTOS____________________________________________________

Antes de partir

- Combustible – SEGÚN

- Puerta principal – ABRIR

- Freno de parqueo - PONER

- Todos los interruptores - OFF

- Planta en tierra - ON

- Luces Panel, Cabina pilotaje, Cabina pasajeros - SEGUN

- Nav, No fumar, Cinturones - ON

Arranque

- Beacon - ON

- Puerta principal - CERRAR

- Limpia parabrisas, Separador inercial - SEGUN

- Mandos Gases - MINIMA

- Mandos Hélices - MAXIMA

- Mandos Combustible - CERRAR

- Convertidores - ON

- Bomba de combustible Aux. Motor 2 - AUTO

- Bomba de combustible Princ. Motor 2 - ON

- Partida Motor 2 - ENCENDIDO

- Con RPM Gases 12%, Mando Combustible Motor 2 - MINIMA

- Con Motor 2 arrancado, Generador 2 - ON

- Bomba de combustible Aux. Motor 1 - AUTO

- Bomba de combustible Princ. Motor 1 - ON

- Partida Motor 1 - ENCENDIDO

- Con RPM Gases 12%,  Mando Combustible Motor 1 - MINIMA

- Con Motor 1 arrancado, Generador 1 ON

- Batería – ON

Rodaje a pista activa

- Luces Taxi – SEGÚN

- Bomba recalque, Autofeather - ON

- Frenos de parqueo – LIBERAR

- Flaps - 25%

Carrera y Despegue

- Limpia parabrisas – OFF

- Separador inercial - SEGUN

- Landing, Strobes - ON

- Acelerar Torque ~ 1300 lbs

- Controlar con steering hasta 40 KIAS, resto con pedales

- Rotar y despegar ~ 90 KIAS

Ascenso

- Climb positivo recoger trenes y chequear - ARRIBA

- A 120 Knots y más 400 fts Flaps - 0%

- Bomba recalque, Autofeather - OFF

- Sincronizador de Propelas - ON

- Luces Taxi, Landing - OFF

- Torque  ~ 1300 lbs

- RPM Propelas - 91%

- Máximo régimen de ascenso -1000 fts/min.

Crucero

- Máximo permitido - 12000 fts

- Torque  ~ 1300 lbs

- RPM Propelas - 83%

Descenso

- Reducir potencia

- Máximo descenso - 1000 fts/min.

- Ajustar potencia

Aproximación

- Ajustar potencia

- RPM Propelas - 91%

- Por debajo de 180 KIAS flaps - 25%

- Bomba recalque, Autofeather - ON

- Sincronizador de Propelas - OFF

- Luces Taxi, Landing - ON

- Flaps - 50%

- Flaps - 75%

- Por debajo de 140 KIAS Trenes - ABAJO

Aterrizaje

- En final Flaps -100%

- Velocidad final ~100 KIAS

- Con aterrizaje asegurado gases – MINIMA

- Propelas - MAXIMA

- Con trenes en pista reversos – PONER

- A menos de 60 KIAS reversos - QUITAR

- Frenos diferenciales - SEGUN

- Flaps - 0%

- Landing, Strobes - OFF

Rodaje a rampa

- Limpia parabrisas, Separador inercial - SEGUN

- Beacon – ON

Apagado

- Freno de parqueo - PONER

- Gases - MINIMA

- Combustible - CERRAR

- Con motores apagados puerta principal - ABRIR

- Todos los interruptores – OFF

PROCEDURES_________________________________________________________

Before taking off

- Fuel – DEPENDS

- Main door – OPEN

- Parking break - LOCK

- All switches - OFF

- Ground plant - ON

- Panel lights, cockpit, passengers cabin – DEPENDS

- Nav, No smoking, Seatbelts - ON

Starting

- Beacon - ON

- Main door - CLOSE

- Windshield brush, Inertial separator - DEPENDS

- Throttle control - MINIMUM

- Propellers control - MAXIMUM

- Fuel mesh control - CLOSE

- Converters - ON

- Fuel pump Aux. Engine 2 - AUTO

- Fuel pump Main Engine 2 - ON

- Starting engine 2 - ON

- With RPM Gas 12%, Fuel mesh engine 2 - MINIMUM

- With engine 2 running, Generator 2 - ON

- Fuel pump Aux. Engine 1 - AUTO

- Fuel pump Main Engine 1 - ON

- Starting engine 1 - ON

- With RPM Gas 12%, Fuel mesh engine 1 - MINIMUM

- With engine 2 running, Generator 1 - ON

- Battery – ON

Taxing to active runway

- Taxi lights – DEPENDS

- Extra hydraulic pump, Autofeather - ON

- Frenos de parqueo – RELEASE

- Flaps - 25%

Run and takeoff

- Windshield brush – OFF

- Inercial separador - SEGUN

- Landing, Strobes - ON

- Gas for torque ~ 1300 lbs

- Take control with steering until 40 KIAS, after with rudder

- Rotate and  takeoff ~ 90 KIAS

Climb

- With climb retract gears and check - UP

- Beyone 120 Knots and  400 fts high Flaps - 0%

- Extra hydraulic pump, Autofeather - OFF

- Propellers cyncronizer- ON

- Taxi, Landing lights - OFF

- Torque  ~ 1300 lbs

- RPM Propellers - 91%

- Max. climb rate -1000 fts/min.

Crucero

- Max. altitude- 12000 fts

- Torque  ~ 1300 lbs

- RPM Propellers - 83%

Descend

- Reduce speed

- Max. descend rate - 1000 fts/min.

- Adjust speed

Approach

- Adjust speed

- RPM propellers - 91%

- Below 180 KIAS flaps - 25%

- Extra hydraulic pump, Autofeather - ON

- Propellers cyncronizer- ON

- Taxi, Landing lights - ON

- Flaps - 50%

- Flaps - 75%

- Below 140 KIAS Gears - DOWN

Landing

- On final Flaps -100%

- Final speed ~100 KIAS

- With secure landing Throttle – MINIMUM

- Propelles control - MAXIMUM

- On touch all gears applay revers – PONER

- Less 60 KIAS revers - QUITAR

- Brakes - SEGUN

- Flaps - 0%

- Landing, Strobes - OFF

Taxing to ramp

- Windshield brush, Inercial separator - SEGUN

- Beacon – ON

Shuttoff

- Parking brakes - LOCK

- Throttles - MINIMUM

- Fuel keys - CLOSE

- With engine off main door - OPEN

- All switches – OFF