Flying the F-111

Pre Flight

- Turn on and adjust white lights

- Move all control surface to check that they are free and clear. (this fixes a start up bug with elevon position)

- Set wing position to wings full forward, and set the Flaps to 25 degrees down.

- Reset Master Caution Light.

- Close weapons bay doors.

- Turn on HUD, TFR, Threat Warning and Attack Radar displays.

- Adjust HUD, TFR, Threat Warning and Attack Radar display modes and/or        brightness

- Set Altimeter barometric pressure  (important for TFR flying)

- Set Airspeed or Mach Command Marker for desired reference.

- Set the Radar Altimeter decision height. If flying TF the decision height light is automatically set to 66% of the selected TF altitude or Radar Altimeter decision height bug, which ever is greater.

- Set Pitch Steer Switch to Alt Ref for Altitude Hold and Mach Hold or TF for Terrain Following flying.

- If using radios for navigation set correct frequencies.

- Select navigation mode with Course Select Knob.

- Select navigation input with Comp/DG Switch

- Set Take Off Trim button

- Release Parking Brake and taxi to runway.

Night Flying

- Turn on and adjust red lights and turn off white lights

- Turn on taxi lights

Take Off

- Advance throttles to full afterburner.

- Rotate.

- Take off

- Retract landing gear and verify (transition light on landing gear lever)

- Retract flaps and slats and verify

- Decrease throttle to shut off afterburners

In Flight

- Turn on Pitch and Roll channel Stability Augmentation. (See Autopilot Operation Documentation).

- Adjust altitude, airspeed, and wing sweep for flight.

- Engage desired autopilot modes. Aircraft should be in a stable attitude before engaging autopilot modes.

- Set Altitude Command Marker for desired or TF or Alt Hold reference.

Note: Afterburners with shut off above 32,000ft and below 200 kias just like the real F-111

Flying Terrain Following Radar mode

- Set Pitch Steer switch to TF

- Set desired above ground level altitude on altimeter command window

- 200 to 1000 feet for auto TF

- 50 to 200 feet for manual TF

- Push Ref Not Engaged light, T key, or assigned joystick button to engage TF mode

- Remember that the TFR is not an engage and forget system. Just like the real aircraft you monitor the flight and override if necessary.

Flying Autopilot

- Excessive speeds will make it harder to maintain course in Flight plans with short distances between waypoints. 500kts or below is recommended

- See Autopilot Operation.doc for details of autopilot modes

Landing

- Reduce airspeed to below 350kts and sweep wings full forward.

- Lower landing gear below 300kts

- Lower flaps and slats to full down

- Set spoiler brake (wing fold option)

- Auto-Brake and Anti-Skid are on by default. Manually using the brakes turns off these features.