Autopilot Operation

This autopilot system emulates real F-111 autopilot system.

It is recommended that you always fly with the Pitch and Roll Autopilot switched to ON, this is the Stability Augmentation Mode.

Reference is engaged using the ‘t’ key*

Pitch and Roll channels can be turn on and off separately (together if using the z key)

- Pitch Channel

- Pitch channel ON activates Stability Augmentation Mode and will hold pitch angle unless control stick pitch input detected and then pitch angle when control stick is released will be maintained. Pitch Damper to OFF will cause the PITCH DAMPER warning light to come on.

- Pitch Cannel Sub Modes Pitch channel autopilot must be on for sub modes to work.

- Altitude Hold when engaged will set Altitude reference to the altitude when engaged, but will not hold altitude until the Reference is activated by clicking the Reference light or pushing the‘t’ key. Altitude can also be set on the Altimeter using the Command window on the altimeter and then activating Reference. Control stick pitch input will override Altitude Hold. When Reference is activated again it will maintain altitude when activated. Altitude Hold can not be engaged if Auto TF is on. It is recommended in the aircraft dash 1 manual that the pilot has the aircraft in a stable altitude and airspeed situation before engaging the Altitude Hold. The same applies with this aircraft.

- Mach Hold when engaged will adjust pitch angle to maintain the mach speed and lock out throttle input when Reference is activated. Control stick pitch input will override Mach Hold until Reference is activated again. When Reference is activated again it will maintain mach speed when activated. Mach Hold can not be set using the mach command window. Mach Hold can not be engaged if Auto TF is on.  It is recommended in the aircraft dash 1 manual that the pilot has the aircraft in a stable altitude and airspeed situation before engaging the Mach Hold. The same applies with this aircraft.

- Auto TF when engaged will maintain a set altitude above ground level. Altitude above ground level is set using the altimeter command window and can only be set to work between 200 and 1000 feet AGL. If you are above 1000 ft AGL and engage the Auto TF, your AGL will be set to 500Ft and then can be adjusted between 200 to 1000 feet. Auto TF will not hold AGL until the Reference is activated. If you are 50 to 200ft AGL Auto TF will send information to the Pitch Steer Bar on the ADI for manually flying TF when Pitch Steer switch is set to TF. Pitch control stick input will override Auto TF until Reference is activated again. When Reference is activated again it will return you to AGL set in the altitude command window.

Note: the Terrain Following Radar can not ‘see’ buildings or trees just like the real aircraft

- Roll Channel

- Roll channel ON activates Stability Augmentation Mode and will hold bank angle unless control stick roll input detected and then bank angle when control stick is released will be maintained. Roll Damper to OFF will cause the ROLL DAMPER warning light to come on. Roll channel autopilot can not be turned on unless the Yaw Damper is turned on first.

- Roll Channel Sub Modes Roll channel autopilot must be on for sub modes to work

- Constant Track when engaged will maintain a constant ground track. The ground track will be established at the time it is Referenced. Roll control stick input will override Constant Track until the Reference is activated. It will then return you to the original ground track. To establish a new ground track, switch off the Constant Track mode and then back on. The new ground track will be set when Referenced.

- Heading/Nav Modes

- Off provide no input to autopilot

- ILS mode will fly Glide Slope and Localizer for landing. Pitch control stick input will disengage Glide Slope autopilot only. Activating Reference will return pitch control to Glide Slope. Roll Control stick input disengages both Glide Slope and Localizer autopilot. Activating Reference will return pitch and roll control to Glide Slope and Localizer.

- Nav Mode will use GPS or Nav1 frequency for heading. Nav or GPS input is selected on the Compass Controller Panel. Roll control stick input will override Nav Mode. Activating Reference will return aircraft to course.

- Manual Heading mode course is set by HSI heading bug. Heading bug can be set from either the HSI or Compass Control Panel. Right clicking Heading bug knob will set heading bug to current compass heading.

- Yaw Damper by default is on. Switching Yaw Damper to OFF will cause the YAW DAMPER warning light to come on and the Roll channel autopilot to disengage. The Yaw Damper must be on before you can engage the Roll channel autopilot.

- Compass Controller Panel

- Comp DG Switch selects Nav Mode input. DG is the NAV1, ILS, or HDG input and Comp is the GPS input.

- Pitch Steer/Flight Director Switch has three settings. Off, ALT REF, and TF. In the ALT REF position the pitch steer bar on the ADI gets its information from the altitude hold autopilot. In the TF position the pitch steer bar get its information from the terrain following radar.

Notes:

  - I recommend that you bind a joystick button to the‘t’ key to make activating Reference much easier.

  - * Note that the FSX ‘t’ key must be the default ‘Transponder (select)’

  - I recommend that you slightly increase the joystick Ailerons axis and Elevator axis Null zones to help prevent control stick movement in one axis from affecting the other axis and disengaging its autopilot also.

  - If you have trim commands mapped to your joystick, make sure that you are using FSX trim commands and not keyboard pitch and roll commands. If you see the joystick move in the virtual cockpit when you trim using your joystick switches, you are using keyboard pitch and roll commands. Change to trim commands or you will not be able engage the autopilot modes if joystick is not neutral.