Alphasim F-111 Aircraft and Panel Version 3.0 Beta

By Steve Hess and Roman Stoviak

Pop-up Panels

- Ctrl + 1	Pilot VFR panel

- Ctrl + 2	Pilot IFR panel

- Ctrl + 3	GPS

- Ctrl + 4	Radios

- Ctrl + 5	Autopilot and Lighting panel

- Ctrl + 6	Inertial Navigation Display

- Ctrl + 7	Horizontal Situation Indicator

- Ctrl + 8	Weapon System Operator VFR panel

- Ctrl + 9	Weapon System Operator IFR panel

Working Panel Gauges

The 2D and Virtual Cockpit layouts are the same. There are four 2D panels. Two for the pilot    (VFR and IFR panels) and two for the WSO (weapons system operator, VFR and IFR panels)

Note: not all gauges are present both 2D and VC panels

- AOA Indexer

- Heads UP Display

- Standby Compass

- Fuel Dump Switch

- Reference Not Engaged Warning

- Autopilot Status lights

- LCOS Panel

- Nose Wheel Steering light

- RADAR altimeter

- Low Altitude Warning light

- Master Caution light

- Terrain following RADAR scope

- Caution and Warning lights

- Autopilot and Marker beacon lights

- Total Airframe Temperature

- Airspeed Mach indicator

- Attitude Direction Indicator

- Altitude Vertical Velocity indicator

- Wing sweep – Slat/Flap indicator

- Landing Gear control

- Coarse Select Panel

- Horizontal situation indicator

- DME indicator

- Control Surface Position Indicator

- Take Off  Trim set

- AOB indicator

- Anti Icing control panel

- Compass Controller panel

- Power Control panel

- Caution and Warning lights

- RPM gauges

- Turbine inlet temperature indicators

- Fuel flow indicators

- Nozzle position indicators

- Engine pressure ratio indicators

- Hydraulic pressure indicators

- Oil pressure indicators

- Fuel Quantity Select switch

- Standby airspeed indicator

- Standby vertical speed indicator

- Clock

- Fuel quantity indicators

- Threat Warning Display

- Bearing Distance Heading Indicator

- Standby attitude indicator

- Standby altimeter

- True Airspeed indicator

- Aircraft call number

- Attack RADAR scope

- Attack Radar Control panel

- Inertial Navigation Display

- Comm Radio

- Nav Radio

- Transponder

- ADF Radio

^ O

Note: Gauge locations may vary for aircraft models

Note: 	In the Settings/Display/Hardware tab the Global Max Texture Size must be set to HIGH or greater or the Virtual Cockpit will appear blurred.

GO TO NEXT PAGE

Click Points

VC and 2D click point are the same

- Fuel dump (and Dump & Burn Effect)

- Reticle Brightness*

- HUD On/Off

- Command Bars brightness*

- RADAR Altimeter Decision Height Adjust

- Reference Engage

- Master Caution (press to reset)

- Tail Hook

- Mach set*

- Airspeed set*

- Barometer set*

- Altitude set*

- Weapon Doors OPEN/CLOSE

- Landing Gear UP/DOWN

- Warning Light Test (Parking Brake must be set & avionics bus on)

- Course Select Knob

- Pitch Steer Reference switch

- Heading Bug Adjust Knob*

- Course Adjust Knob*

- Take Off Trim set

- HSI pop-up panel

- Anti Ice, Compass Select, Power pop-up panel

- Pitot Heat ON/OFF

- Engine Inlet Heat ON/OFF

- Heading Bug Adjust Knob*

- Left Engine Generator and Engine Start switch

- Battery ON/OFF

- Rain Removal switch (dummy)

- Nav-GPS switch

- Ground APU ON/OFF

- Avionics ON/OFF

- Right Engine Generator and Engine Start switch

- EPR bug set knob*

- Fuel Indicator Select switch*

- Fuel Indicator test switch

- TFR Cursor Brightness*

- TFR Memory

- TFR Display brightness*

- TFR Off-STBY-Sweep Mode switch

- TFR Range switch (5-10-15 miles)

- Threat Scope ON-OFF

- Threat Scope Range (20-40 miles)

- BDHI Course knob*

- Attack Radar Ground Track ON-OFF

- Attack Radar Range Rings ON-OFF

- Attack Radar Range (5-15-30-80-200 miles)

- Attack Radar Return Mode (OFF-Air to Ground – Air to Air)

- Attack Radar On-OFF

- Comm 1 Frequency select knob

- Comm 1 Active-STBY Swap switch

- NAV 1 Frequency select knob

- NAV 1 Active-STBY Swap switch

- Transponder Frequency Select knobs

- INS pop-up panel

- Flare Dispenser Control

- best adjusted using the scroll wheel on the mouse

Flight Instruments and Functions

Attitude Direction Indicator

- Course warning flag

- Bank steering bar

- Glide slope warning flag

- Glide slope indicator

- Power off warning flag

- Pitch steering bar

- Bank scale

- Turn and slip indicator

- Heading reference scale

- Miniature aircraft

- Pitch trim

- Course Warning Flag in view if no input from NAV systems

- Bank Steer Bar is in view and shows fly to information when signal is present from Nav systems

- Glide Slope Warning Flag in view when signal is lost in ILS mode

- Glide Slope Indicator shows deviation from glide slope

- Power Off Flag is in view if ADI malfunctions or power is shut off

- Pitch Steer Bar is in view when the Pitch Steer Switch is in either the ALT REF of TF position. In ALT REF position it will display climb or dive directions to the altitude set in the Altimeter Command Window. In the TF position it will show climb or dive directions to maintain AGL altitude set in the Altimeter Command Window.

- Bank Scale shows degrees of roll in 10 degree increments.

- Turn and Slip displays the rate of turn and coordinated turn information.

- Heading Reference Scale displays compass direction

- Miniature Aircraft provides horizon reference

- Pitch Trim adjusts horizon reference.

Horizontal Situation Indicator

- Range indicator and warning flag

- Course arrow head

- Heading marker

- Course selector window

- Bearing pointer

- Course deviation indicator

- Course warning flag

- Heading set knob

- Compass card

- Power off warning flag

- Course To/From arrows

- Course set knob

- Course deviation scale

- Aircraft symbol

- Course arrow tail

- Range Indicator and Warning Flag displays distance in miles to NAV waypoint or Nav radio signal. Warning flag in view if there is no waypoint or valid Nav signal

- Course Arrow Head gets is input from the Nav radios and adjusts using Course Set Knob.

- Heading Marker adjusts using the Heading Set Knob. It can be used for manual heading reference or input to the autopilot in the Man HDG mode on the Autopilot Heading Control panel.

- Course Selector Window displays the Course Arrow Heading

- Bearing pointer displays heading for next GPS waypoint or Nav 2 radio

- Course Deviation Indicator is set by adjusting the Course Set Knob to align the bar with the Course Arrow Head.

- Course Warning Flag is in view when there is no valid Nav radio signal.

- Heading Set Knob adjusts the Heading Marker position. Rt. Click to synch current heading

- Compass Card displays 360 degree heading in 5 degree increments.

- Power Off Warning Flag is in view if HSI malfunctions or power is shut off

- Course To/From Arrows show if you are flying towards or away from the Nav radio signal.

- Course Set Knob adjusts the Course Arrow Head to align the Course Deviation Indicator with the Course Arrow Head and changes the Course Selector Window read out.  Rt. Click for inc/dec by 10

- Course Deviation Scale shows off course direction and amount.

- Aircraft Symbol is the reference point for the HSI

- Course Arrow Tail is the opposite end of the Course Arrow Head

Airspeed Mach Indicator

- Angle of Attack indicator

- Accelerometer

- Mach Indicator

- Airspeed Indicator

- Command Mach Marker

- Command Airspeed Marker

- Mach Warning Flag

- Airspeed Warning Flag

- Maximum Allowable Mach Marker

- Acceleration Readout Window

- Command Mach Readout Window

- Command Airspeed Readout Window

- Command Mach Slewing Switch

- Command Airspeed Slewing Switch

- Angle of Attack Indicator white number are positive AOA and red numbers are negative AOA

- Accelerometer white number are positive G Force and red numbers are negative G Force

- Mach Indicator

- Airspeed Indicator

- Command Mach Marker is controlled by the Command Mach Slewing Switch to set a Mach reference point only.

- Command Airspeed Marker is controlled by the Command Airspeed Slewing Switch to set a airspeed reference point only.

- Mach Warning Flag is in view for Mach Indicator malfunction or power off.

- Airspeed Warning Flag is in view for airspeed Indicator malfunction or power off.

- Maximum Allowable Mach Marker show Maximum safe speed for the aircraft.

- Acceleration Readout Window shows a digital readout of the G Forces

- Command Mach Readout Window shows a digital readout of the Command Mach Marker and is set with the Command Mach Slewing Switch.

- Command Airspeed Readout Window shows a digital readout of the Command Airspeed Marker and is set with the Command Airspeed Slewing Switch

- Command Mach Slewing Switch controls the Command Mach Readout Window and the Command Mach Marker.

- Command Airspeed Slewing Switch controls the Command Airspeed Readout Window and the Command Airspeed Marker.

Altitude Vertical Velocity Indicator

- Vertical Velocity Indicator

- Vernier  Altimeter

- Coarse Altimeter

- Gross Altimeter

- Altitude Warning Flag

- Coarse Command Marker

- Vernier Command Marker

- Barometric Pressure Readout Window

- Command Altitude Readout Window

- Barometric Pressure Set Knob

- Command Altitude Slewing Switch

- Vertical Velocity Indicator.

- Vernier Altimeter tape shows the altitude 100 ft increments.

- Coarse Altimeter tape shows the altitude 1,000 ft increments.

- Gross Altimeter tape shows altitude 10,000 ft increments

- Altitude Warning Flag in view when Altimeter malfunctions or power is off.

- Coarse Command Marker is controlled by the Command Altitude Slewing Switch to set an altitude reference point.

- Vernier Command Marker is controlled by the Command Altitude Slewing Switch to set an altitude reference point

- Barometric Pressure Readout Window is set by the Barometric Pressure Set Knob.

- Command Altitude Readout Window is set by the Command Altitude Slewing Switch

- Barometric Pressure Set Knob

- Command Altitude Slewing Switch primary use is to set the altitude Vernier, Coarse, and Altitude Window reference points.

- Also used to set AGL in TF autopilot mode (From 200ft to 1000ft for Auto TF and 50ft to 200ft for manual TF)

- Also used to set altitude in Altitude Hold Autopilot mode.

HUD and LCOS Panel

- Pitch and Glide Slope deviation display

- Bank Steer Bar

- Pitch Steer Bar

- Airspeed deviation display

- Time to Destination Scale

- Reticle

- Reticle Brightness control

- HUD Power On/Off

- Pitch and Bank Steer bar brightness control

- Pitch and Glide Slope deviation display:  If the green bar is below the center line then you are above your altitude reference and need to decrease your altitude. If the green bar is above the center line then you are below your altitude reference and need to increase your altitude.

- Airspeed deviation display: If the green bar is below the center line then you are slower than your airspeed reference and need to increase your airspeed. If the green bar is above the center line then you are faster than your airspeed reference and need to decrease your airspeed

- Time to Destination Scale 9 to 6 o’clock portion of the scale counts down time to destination. If waypoint is overshot then 6 to 3 o’clock  portion of the scale show time past waypoint

Lights and Autopilot Panel

Ctrl + 5

Working switch positions shown in green.

Lighting Panel

Nav

Beacon

Landing

Taxi

Red Panel Lighting

White Panel Lighting (VC only)

Brightness Control

*Red and White lights (VC only)

Autopilot Panel

See Autopilot Operation.Doc for functions

Pitch & Roll Channel Autopilot ON/OFF

Yaw Damper ON/OFF

Altitude Hold ON/OFF

Mach Hold ON/OFF

Const track

Heading Nav

Auto TF (Terrain Following Radar) On/OFF

Course Select Panel

Off

ILS

NAV (uses NAV or GPS heading input)

MAN HDG (uses heading bug input)

FLT DIR (flight director)

Gear Indicator

Compass Controller Panel

HDG knob (controls HSI heading bug)

Comp (GPS input)

DG (NAV input)

Inertial Navigation Display

Ctrl + 6

- Present Latitude in degrees

- Present Longitude in degrees

- Present Altitude (feet x 10)

- Selected or Next Waypoint Latitude (all zeros if no flight plan selected)

- Selected or Next Waypoint Longitude (all zeros if no flight plan selected)

- Autopilot Altitude Selected (feet x 10)

- Sequence Legs (toggles display 19 between current waypoint and total number of waypoints)

- Previous Waypoint Select

- Next Waypoint Select

- Ground Heading

- True Heading

- Ground Speed

- Airspeed

- Miles to Destination (next waypoint) (all zeros if no flight plan selected)

- Wind Direction (from)

- Wind Speed (knots)

- False display (no astrocompass in FSX)

- Time to Next Waypoint (all zeros if no flight plan selected)

- Waypoint Number Selected (all zeros if no flight plan selected)

- Waypoint Select tens unit (use mouse wheel to scroll)

- Waypoint Select ones unit (use mouse wheel to scroll)

- Activate Selected Waypoint (as displayed on thumbwheels 20 & 21)

*Latitude and Longitude are shown in degrees on the INS display. Using the FSX shift + Z key displays latitude and longitude in decimal. The numbers will not agree.

Special Thank You

Thank you to Douglas Dawson for helping to create the look ahead Terrain Following Radar

I am very grateful to Roman Stoviak for his hard work and dedication to creating the gauges, Terrain Following Radar, and air.cfg file modifications that took this project to the next level. Roman’s contributions are truly too numerous list here, but without his involvement, this project would not have been what it became.

Steve Hess